[AGENCY NAME]
Unified Unmanned Aircraft Systems
Operations and Policy Manual + Training

Template for Oregon State Agencies, Counties, Cities, Special Districts, and Other Public Bodies

Version: [Draft / Adopted Version]
Effective Date: [Date]
Review Cycle: [Annual / Biennial]
Program Owner: [Office / Division / Program]
Custodian: [UAS Program Manager / Policy Owner]

Prepared as a customizable policy and operations manual. All bracketed items should be reviewed and completed by the adopting agency before approval.

# How to Use This Template

Template purpose: This manual is written for a generic Oregon public agency UAS program. It is intentionally broad enough for transportation, public works, planning, environmental, emergency management, parks, utilities, natural resources, and general administrative uses. Public safety and law enforcement material is separated into optional annexes. Non-public-safety agencies should not adopt those annexes unless the agency has an actual need and legal authority for those missions.

Bracketed text such as [AGENCY NAME], [PROGRAM MANAGER], and [RETENTION PERIOD] identifies items that should be customized before adoption.

This template should be reviewed by agency counsel, the agency records officer, information security staff, risk management, procurement, and the official with authority to approve agency aviation or UAS operations.

The manual is current to the legal sources reviewed on June 9, 2026. Federal and Oregon UAS laws can change. The UAS Program Manager should verify the law during each annual review and before high-risk operations.

The core manual is intended for routine civil/public agency operations under 14 CFR Part 107 unless an agency specifically elects to operate under a public aircraft Certificate of Waiver or Authorization or another FAA authorization.

## Integration Priority and Use of Supplemental DFR / Training Material

This integrated version preserves the Oregon Public Agency UAS Operations and Policy Manual Template as the controlling base document. The Unified UAS / DFR Policy, Operations, Training, and Test Bank Manual is incorporated as supplemental material that adds DFR, public-safety, law-enforcement, mutual-aid, BVLOS/COA, remote-operations, training, proficiency, practical-evaluation, and test-bank content.

If a provision in the supplemental material overlaps with the generic public agency template, the generic public agency template controls unless the adopting agency expressly adopts the supplemental DFR, public-safety, law-enforcement, training, or advanced-operations provision after legal, risk, records, IT/security, procurement, and FAA-authorization review.

The supplemental material should be deleted, marked not adopted, or customized where the adopting agency does not conduct the mission type or does not have the required legal authority, FAA authorization, staffing, training, data governance, or operational-control framework.

## Primary Customization Checklist

| Item | Agency Action |
| --- | --- |
| Agency identity and approval | Insert agency name, division, policy owner, approval authority, effective date, and review interval. |
| Program authority | Identify the UAS Program Manager, flight operations coordinator or chief pilot, records officer, IT/security lead, and procurement lead. |
| Mission types | Select authorized mission categories and delete those that do not apply. |
| Operating authority | State whether flights will generally be conducted under Part 107, a public aircraft COA, emergency COA, or a combination. |
| Data policy | Set retention periods, storage systems, public posting location, third-party storage terms, and intergovernmental disclosure rules. |
| Training and currency | Set agency-specific flight currency, annual review, platform checkout, and records requirements. |
| Public safety annex | Adopt only if the agency conducts public safety, emergency response, incident support, fire, SAR, or disaster operations. |
| Law enforcement annex | Adopt only for a law enforcement agency or a non-law-enforcement agency providing UAS support to law enforcement under counsel-reviewed procedures. |

## Revision and Approval Record

| Version | Date | Summary of Change | Reviewed By | Approved By |
| --- | --- | --- | --- | --- |
| 0.1 | [Date] | Initial agency customization from template. | [Name / Office] | [Name / Title] |

# Contents

1. Policy Statement, Scope, and Authority

2. Definitions and Acronyms

3. Program Governance, Roles, and Responsibilities

4. Authorized Missions and Prohibited Uses

5. Federal and Oregon Legal Compliance

6. Acquisition, Inventory, Registration, and Contractor Services

7. Personnel Qualifications, Training, and Currency

8. Safety Management System

9. Mission Request, Authorization, and Planning

10. Standard Flight Operations Procedures

11. Emergency Response, Mishaps, and Reporting

12. Data Management, Privacy, Cybersecurity, and Public Records

13. Maintenance and Continued Airworthiness

14. Interagency Operations and Mutual Aid

15. Optional Annex A: Public Safety and Emergency Operations

16. Optional Annex B: Law Enforcement Operations

17. Appendices and Forms

18. Sources Consulted and Legal References

# 1. Policy Statement, Scope, and Authority

## 1.1 Policy Statement

[AGENCY NAME] uses Unmanned Aircraft Systems (UAS) only for authorized public purposes, only under appropriate operational control, and only when the operation can be conducted safely, legally, and in a manner that respects privacy, civil liberties, public records obligations, and the trust placed in public agencies.

The agency will manage UAS operations as aviation activity. All UAS operations must be planned, briefed, documented, and reviewed commensurate with mission risk. The Remote Pilot in Command has final authority over the safe conduct of a flight and may cancel, postpone, modify, or terminate a flight when safety, regulatory compliance, equipment condition, weather, airspace, crew readiness, public concerns, or mission conditions warrant that decision.

## 1.2 Purpose

Establish a consistent agency framework for UAS governance, mission approval, flight operations, safety management, data handling, maintenance, training, and reporting.

Support responsible use of UAS for public missions such as infrastructure inspection, mapping, environmental assessment, transportation, engineering, planning, emergency management, utilities, parks, natural resources, communications, and other agency functions.

Ensure compliance with applicable FAA regulations, Oregon statutes, Oregon administrative rules, public records requirements, agency information security policies, records retention schedules, and procurement requirements.

Separate optional public safety and law enforcement content from general agency UAS operations so that agencies do not unintentionally adopt procedures that do not apply to their mission or legal authority.

## 1.3 Scope

This manual applies to:

All [AGENCY NAME] employees, volunteers, interns, and agents who plan, approve, support, crew, pilot, maintain, procure, store, process data from, or otherwise participate in agency UAS operations.

All UAS owned, leased, borrowed, loaned to, or operated by or on behalf of [AGENCY NAME].

Contracted UAS services when the work is performed for [AGENCY NAME] or when agency data, agency property, public operations, or agency-managed missions are involved.

Joint operations when [AGENCY NAME] has operational control, shares operational control, provides personnel, requests a UAS mission, or receives UAS data.

This manual does not regulate private UAS operators generally. A local agency adopting this template should treat it as an internal agency operating policy unless separate legal authority exists for externally applicable rules, such as a park rule adopted under express statutory authority.

## 1.4 Authority

| Authority | How it applies |
| --- | --- |
| 14 CFR Part 107 | Baseline FAA operating rule for most routine small UAS operations. Requires a remote pilot certificate or direct supervision by a certificated remote pilot, preflight inspection, compliance with operating limitations, airspace authorization when required, and safety event reporting. |
| 14 CFR Part 48 and FAA DroneZone | FAA aircraft registration and marking requirements for covered unmanned aircraft. |
| 14 CFR Part 89 | Remote ID requirements for UAS required to be registered with the FAA, subject to applicable exceptions. |
| 49 CFR Part 830 | NTSB notification and reporting requirements for aircraft accidents and listed serious incidents. |
| ORS 837.300 to 837.390 and ORS 837.995 | Oregon UAS statutes, including definitions, law enforcement restrictions, public body registration, data policy requirements, weaponization restrictions, private property provisions, critical infrastructure provisions, and preemption. |
| OAR 738-080-0045 and 738-080-0050 | Oregon Department of Aviation public body UAS registration and annual reporting rules. |
| ORS 192.001 and ORS 192.311 to 192.478 | Oregon public records policy, inspection, exemptions, and related public records procedures. |
| Agency rules, policies, and contracts | Agency-specific records retention, IT security, procurement, vehicle, safety, privacy, risk, contracting, and incident response policies. |

## 1.5 Relationship to Other Policies

This manual supplements, and does not replace, agency policies on records management, information security, privacy, procurement, vehicle use, risk management, emergency management, media relations, public records, workplace safety, incident command, and contract administration. If there is a conflict between this manual and a law, regulation, FAA authorization, court order, contract, or agency policy with higher authority, the higher authority controls. The UAS Program Manager must coordinate with counsel before approving a mission where a conflict or legal uncertainty is identified.

This manual is intended to be used with the agency’s existing safety, risk management, procurement, information technology, public records, privacy, cybersecurity, emergency management, and incident response policies. When UAS operations are conducted with another agency, contractor, mutual aid partner, emergency management entity, public safety agency, or federal partner, the agency shall identify which entity has operational control of the flight before operations begin. The agency shall also review applicable host-agency, land-management, emergency response, incident command, or aviation procedures before conducting operations in another agency’s operational area.

## 1.6 Program Exceptions

Exceptions to this manual may be approved only by [APPROVAL AUTHORITY] after consultation with the UAS Program Manager. Exceptions involving legal compliance, law enforcement support, public records, privacy, data retention, cybersecurity, procurement, operations over people, beyond visual line of sight, operations from moving vehicles, operations in controlled airspace, operations near emergency response aircraft, or operations under a COA must be reviewed by counsel or other appropriate subject matter experts before flight unless an emergency requires immediate action. Any emergency deviation must be documented within [72 hours] or sooner if required by law or FAA authorization.

# 2. Definitions and Acronyms

Customization note: Agencies should preserve definitions that are drawn from federal and Oregon law. Agencies may add internal terms, but should avoid redefining statutory or FAA terms in a way that changes their legal meaning.

| Term | Definition |
| --- | --- |
| Agency UAS | A UAS owned, leased, borrowed, operated, contracted for, or otherwise used for an authorized [AGENCY NAME] mission. |
| Airspace authorization | FAA authorization, including LAANC or manual authorization, permitting UAS operations in controlled airspace or in other airspace where authorization is required. |
| Certificate of Waiver or Authorization (COA) | An FAA authorization issued to a public operator or other eligible entity for specific UAS operations not otherwise covered by Part 107 or standard rules. |
| Crewmember | A person directly involved in setup, launch, recovery, control, visual observation, payload operation, communications, safety, or mission support. |
| Emergency COA (ECOA) | An expedited FAA authorization for certain emergency UAS activity, generally coordinated through the FAA for qualifying public operations. |
| LAANC | Low Altitude Authorization and Notification Capability, the FAA system that supports near real-time airspace authorization where available. |
| Mission Commander / Project Lead | The agency person responsible for the operational purpose of the mission, coordination with the UAS Program Manager, site access, safety coordination, and use of mission data. This person is not the RPIC unless separately designated. |
| Operational control | The authority over initiating, conducting, or terminating a flight. Operational control must be clear before any UAS mission begins. |
| Remote Pilot in Command (RPIC or PIC) | The person designated before or during the flight who is directly responsible for and has final authority over the operation of the small UAS. |
| Remote ID | FAA-required broadcast identification and location information for UAS required to comply with Part 89, unless an exception applies. |
| Small UAS | A small unmanned aircraft and associated elements weighing less than 55 pounds at takeoff, including everything on board or attached to the aircraft. |
| Unmanned Aircraft System (UAS) | An unmanned flying machine, commonly known as a drone, and its associated elements, including communication links and control components. |
| Visual Observer (VO) | A trained crewmember who assists the RPIC with see-and-avoid, obstacle awareness, communications, ground safety, and mission awareness. |

## Common Acronyms

| Acronym | Meaning |
| --- | --- |
| AGL | Above Ground Level |
| ATC | Air Traffic Control |
| COA | Certificate of Authorization |
| CFR | Code of Federal Regulations |
| ERP | Emergency Response Plan |
| FAA | Federal Aviation Administration |
| FRIA | FAA-Recognized Identification Area |
| GNSS | Global Navigation Satellite System |
| GPS | Global Positioning System |
| LAANC | Low Altitude Authorization and Notification Capability |
| NAS | National Airspace System |
| NOTAM | Notice to Air Missions |
| NTSB | National Transportation Safety Board |
| ODAV | Oregon Department of Aviation |
| OAR | Oregon Administrative Rules |
| ORS | Oregon Revised Statutes |
| PASP | Project Aviation Safety Plan |
| PII | Personally Identifiable Information |
| RPIC | Remote Pilot in Command |
| SMS | Safety Management System |
| SOP | Standard Operating Procedure |
| TFR | Temporary Flight Restriction |
| UAS | Unmanned Aircraft System |
| VO | Visual Observer |

# 3. Program Governance, Roles, and Responsibilities

A UAS program needs a clear chain of responsibility. A single person may hold more than one role in a small agency, but the manual should still identify who is accountable for policy, flight safety, records, data, maintenance, procurement, and annual reporting.

| Role | Core Responsibilities |
| --- | --- |
| Agency Head or Designee | Approves the agency UAS program, adopts this manual, assigns program authority, resolves policy issues, and ensures resources are available for safe operations. |
| UAS Program Manager | Owns the program, maintains this manual, approves pilots and aircraft for agency use, maintains fleet and registration records, coordinates annual ODAV reporting, reviews missions, manages audits, and serves as primary point of contact for UAS policy questions. |
| Flight Operations Coordinator / Chief Pilot | Oversees operational procedures, pilot standardization, mission planning quality, platform checkouts, crew coordination, flight logs, and operational risk management. In a small agency this role may be held by the UAS Program Manager. |
| Remote Pilot in Command | Has final authority and responsibility for safe flight; verifies aircraft condition, crew readiness, airspace, weather, Remote ID, registration, mission authorization, site suitability, emergency procedures, and postflight reporting. May cancel, modify, or terminate the mission. |
| Visual Observer | Maintains visual scan, assists with traffic and obstacle avoidance, monitors ground hazards, supports communications, and communicates changes to the RPIC. A VO must be briefed before flight and trained for the mission environment. |
| Payload Operator / Sensor Operator | Operates cameras or sensors only under the direction of the RPIC and mission plan, avoids unnecessary collection, and follows data handling rules. |
| Mission Requester / Project Lead | Defines the public purpose, provides mission location and access needs, coordinates site safety, obtains landowner or facility coordination when needed, and confirms how data will be used. |
| Records Officer / Data Steward | Ensures flight logs, imagery, video, maps, incident records, maintenance logs, registration documents, and public records are retained and disposed according to policy and law. |
| Information Security / IT Lead | Reviews systems used to store, transmit, process, or share UAS data; approves cloud storage or third-party platforms; supports incident response for lost devices or data exposure. |
| Maintenance / Inventory Coordinator | Maintains aircraft inventory, battery records, maintenance logs, firmware history, repair status, and airworthiness release after maintenance. |
| Procurement / Contract Administrator | Ensures aircraft purchases and UAS services are procured through approved channels and that contracts include safety, insurance, data, records, cybersecurity, and operational control terms. |
| Public Information Officer | Coordinates public-facing UAS program information, website posting, media inquiries, public complaints, and mission notifications where appropriate. |

## 3.1 Safety Review and Program Review

The agency should establish a UAS Safety Review Group appropriate to program size. For a small program, the group may consist of the UAS Program Manager, chief pilot, records officer, IT/security representative, and a supervisor from the primary user division. For a larger program, the group should include program leadership, operations, records, legal, risk, procurement, and mission representatives. The group should meet at least annually and after any reportable mishap, major incident, significant policy change, or introduction of new aircraft or new mission type.

Review flight activity, incident trends, near-misses, equipment issues, pilot currency, training, maintenance, data retention, public complaints, and legal updates.

Approve corrective actions and assign responsible persons and deadlines.

Assess whether new mission types, aircraft, software, payloads, or data systems require updated training, procedures, or counsel review.

Review annual ODAV reporting before submission when feasible.

## 3.2 Operational Control

Operational control means the authority over initiating, conducting, or terminating a flight. The entity exercising operational control is responsible for the safety, regulatory compliance, mission approval, and termination decision for the flight.

Before any UAS operation involving more than one agency, a contractor, a mutual aid partner, or a host agency, the UAS Program Manager or designee shall determine and document which entity has operational control. The following questions should be used to make that determination:

- Which agency or entity requested or ordered the aircraft?

- Which agency or entity is paying for the aircraft, service, or operator?

- Which agency or entity is directing the aircraft, assigning the mission, or controlling the flight objectives?

- Which agency or entity is benefiting from the aircraft operation?

When the answers point to more than one entity, the participating agencies shall identify the lead agency before flight. The Remote Pilot in Command retains final authority over the safe operation of the aircraft and may delay, modify, or terminate the flight when safety, airspace, weather, equipment, crew readiness, or legal compliance requires it.

Agency employees shall not direct the operation of an aircraft or UAS that is outside the agency’s operational control unless authorized by written agreement, incident command assignment, mutual aid agreement, contract, or other lawful authority.

# 4. Authorized Missions and Prohibited Uses

## 4.1 Authorized Mission Categories

UAS may be used only for authorized public purposes that are consistent with agency mission, law, policy, and approved procedures. The adopting agency should select from the examples below and delete categories that do not apply.

| Mission Category | Examples | Adopt? |
| --- | --- | --- |
| Infrastructure and facility inspection | Bridges, roads, airports, buildings, towers, roofs, water systems, stormwater infrastructure, utilities, rights of way, construction progress, damage assessment. | [Yes / No] |
| Mapping, survey, and engineering support | Orthomosaic mapping, photogrammetry, volumetric measurement, construction documentation, topographic support, asset inventory. | [Yes / No] |
| Environmental and natural resources | Habitat mapping, shoreline assessment, vegetation monitoring, erosion assessment, waterway documentation, wildlife-adjacent work with appropriate approvals. | [Yes / No] |
| Emergency management and damage assessment | Post-storm, flood, earthquake, wildfire, landslide, hazardous access, shelter or infrastructure assessment. Adopt Annex A if used for incident response. | [Yes / No] |
| Training and proficiency | Pilot currency, equipment checkout, controlled environment exercises, safety drills, data handling exercises. | [Yes / No] |
| Public information and outreach | Documentation of public projects, educational demonstrations, public meetings, media materials, subject to privacy and public records controls. | [Yes / No] |
| Public safety support | Search and rescue support, fire support, incident overwatch, evacuation route assessment, disaster response. Optional. Adopt Annex A. | [Yes / No / N/A] |
| Law enforcement support | Only for law enforcement agencies or counsel-reviewed support to law enforcement. Optional. Adopt Annex B. | [Yes / No / N/A] |

## 4.2 Prohibited Uses

Unless specifically authorized by law, written agency approval, FAA authorization when required, and counsel review, UAS shall not be used for:

Personal, recreational, political, campaign, or non-agency purposes.

General surveillance of persons, homes, private property, workplaces, events, or constitutionally protected activity unrelated to an approved agency mission.

Collection of data beyond the stated mission objective when avoidable.

Operations intended to harass, annoy, intimidate, or interfere with a person, property owner, aircraft, emergency responder, wildlife, or lawful activity.

Operations in a way that functions as a dangerous weapon or involves firing a bullet or projectile, unless a specific statutory exception, FAA authorization, notification, public notice, insurance, and counsel-reviewed mission approval are in place.

Operations that drop items from the UAS unless permission is granted from the Oregon Department of Aviation.

Operations that interfere with manned aircraft, emergency aircraft, airport operations, wildfire aviation, medevac operations, or other UAS operations.

Flights over people, moving vehicles, controlled airspace, beyond visual line of sight, from a moving vehicle, or using multiple aircraft by one pilot unless permitted under Part 107, waiver, COA, or another FAA authorization and approved by the UAS Program Manager.

Use of privately owned UAS for agency business unless specifically authorized in writing by the UAS Program Manager and risk management, and unless registration, insurance, data, and maintenance requirements are satisfied.

## 4.3 Public Trust and Data Minimization

Agency UAS operations should be planned to collect only the data needed for the authorized mission. Pilots and payload operators should use altitude, camera angle, sensor settings, geofencing, masking, or post-processing controls when feasible to reduce unnecessary capture of people, license plates, homes, backyards, and other sensitive details. Incidental capture should be managed according to the agency data policy and records retention schedule.

# 5. Federal and Oregon Legal Compliance

Legal review note: This section summarizes key legal requirements for program administration. It should be verified during agency adoption and annual review. It does not substitute for legal advice.

## 5.1 FAA Operating Authority

Agency UAS operations shall be conducted only under an FAA-recognized operating authority. The standard operating pathway for most agency missions is 14 CFR Part 107. The agency may use another pathway only when approved by the UAS Program Manager, legal counsel when appropriate, and the appropriate FAA process.

Authorized pathways may include:

- 14 CFR Part 107. The default operating authority for most small UAS operations under 55 pounds.

- Public Aircraft Certificate of Authorization. A public aircraft COA may be used when appropriate for qualifying governmental operations.

- Emergency COA. An emergency COA may be requested for qualifying urgent governmental operations when the operation cannot be completed under standard Part 107 authority.

- Special Government Interest Waiver. An SGI waiver may be requested for urgent operations that support a critical public good and require expedited FAA action.

- FAA waiver or exemption. Operations outside Part 107 limitations may require a Part 107 waiver, exemption under 49 U.S.C. § 44807, aircraft certification, or another FAA authorization.

The agency shall not assume that a governmental mission is automatically exempt from FAA rules. The Remote Pilot in Command shall verify the applicable operating authority before flight.

| Operating Authority | Use When | Program Requirements |
| --- | --- | --- |
| Part 107 | Routine small UAS missions under 55 pounds that can comply with Part 107 operating rules or an FAA Part 107 waiver. | FAA-certificated remote pilot; FAA registration; Remote ID when required; airspace authorization when required; compliance with Part 107 limitations and reporting. |
| Public aircraft COA | Governmental functions where public aircraft status is appropriate and the agency has an FAA COA covering the operation. | COA compliance; agency responsibility for pilot qualification, airworthiness, operational control, safety oversight, and any reporting required by the COA. |
| Emergency COA | Qualifying emergency operations requiring expedited FAA authorization. | Agency emergency authority, FAA coordination, defined mission, defined airspace, documentation, and post-operation reporting. |
| Contractor Part 107 | UAS services performed by a vendor under agency contract. | Contract terms must assign operational control, pilot qualifications, insurance, records, data, cybersecurity, public records, incident reporting, and compliance duties. |

## 5.2 Part 107 Baseline Requirements

The RPIC must ensure compliance with all applicable Part 107 requirements. At minimum, mission planning must address: remote pilot certificate and recency; aircraft registration; condition for safe operation; preflight inspection; medical fitness; visual line of sight; right-of-way to other aircraft; operations over people and vehicles; airspace authorization; airport vicinity; prohibited or restricted areas; NOTAMs and TFRs; operating limitations; accident reporting; and any waiver conditions.

Under 14 CFR 107.19, the RPIC must be designated before or during flight and is directly responsible for and the final authority over the small UAS operation. Under 14 CFR 107.9, reportable safety events must be reported to the FAA no later than 10 calendar days after a qualifying operation involving serious injury, loss of consciousness, or covered property damage.

## 5.3 Airspace, LAANC, NOTAMs, and TFRs

Before flight, the RPIC must verify airspace classification, UAS facility map status where applicable, airport proximity, heliports, special use airspace, military training routes, TFRs, NOTAMs, parachute operations, wildfire TFRs, stadium TFRs, and other temporary restrictions.

Controlled airspace operations must have an FAA authorization through LAANC or FAA DroneZone/manual authorization unless conducted under a COA that covers the operation.

The RPIC must carry or have electronic access to authorizations, waivers, COAs, and mission documents during the operation. Physical copies are preferred by the FAA.

Operations near wildfire, emergency response, medevac, law enforcement, military, or utility aviation require heightened deconfliction. When in doubt, do not launch until the airspace is coordinated.

- Incident Airspace, TFRs, and SGI Coordination

Before each flight, the Remote Pilot in Command shall check applicable airspace, NOTAMs, LAANC status, Temporary Flight Restrictions, special use airspace, military training routes when relevant, and local airport or heliport activity.

UAS operations inside an incident TFR shall not occur unless authorized by the FAA and coordinated with the official in charge of the on-scene emergency response activity. For urgent governmental operations, the agency may request an SGI waiver when the mission directly supports a critical public good and cannot reasonably wait for a standard authorization process.

For routine agency operations, SGI procedures should not be used as a convenience tool. SGI should be reserved for urgent circumstances such as emergency response, disaster response, wildfire support, search and rescue support, critical infrastructure protection, or other time-sensitive governmental missions.

If an agency UAS operation may affect incident aircraft, emergency response aircraft, medevac aircraft, firefighting aircraft, or other crewed aircraft, the UAS shall remain grounded until deconfliction has occurred.

## 5.4 FAA Registration, Marking, and Remote ID

Each agency UAS that must be registered with the FAA must be registered before flight and marked as required by FAA rules. FAA registrations should be tracked in the fleet inventory with expiration dates. UAS operations must also comply with Remote ID through a standard Remote ID aircraft, an FAA-accepted broadcast module, or another lawful method or exception. The RPIC must verify Remote ID compliance as part of preflight.

## 5.5 Oregon Public Body UAS Registration

ORS 837.360 requires a public body, other than an educational institution, to register a UAS with the Oregon Department of Aviation before operating it in the airspace over Oregon. Educational institutions register as a user of UAS rather than registering each individual aircraft under that statute. ODAV may impose a civil penalty of up to $10,000 for a violation, and evidence obtained in violation of the registration requirement is not admissible in judicial or administrative proceedings and may not be used to establish reasonable suspicion or probable cause.

OAR 738-080-0045 requires public bodies to register each UAS with the FAA before ODAV registration. ODAV registration requires the public body name, operator contact information, year, make, model, serial number, weight class, color, and FAA registration number. The rule currently lists fees of $25 for each UAS under 55 pounds and $50 for each UAS weighing 55 pounds or more. Educational institutions have separate user registration requirements and no ODAV registration fee under the rule.

## 5.6 Oregon Annual UAS Reporting

ORS 837.360 requires a public body that registers one or more UAS to provide an annual report to ODAV summarizing the frequency of use during the preceding calendar year, the purposes for which UAS were used, and how the public can access the agency policies and procedures established under ORS 837.362. OAR 738-080-0050 requires the annual written report to be submitted by January 31 for the preceding calendar year. The UAS Program Manager is responsible for preparing or coordinating this report unless the agency assigns that duty to another position.

## 5.7 Oregon UAS Data Policies

ORS 837.362 requires a public body operating a UAS to establish policies and procedures for the use, storage, accessing, sharing, and retention of data resulting from UAS operations, including video and audio recordings. The public body must post or otherwise make available those policies and procedures and the text of ORS 192.345. The policies must include retention time, specifications for third-party storage including handling, security, and access, and a policy on disclosure through intergovernmental agreements.

## 5.8 Oregon Public Records and Retention

UAS data and records created, received, or retained by a public body may be public records. The agency records officer must identify applicable retention schedules for flight logs, mission requests, imagery, video, maps, derived products, incident records, maintenance records, training records, registration records, and annual reports. Public records requests must be processed under agency procedures and Oregon Public Records Law. Potential exemptions should be evaluated record by record by the records officer and counsel.

## 5.9 Private Property, Critical Infrastructure, Parks, and Local Rules

UAS operations should avoid intentional, knowing, or reckless harassment or annoyance of owners or occupants of privately owned premises. Repeated overflights after notice from a property owner can create civil risk under Oregon law, subject to statutory exceptions and defenses.

UAS operations over or near critical infrastructure should be coordinated with the owner/operator where feasible, even when a public body exemption or other exception may apply. Do not rely on an exception when written coordination can be obtained.

Local governments should be aware that Oregon law generally preempts local regulation of UAS ownership or operation except as expressly authorized by statute. This manual is an internal agency policy and should not be used as a public regulatory code without counsel review.

Operations in parks, natural areas, wildlife areas, cultural sites, tribal lands, airports, schools, ports, utility facilities, and sensitive public spaces may require additional authorization, public notice, environmental review, or coordination.

## 5.10 Law Enforcement Restrictions

Optional law enforcement material: Law enforcement agencies and agencies providing support to law enforcement should review Annex B. Non-law-enforcement agencies should not adopt law enforcement operating language unless they actually support law enforcement operations and counsel approves the procedures.

# 6. Acquisition, Inventory, Registration, and Contractor Services

## 6.1 Acquisition Approval

No agency UAS, payload, Remote ID module, controller, flight software, cloud service, battery system, or mission-critical accessory may be purchased, leased, borrowed, accepted as a donation, or placed into service without approval by the UAS Program Manager and compliance with agency procurement and IT/security requirements.

The requesting program must describe the mission need, expected use, data type, required sensors, operating environments, training requirements, lifecycle costs, maintenance support, Remote ID compliance, cybersecurity considerations, and whether grant or federal restrictions apply.

The UAS Program Manager should evaluate compatibility with existing fleet, pilot training, maintenance capacity, spare parts, data systems, and agency risk tolerance.

The IT/security lead should review manufacturer cloud services, mobile applications, account structures, firmware update pathways, telemetry storage, geofencing dependencies, data export controls, and authentication methods.

The records officer should review data outputs, metadata, retention, public records implications, and how data will be stored and searched.

## 6.2 Inventory and Registration Tracker

The UAS Program Manager must maintain an inventory for all agency UAS and required Remote ID equipment. The inventory should include at least: year, make, model, serial number, aircraft type, weight, color, FAA registration number and expiration, ODAV registration and expiration, Remote ID serial number, assigned unit, assigned pilots, maintenance status, battery sets, payloads, software/firmware version, and disposal status.

## 6.3 Marking and Documents

Each UAS must be marked with the FAA registration number in a manner required by FAA rules. The agency should also label aircraft cases and batteries with agency asset numbers. Registration documents, airspace authorizations, waivers, COAs, aircraft manuals, checklists, and emergency contact information must be accessible to the RPIC during operations.

## 6.4 Contractor UAS Services

Contracted UAS services must be reviewed before use. The contract, purchase order, intergovernmental agreement, or task order should specify operational control, operating authority, pilot qualifications, aircraft registration, Remote ID compliance, insurance, safety plan, site coordination, data ownership, public records duties, retention, cybersecurity, incident reporting, indemnity (insurance), and deliverables.

Before using a contractor UAS operator, the agency shall determine whether the agency or the contractor has operational control of the flight. The contract, scope of work, purchase order, or task order should identify:

- The operating authority for the flight.

- The Remote Pilot in Command.

- The entity responsible for airspace authorization.

- The entity responsible for aircraft registration, Remote ID compliance, and aircraft marking.

- The entity responsible for maintenance records and airworthiness.

- The entity responsible for data handling, storage, retention, cybersecurity, and public records compliance.

- The entity responsible for incident, accident, or mishap reporting.

- The process for stopping, delaying, or modifying operations due to safety concerns.

Contractor aircraft and pilots shall meet applicable federal, state, local, agency, and contract requirements. The agency may request proof of pilot certification, aircraft registration, Remote ID compliance, insurance, maintenance records, data security practices, and flight logs before, during, or after the operation.

| Contract Term | Minimum Requirement |
| --- | --- |
| Operational control | Identify who has authority over launch, flight, termination, mission changes, and emergency actions. |
| Pilot qualifications | Require Part 107 remote pilot certificate and any agency-required platform or mission qualifications. |
| Aircraft compliance | Require FAA registration, Remote ID compliance, manufacturer maintenance, and appropriate condition for safe operation. |
| Insurance | Require coverage appropriate to mission risk, including aviation/UAS liability when available. |
| Data and public records | State that data created for the agency may be a public record and must be delivered, stored, secured, retained, and disposed according to agency requirements. |
| Privacy and collection limits | Limit collection to mission scope and require prompt deletion or segregation of incidental data if approved by records policy. |
| Incident reporting | Require immediate notification of injury, property damage, flyaway, lost link, lost aircraft, battery fire, data loss, public complaint, law enforcement contact, or FAA/NTSB reportable events. |

# 7. Personnel Qualifications, Training, and Currency

## 7.1 Remote Pilot Minimum Qualifications

Hold a current FAA Remote Pilot Certificate with a small UAS rating, or operate only under the direct supervision of a certificated RPIC who can immediately take direct control, if permitted for the training event.

Complete agency UAS orientation, this manual review, privacy/data training, records awareness, emergency response briefing, and platform-specific training before acting as RPIC for agency missions.

Demonstrate platform proficiency for each aircraft family before unsupervised operations.

Maintain FAA aeronautical knowledge recency and agency currency requirements.

Self-ground when the pilot has a physical or mental condition, fatigue, medication, stress, distraction, or workload issue that could interfere with safe operation.

## 7.2 Visual Observer Minimum Qualifications

Complete agency VO training before participating in operational missions.

Understand scanning technique, airspace awareness, crew communications, lost link procedures, emergency landing zones, ground hazard monitoring, and how to communicate clearly with the RPIC.

Be able to maintain awareness of the unmanned aircraft, surrounding airspace, obstacles, people, animals, vehicles, and mission hazards.

Avoid distractions unrelated to the VO role during critical phases of flight.

## 7.3 Training Curriculum

| Training Area | RPIC | VO | Payload Operator | Program Staff |
| --- | --- | --- | --- | --- |
| Agency UAS manual and policy | Required | Required | Required | Required |
| FAA Part 107 / operating rules | Required | Awareness | Awareness | Awareness |
| Airspace, LAANC, NOTAMs, TFRs | Required | Awareness | Awareness | Awareness |
| Aircraft systems and preflight | Required | Awareness | As needed | As needed |
| Emergency procedures | Required | Required | Required | Awareness |
| Crew Resource Management / Threat and Error Management | Required | Required | Recommended | Recommended |
| Privacy, public records, retention, and data handling | Required | Required | Required | Required |
| Mission planning and risk assessment | Required | Recommended | Recommended | Recommended |
| Maintenance and battery safety | Required | Awareness | Awareness | As needed |
| Optional public safety annex | If assigned | If assigned | If assigned | If assigned |
| Optional law enforcement annex | Only if assigned and authorized | Only if assigned and authorized | Only if assigned and authorized | Only if assigned and authorized |

## 7.4 Currency

The agency should set a clear and auditable currency standard. The following is a suggested baseline that may be modified based on mission risk and agency size:

RPICs must complete FAA recurrent training or otherwise satisfy FAA aeronautical knowledge recency as required.

RPICs must complete at least three takeoffs and landings with each assigned aircraft family within the preceding 90 days before conducting unsupervised operational missions, or complete a supervised proficiency flight.

RPICs assigned to higher-risk missions should log at least [10] flight hours on the assigned platform or similar make/model before unsupervised mission work. The agency may allow up to [5] hours of simulator time if appropriate.

Each RPIC must complete an annual agency UAS policy, privacy, safety, and emergency procedures review.

VOs must complete annual refresher training and participate in at least one briefing or drill each year.

## 7.5 Training Records

The UAS Program Manager or Training Officer must maintain a master file for each RPIC, VO, payload operator, maintenance designee, and other assigned crewmember. The file should include certificates, training dates, platform checkouts, test results, currency, flight logs, incident involvement, corrective actions, and expiration dates.

## 7.6 Duty Time, Fatigue, and Fitness for Flight

Remote pilots, visual observers, payload operators, and other UAS crewmembers shall be fit for duty before participating in flight operations. A person may not serve as Remote Pilot in Command, visual observer, or required crewmember when the person knows or has reason to know of a physical, mental, medical, fatigue-related, or other condition that would interfere with safe operation.

The agency may establish stricter internal duty, rest, and flight-time limits based on mission type, environmental conditions, operational tempo, and risk. For extended operations, emergency response operations, wildfire support, disaster response, field deployments, or training events, the UAS Program Manager or Incident Supervisor should evaluate fatigue risk and may reduce flight periods, rotate pilots, require rest periods, or suspend operations.

# 8. Safety Management System

The agency UAS program will use a scalable Safety Management System built around four components: safety policy, safety risk management, safety assurance, and safety promotion. The purpose is to identify hazards, reduce risk, learn from operations, and create a culture where personnel can raise safety concerns without fear of retaliation for good-faith reporting.

| SMS Component | Agency Practice |
| --- | --- |
| Safety policy | Adopt this manual, define authority, assign roles, set training and reporting expectations, and make safety a program responsibility. |
| Safety risk management | Use mission planning, site surveys, risk assessments, crew briefings, weather and airspace review, and mitigation before flight. |
| Safety assurance | Review flight logs, maintenance logs, incidents, near-misses, deviations, public complaints, audits, and corrective actions. |
| Safety promotion | Share lessons learned, provide refresher training, brief hazards, encourage reporting, and review changes in law, technology, and operating environments. |

## 8.1 Hazard Identification

Hazards should be considered in three broad categories: aircraft and equipment, operating environment, and human factors. Hazards should be documented in the Project Aviation Safety Plan or risk worksheet when they are mission-specific, and in the program risk register when they are recurring.

| Hazard Category | Examples |
| --- | --- |
| Aircraft and equipment | Battery condition, propellers, motors, firmware, payload mount, Remote ID, compass/GNSS performance, control link, sensor failure, ground control station, device overheating, software updates. |
| Operating environment | Airspace, weather, wind, precipitation, smoke, dust, darkness, terrain, wires, towers, trees, people, vehicles, wildlife, private property, public events, emergency aircraft, wildfire TFRs, RF interference, magnetic interference. |
| Human factors | Fatigue, workload, distraction, normalization of deviance, overreliance on automation, poor crew communication, pressure to complete the mission, loss of situational awareness, inadequate briefing, unclear authority. |

## 8.2 Risk Assessment Matrix

| UAS Risk Assessment Worksheet |  |  |  |  |
| --- | --- | --- | --- | --- |
| Assess the risks involved with the proposed operations. Use additional sheets if necessary. |  |  |  |  |
| Risk Assessment Matrix |  |  |  |  |
|  | Severity |  |  |  |
| Likelihood | Negligible IV | Marginal III | Critical II | Catastrophic I |
| Frequent A |  |  |  |  |
| Probable B |  |  |  | HIGH 4 |
| Occasional C |  |  | Serious 3 |  |
| Remote D |  | Medium 2 |  |  |
| Improbable E | LOW 1 |  |  |  |

| Severity Scale Definitions |  |
| --- | --- |
| Catastrophic | Results in fatalities and/or loss of system |
| Critical | Severe injury and/or major system damage |
| Marginal | Minor injury and/or minor system damage |
| Negligible | Less than minor injury and/or less than minor system damage. |

| Likelihood Scale Definition |  |
| --- | --- |
| Frequent | Likely to occur often |
| Probable | Will occur several times |
| Occasional | Likely to occur sometime |
| Remote | Unlikely to occur, but possible |
| Improbable | So unlikely, it can be assumed it will not occur |

| Assess the risks involved with the proposed operation. Use additional sheets if necessary. |  |  |  |
| --- | --- | --- | --- |
| Describe the Hazard: | Pre-Mitigation hazards rate out as: |  |  |
|  | Likelihood A-E | Severity I-IV | Risk Level |
| - Mid-air collision with another aircraft | D | I | 3 |
| - Collision with personnel | C | II | 3 |
| - Collision with vehicles | B | II | 4 |
| - UAS operating outside of approved area | B | II | 3 |
| - UAS operating outside of performance envelope | D | II | 3 |
| - Fire | D | II | 2 |
| - Loss of link | C | II | 3 |
| - Laceration from spinning blades | C | II | 3 |
| - Wildlife such as eagles or seagulls | C | III | 2 |
| - Collision with obstruction | C | II | 3 |
| - Air intrusion | B | II | 1 |
| - Ground intrusion | C | II | 1 |
| - Adverse Weather | C | II | 3 |

Sample risk assessment

Pre-mitigation Overall Rating:

| Mitigations: | Post-Mitigation hazards rate out as: |  |  |
| --- | --- | --- | --- |
|  | Likelihood A-E | Severity I-IV | Risk Level |
| - NOTAM will be filed and local NOTAMS checked. Deconfliction will be done on scene using visual observers and situational awareness. | E | I | 2 |
| - Flight patterns will be planned to avoid people on the ground during takeoff and landings. Non-participating people will remain clear of the ground control station so as to not distract the operators. Takeoff/landing areas will be established to minimize risk. Overflight of the personnel be minimized. Observers will be kept at a safe distance from the launch/recovery area and out of the flight path of the aircraft. | D | II | 2 |
| - Vehicles will be parked clear of approach and departure routes. Flight pattern will avoid non-participating vehicles. | E | IV | 1 |
| - Boundaries will briefed and maps will be reviewed. Geo-fencing employed if required. | D | IV | 1 |
| - RPIC will verify that all control surfaces are with operational limitations immediately after take-off and before beginning operation. | C | IV | 1 |
| - RPIC will be immediately notified of fire. Fire extinguisher on site, and ERP plan reviewed. | E | II | 1 |
| - In case of loss link and the UAS does not return to launch and leaves the defined operating area as described in the NOTAM, the UAS will be considered a fly-away. The appropriate ATC having jurisdiction over the airspace will be notified with the last location, heading, speed, and approximate battery/time remaining of the UAV. | C | IV | 1 |
| - Checklist procedures will be followed to ensure that personnel stay clear of rotating blades. | D | II | 2 |
| - Personnel and VOs will be briefed on the hazard and be advised to be on the lookout for hazardous wildlife. | C | III | 2 |
| - Prior to operating, the RPIC and VOs will conduct a walkthrough to check for obstructions and mark them on a map. | E | IV | 1 |
| - VOs will monitor airspace, and CTAF will be monitored if flying within controlled airspace. Communication protocols in place. | E | I | 2 |
| - VOs and crew will monitor ground for any ground based intrusions. Communication protocols in place. | E | I | 2 |
| - Flight crew will monitor weather and ensure UAS is flown within Part 107 requirements. | C | II | 2 |
| Post-Mitigation Overall Rating: |  |  | 2 |

Missions rated Critical may not proceed without UAS Program Manager approval and documented mitigation. Missions rated High require documented mitigation and RPIC concurrence. Missions rated Medium require mitigation or acceptance by the RPIC and mission lead. Missions rated Low may proceed under standard procedures if all required checks are complete.

## 8.3 Check 3 GPS Method

As a practical field tool, crews should use a “Check 3 GPS” habit before each operation:

Check Gear: aircraft, batteries, propellers, payload, controller, Remote ID, memory cards, firmware status, first aid kit, fire extinguisher, PPE, and documents.

Check Plan: mission objective, site, airspace, weather, NOTAMs/TFRs, permissions, launch/recovery, emergency landing zones, communications, data plan, and public interaction plan.

Check Skills: RPIC currency, VO readiness, platform proficiency, fatigue, workload, crew roles, abnormal procedures, and authority to stop the operation.

## 8.4 Stop-Work / Stop-Flight Authority

Any crewmember may call for a pause or stop if they observe a condition that may affect safety. The RPIC must evaluate the concern and may not continue until the concern has been resolved, mitigated, or documented as acceptable. Stop-flight authority applies before launch, during flight, during battery change, during data collection, and during postflight handling if a safety or data security issue exists.

## 8.5 Voluntary and Mandatory Reporting

Mandatory reports include injuries, property damage, flyaways, lost aircraft, lost link with unsafe behavior, collisions, fire, battery thermal events, emergency landings, public complaints, data loss, FAA/NTSB reportable events, law enforcement contact, or deviations from authorization.

Voluntary reports may include hazards, near-misses, unclear procedures, equipment concerns, fatigue concerns, public interaction concerns, or suggestions for safer operations.

Good-faith safety reporting should be treated as non-punitive unless the report reflects intentional misconduct, gross negligence, falsification, reckless operation, or knowing violation of law or policy.

## 8.6 Aviation Risk Assessment Questions

- Before conducting a UAS operation, the Remote Pilot in Command and mission lead should consider the following questions:

- Is the flight necessary?

- Who is in charge of the mission?

- Have all known hazards been identified and communicated?

- Should the operation be stopped because of changes in communications, weather, confusion, equipment, competing priorities, or personnel readiness?

- Is there a safer or more effective way to complete the mission?

- Is the mission being driven by an inappropriate sense of urgency?

- Can the planned action be justified if reviewed later?

- Are there crewed or uncrewed aircraft in the area?

- Does the operation have a safe exit, landing, lost-link, or termination plan?

- Would the operation violate any law, rule, authorization, waiver, policy, or manufacturer limitation?

- Are communications becoming tense, unclear, or unreliable?

- Is the operation deviating from the approved mission plan?

- If the answer to any question creates a safety concern, the Remote Pilot in Command shall delay, modify, or stop the operation until the concern is resolved.

# 9. Mission Request, Authorization, and Planning

## 9.1 Mission Request

Every operational mission must begin with a defined public purpose. The mission requester or project lead should submit a mission request using the form in Appendix F or an approved electronic equivalent. Routine training in designated practice areas may be approved through a standing training plan.

## 9.2 Project Aviation Safety Plan

A Project Aviation Safety Plan (PASP) is required for missions outside designated practice areas, missions involving contractors, operations near controlled airspace or airports, operations near public events, operations involving multiple aircraft or crews, missions with public safety involvement, or any mission rated Medium or higher after initial risk screening.

| PASP Element | Required Information |
| --- | --- |
| Mission objective | Clear statement of public purpose and intended deliverables. |
| Dates and location | Operational period, staging area, launch/recovery, mission area, coordinates, maps. |
| Operating authority | Part 107, waiver, COA, ECOA, or contractor authority. |
| Crew | RPIC, VO, payload operator, mission lead, trainee, contractor, safety officer. |
| Aircraft and payload | Make/model, registration, Remote ID, payload, batteries, mission-specific equipment. |
| Airspace | Class, LAANC/manual authorization, NOTAMs, TFRs, airports, heliports, special use airspace, frequencies. |
| Site hazards | Wires, towers, terrain, people, vehicles, wildlife, trees, water, construction, RF/magnetic interference, public access. |
| Risk mitigation | Altitude limits, geofence, VO placement, sterile area, signage, landing zones, communication plan, weather minimums, emergency plan. |
| Data plan | Data to collect, storage, access, retention, public records, third-party systems, intergovernmental sharing. |
| Notifications and coordination | Landowner/facility coordination, airport/ATC if required, incident command, public affairs, law enforcement if applicable, contractors. |

The Project Aviation Safety Plan should address, at minimum:

- Mission objective and operational need.

- Agency, division, program, project manager, and UAS Program Manager contact information.

- Remote Pilot in Command and assigned crew.

- Aircraft, payload, Remote ID status, FAA registration, and Oregon public body registration status when applicable.

- Operating authority, airspace authorization, LAANC approval, COA, SGI waiver, or other FAA authorization when required.

- Location, coordinates, operating area, launch and recovery sites, emergency landing areas, and map attachments.

- Land ownership or site-control status, including permission for launch and recovery when needed.

- Nearby airports, heliports, seaplane bases, private strips, helipads, special use airspace, military training routes, TFRs, NOTAMs, and known aviation hazards.

- Weather minimums, lighting conditions, terrain, ground hazards, utility lines, towers, roadways, railways, waterways, public access areas, wildlife, and environmental considerations.

- Communications plan, including crew communications, emergency contacts, and airspace coordination contacts when needed.

- Data objective, data type, payload settings, data storage location, retention category, and public records considerations.

- Privacy controls, including measures to avoid unnecessary collection of uninvolved persons, occupied structures, or unrelated private activity.

- Site control measures, including signage, cones, barriers, visual observers, public contact plan, and ground hazard mitigation.

- Emergency procedures for lost link, flyaway, low battery, crash, injury, fire, intrusion, or crewed-aircraft conflict.

- Postflight documentation, including flight logs, maintenance discrepancies, incident reports, data intake, and retention entries.

## 9.3 Site Survey

The RPIC or designated planner should conduct a remote site assessment before the mission and an onsite survey before flight. The survey should identify launch and recovery sites, emergency landing zones, VO positions, public access controls, crew staging, obstacles, RF or magnetic interference, privacy-sensitive areas, wildlife or livestock, roads and rail, overhead utilities, heliports, airports, and areas to avoid.

## 9.4 Crew Briefing

Before each operational period, the RPIC must conduct a crew briefing. The briefing should include roles, mission objective, aircraft and payload, airspace, weather, launch/recovery, flight boundaries, lost link, emergency procedures, communications, sterile cockpit expectations, public interaction plan, data plan, stop-flight authority, and any crew concerns.

## 9.5 Public Interaction and Site Control

The agency should manage public interaction in a professional and transparent manner. When feasible, use signage, cones, vests, a designated public contact, or advance notice to explain that a UAS operation is underway. The RPIC should not be the primary public contact while the aircraft is in flight. If a member of the public raises a concern, the crew should listen respectfully, avoid argument, document the concern, and elevate it after the flight unless safety requires immediate action.

# 10. Standard Flight Operations Procedures

## 10.1 Day-Before or Pre-Mission Review

Confirm mission approval, public purpose, site access, operating authority, registration, Remote ID, pilot assignment, VO assignment, aircraft status, insurance or contract coverage, and data handling plan.

Review airspace, LAANC/manual authorization, NOTAMs, TFRs, weather, obstacles, launch/recovery area, emergency landing zones, and communications.

Confirm that batteries are charged and balanced, firmware changes have been assessed, memory cards are available, and required documents are accessible.

Confirm that flight logs, risk worksheet, PASP, and incident report form are available in paper or electronic form.

## 10.2 Site Setup

Establish launch/recovery area with clear visibility, sufficient space, and protection from public intrusion.

Identify emergency landing zones and avoid zones.

Position VOs to cover the mission area without gaps in communication.

Check for wires, towers, trees, antennas, water, moving vehicles, pedestrians, animals, terrain, construction equipment, and other hazards.

Assess RF and magnetic interference when operating near metal structures, vehicles, power infrastructure, urban canyons, canyons, forests, or communications sites.

Verify final weather, airspace, NOTAMs, TFRs, and emergency aircraft activity before launch.

## 10.3 Preflight

Verify RPIC, crew, and mission authority.

Inspect airframe, arms, landing gear, propellers, motors, payload mount, sensors, gimbal, lights, Remote ID, batteries, controller, mobile device, antennas, memory card, and software status.

Verify aircraft registration, Remote ID status, compass/IMU/GNSS status, home point, return-to-home altitude, battery warnings, lost-link settings, geofence settings, and obstacle sensing configuration.

Review control inputs and verify positive control response.

Verify that the aircraft is in condition for safe operation and that the mission remains within authorized limitations.

Complete crew briefing and document the preflight check.

## 10.4 Launch and Flight

Announce launch and confirm crew readiness.

Climb to a safe check altitude and verify stability, control link, video link, telemetry, battery status, GNSS, and Remote ID.

Maintain visual line of sight unless operating under an authorization that permits otherwise.

Yield right-of-way to all crewed aircraft and discontinue the operation or land when required.

Maintain sterile cockpit during takeoff, landing, emergencies, low altitude flight near obstacles, lost link recovery, and other high workload periods.

Maintain mission boundaries and avoid unnecessary collection outside the mission area.

Monitor battery, weather, people, vehicles, air traffic, lost link warnings, geofence warnings, sensor warnings, and crew communications.

Do not continue a mission when aircraft condition, airspace, weather, crew readiness, public safety, data compliance, or mission conditions no longer support safe operation.

## 10.5 Landing and Postflight

Announce landing and confirm landing zone is clear.

Power down aircraft and controller according to manufacturer instructions.

Inspect aircraft, propellers, payload, batteries, and controller for damage, overheating, debris, loose components, or abnormalities.

Secure data, batteries, aircraft, and accessories.

Complete flight log, battery log, maintenance notes, data transfer, and post-operation debrief.

Document hazards, near-misses, deviations, public complaints, unusual events, and lessons learned.

## 10.6 Abnormal and Contingency Procedures

| Condition | Expected Response |
| --- | --- |
| Manned aircraft or emergency aircraft observed | Descend or land immediately if required to maintain separation. Announce to crew. Do not relaunch until safe. |
| Lost link | Follow pre-briefed lost link procedure and RTH settings. Monitor for expected behavior. Notify crew. Prepare emergency landing or recovery. |
| Low battery | Return and land before critical battery. Use contingency landing zone if primary landing zone is not reachable. |
| GNSS/compass issue | Stop mission. Transition to appropriate mode only if trained and safe. Land as soon as practical. |
| Flyaway or loss of control | Warn crew and people on ground. Attempt recovery if safe. Activate RTH or flight termination only according to training and when necessary to protect people or aircraft. Report event. |
| Battery fire or thermal runaway | Move people away, use appropriate fire extinguisher or containment if trained, call 911 if needed, preserve evidence, report. |
| Public enters launch/recovery area | Pause launch or abort landing as appropriate. Use VO or site lead to clear area. |
| Loss of video feed | Continue only if the RPIC can maintain safe flight and mission does not depend on payload view for safety. Otherwise return and land. |
| Controller or app failure | Use backup device or lost link/RTH procedures as trained. Land when safe. |
| Weather deterioration | Return and land before limits are exceeded. Document condition and decision. |

Flyaway, Lost Link, or Loss of Visual Contact

If the aircraft experiences a flyaway, lost command-and-control link, loss of visual contact, or other unrecoverable abnormal condition, the Remote Pilot in Command shall:

- Announce the emergency to the crew.

- Attempt to regain control using manufacturer procedures and the approved lost-link procedure.

- Notify any affected aircraft, airspace coordinator, dispatch, flight following contact, or incident supervisor when applicable.

- Clear the affected operating area when necessary.

- Suspend nearby flight operations if continued operation may create a hazard.

- Wait for the expected remaining battery or fuel duration when needed before beginning recovery.

- Search for the missing aircraft only when it can be done safely and lawfully.

- Preserve evidence and flight logs when an accident, injury, property damage, or regulatory reporting event may have occurred.

- Complete required internal, FAA, NTSB, ODAV, insurer, and interagency reports as applicable.

# 11. Emergency Response, Mishaps, and Reporting

## 11.1 Priority of Actions

When an accident, incident, crash, flyaway, injury, fire, or serious anomaly occurs, the crew should follow these priorities:

Start a log of actions, times, calls, and decisions as soon as practical.

Assess the situation and determine whether there is danger to people, aircraft, property, responders, or the public.

Protect people. Life safety takes priority over aircraft, data, and property.

Protect property from unnecessary further damage when safe to do so.

Preserve evidence. Do not disturb wreckage, batteries, memory cards, controller logs, or site conditions unless needed for safety or to prevent further damage.

Notify through the agency chain of command and required external reporting pathways. Do not delay required notification because complete details are not yet available.

Recover equipment only when safe and when release or documentation is complete.

Complete post-incident reports, corrective actions, and safety review before returning the aircraft or crew to mission status if required.

## 11.2 Immediate Notifications

| Event | Notify |
| --- | --- |
| Injury, serious injury, fire, threat to public, hazardous material, or emergency | 911 first, then agency supervisor and UAS Program Manager. |
| FAA Part 107 safety event | FAA within 10 calendar days when criteria are met. |
| NTSB accident or listed serious incident | NTSB immediately and by the most expeditious means when 49 CFR Part 830 criteria are met. |
| Lost aircraft, flyaway, unsafe lost link, collision, or substantial property damage | Agency supervisor, UAS Program Manager, property owner if applicable, FAA/NTSB if reportable. |
| Public complaint or privacy concern | UAS Program Manager, Public Information Officer or records/privacy officer, and supervisor. |
| Data loss, device loss, unauthorized disclosure, or suspected compromise | IT/security lead, records officer, UAS Program Manager, and agency incident response process. |
| Law enforcement operation or evidence concern | Law enforcement chain of command, agency counsel, records/evidence officer, and Annex B procedures. |

For operations conducted with another agency, contractor, emergency response partner, dispatch center, or incident command structure, the Remote Pilot in Command or mission lead shall also notify the designated operational contact for that entity. If the event occurred during interagency aviation operations, wildfire operations, incident response, or other aviation-managed operations, the agency should determine whether a SAFECOM or equivalent aviation safety report is required or appropriate.

## 11.3 Return-to-Service After Incident

An aircraft involved in a crash, hard landing, propeller strike, motor anomaly, battery event, flyaway, abnormal lost link, water exposure, significant software anomaly, or suspected structural issue may not be returned to service until inspected, repaired if needed, test flown in a safe environment if appropriate, and released by the UAS Program Manager or Maintenance Coordinator. The release must be documented in the maintenance log.

## 11.4 Emergency Drills

The UAS Program Manager should conduct at least one annual emergency response drill or tabletop exercise. The drill should include lost link, crash with injury, battery fire, public complaint, and data incident components. Public safety agencies should integrate UAS emergency procedures into incident command training when UAS are used at incidents.

# 12. Data Management, Privacy, Cybersecurity, and Public Records

## 12.1 Data Governance Principles

Collect only what is reasonably necessary for the authorized mission.

Identify expected data products before flight, including still imagery, video, thermal imagery, LiDAR, orthomosaics, point clouds, telemetry, flight logs, and derived maps.

Assign a data steward before the mission when data will be retained or shared.

Store data only in approved agency systems unless temporary field storage is necessary and documented.

Restrict access based on agency role, mission need, public records obligations, and applicable confidentiality requirements.

Retain and dispose of data under the agency records retention schedule and legal hold requirements.

Document third-party storage, contractor platforms, cloud systems, and intergovernmental sharing as required by ORS 837.362.

- Agency UAS data shall be collected only for an approved agency purpose. UAS operators and observers shall take reasonable steps to minimize unnecessary collection of uninvolved persons, occupied structures, private activities, license plates, or other information unrelated to the approved mission.

- When practicable, cameras and sensors should be oriented away from occupied structures, private gatherings, or unrelated activity. The agency shall not conduct random surveillance. UAS data shall be stored, secured, retained, disclosed, and destroyed in accordance with agency information technology policies, cybersecurity policies, public records laws, records retention schedules, and the agency’s posted UAS data policy.

## 12.2 Default Data Retention Framework

The agency must select retention periods consistent with Oregon public records requirements and agency retention schedules. The following framework is a policy starting point and must be customized:

| Data Type | Suggested Handling | Retention Placeholder |
| --- | --- | --- |
| Administrative records | Mission requests, approvals, PASPs, risk assessments, authorizations, waivers, registrations, annual reports. | [Agency schedule / insert citation] |
| Flight logs and telemetry | Retain enough to verify compliance, maintenance, safety assurance, and annual reporting. | [Agency schedule / insert period] |
| Routine imagery/video with no ongoing business need | Review, retain deliverables, dispose of raw data when allowed. | [Insert period, e.g., 30-180 days] |
| Project deliverables | Orthomosaic, inspection images, engineering maps, reports, asset records. | [Agency project record schedule] |
| Incidental PII or private property images | Limit access, segregate if needed, delete when no longer required and allowed by retention schedule. | [Insert period] |
| Public safety incident data | Handle under incident, emergency management, fire, or evidence-related schedules if applicable. | [Insert schedule] |
| Law enforcement data | Handle under Annex B, evidence rules, public records, court orders, and law enforcement retention schedules. | [Insert schedule] |

## 12.3 Website Posting and Public Availability

Before operational use, the agency should post or otherwise make available the UAS data policies and procedures required by ORS 837.362 and the text of ORS 192.345. The posting should also identify a public point of contact, public records request instructions, and the annual UAS report location if the agency posts annual reports.

## 12.4 Third-Party Storage and Software

Third-party UAS data systems, cloud processing tools, mapping platforms, vendor portals, and mobile applications must be reviewed by IT/security and procurement before use.

Contracts must identify where data will be stored, who may access it, security controls, breach notification, deletion or return requirements, public records support, audit rights, subcontractors, and intergovernmental sharing restrictions.

Field crews should avoid uploading agency data to personal accounts, unapproved cloud services, social media, or vendor systems not approved for the mission.

## 12.5 Public Records Requests

Personnel receiving a request for UAS data or records must promptly route the request to the agency public records coordinator. Crewmembers should not release data directly unless authorized by agency public records procedures. The agency should preserve potentially responsive data while the request is processed and should coordinate with counsel when exemptions, privacy, safety, law enforcement, critical infrastructure, cybersecurity, tribal, or third-party rights may be implicated.

# 13. Maintenance and Continued Airworthiness

## 13.1 General

The agency must operate only UAS that are in a condition for safe operation. The RPIC must perform or verify a preflight inspection before each flight. Aircraft must be maintained in accordance with manufacturer guidance. If manufacturer guidance is incomplete, the agency should use documented industry best practices and conservative inspection intervals.

## 13.2 Maintenance Records

Each UAS must have a maintenance log that records inspections, damage, repairs, firmware/software changes, hardware changes, battery events, payload changes that affect performance, test flights, anomalies, and return-to-service approvals. Records should include date, aircraft identifier, person performing maintenance, description, parts replaced, software/firmware version, test flight results, and return-to-service decision.

## 13.3 Preflight and Periodic Inspection

| Inspection Type | Minimum Content |
| --- | --- |
| Preflight | Airframe, propellers, motors, landing gear, payload, mounts, controller, antennas, batteries, Remote ID, software status, memory, compass/GNSS/IMU, failsafe settings, RTH altitude, control inputs, launch area. |
| Daily or operational period | Flight critical parts, battery set condition, controller devices, payload mounting, case damage, spares, fire extinguisher, first aid kit, documentation. |
| Weekly or monthly program check | Software update review, firmware inventory, battery cycle review, wiring, corrosion, propeller/motor condition, lost-link behavior in a safe simulated environment if appropriate. |
| Post-maintenance test flight | Required after changes or repairs that could affect flight performance, control, sensors, power, payload, software, or failsafe behavior. |

Each agency-owned UAS shall receive a documented periodic inspection at least annually, or more frequently if required by the manufacturer, agency policy, operational tempo, repair history, or mission risk. The inspection should include, at minimum:

- Confirmation that the aircraft configuration conforms to the manufacturer’s design or approved agency configuration.

- Airframe condition and serviceability.

- Propellers, motors, arms, landing gear, payload mounts, and attachment points.

- Ground Control Station condition and serial number.

- Aircraft serial number, FAA registration, Remote ID status, and Oregon public body registration status when applicable.

- Firmware, software, geofencing, Remote ID, and configuration status.

- Battery condition, cycle count, charging equipment, and battery storage condition.

- Preflight checklist completion.

- System diagnostics.

- Ground motor run or equivalent system check.

- Payload, sensor, charger, case, and peripheral equipment status.

- Missing, damaged, or unserviceable kit items.

- Maintenance discrepancies shall be documented and corrected before further flight unless the Remote Pilot in Command determines the discrepancy does not affect safe operation and the aircraft remains airworthy under applicable regulations, manufacturer limitations, and agency policy.

## 13.4 Batteries

Use only approved batteries and chargers.

Track battery identification, cycle count, condition, swelling, damage, storage voltage, temperature, and retirement.

Do not use damaged, swollen, overheated, over-discharged, or questionable batteries.

Transport and store batteries in accordance with manufacturer guidance and agency safety policy.

Keep an appropriate fire extinguisher, containment method, or fire response plan available during charging and operations.

## 13.5 Firmware, Software, and Configuration Changes

Firmware and software changes can alter aircraft behavior, failsafe settings, Remote ID, geofencing, flight modes, sensor function, or data handling. Updates should be reviewed before installation when feasible, documented in the maintenance log, and followed by a controlled test flight if the update affects flight performance or critical systems. The agency should avoid installing major updates immediately before operational missions unless required for safety or compliance.

# 14. Interagency Operations and Mutual Aid

Interagency operations require clear authority, clear operational control, compatible communications, common safety expectations, and a shared understanding of data ownership and public records responsibilities. Before a joint mission, participating agencies should identify who owns the mission, who controls the flight, who may task the aircraft, who may receive data, who handles public records requests, and who reports incidents.

Use written agreements, mission tasking documents, incident action plans, or PASPs when time allows.

Identify a single RPIC for each aircraft and a single airspace coordination point for multi-aircraft or incident operations.

For emergency management or public safety incidents, integrate UAS into the incident command structure and deconflict with crewed aircraft.

For contractor or mutual aid operations, confirm insurance, registration, Remote ID, operating authority, training, data handling, and incident reporting before launch.

When [AGENCY NAME] receives UAS data from another public body, the records officer should determine whether it becomes an agency record and how it must be retained or disclosed.

# 15. Optional Annex A: Public Safety and Emergency Operations

Optional annex: Adopt this annex only if the agency conducts public safety, emergency response, fire, search and rescue, disaster response, incident management, or emergency management missions. Agencies that do not conduct these operations should delete or mark this annex “Not adopted.”

## 15.1 Purpose

This annex provides supplemental procedures for UAS support to public safety and emergency operations. It does not create law enforcement authority. Law enforcement operations must follow Annex B and ORS 837.310 to 837.345.

## 15.2 Public Safety Mission Types

Drones as a First Responder

Search and rescue support, subject to incident command and applicable authority.

Fire support, wildfire damage assessment, structure fire overwatch, fire perimeter mapping, and post-fire documentation.

Disaster damage assessment for floods, storms, earthquakes, landslides, hazardous access, infrastructure failure, or evacuation route assessment.

Hazardous materials or dangerous scene assessment from a safe standoff distance, with appropriate sensor limitations and responder coordination.

Incident mapping, overwatch, public information support, and resource coordination when integrated into incident command.

## 15.3 Incident Command Integration

UAS operations at incidents should be requested, assigned, and deconflicted through the Incident Commander or designated air/operations function.

The RPIC retains final authority over flight safety even when operating under incident command tasking.

Crewed aircraft have priority. UAS operations must be suspended when deconfliction cannot be assured.

UAS crews should monitor assigned incident communications, CTAF/UNICOM or other aviation frequencies when appropriate and lawful, and any air operations briefings applicable to the incident.

UAS data should be delivered through approved incident data pathways and retained under applicable incident records schedules.

## 15.4 Emergency COA and Special Authorization

When an emergency mission requires operations outside Part 107 or existing authorizations, the agency may seek an Emergency COA or other FAA authorization if eligible. The request should be coordinated through the agency official authorized to request emergency airspace support. The agency must document the emergency need, location, altitude, operating period, crew, aircraft, deconfliction plan, and reporting requirements.

Optional: Incident TFR and SGI Operations

This section applies only to agencies conducting emergency response, public safety, disaster response, wildfire support, search and rescue support, or other urgent governmental operations.

UAS operations within an incident TFR shall not be conducted unless authorized by the FAA and coordinated with the official in charge of the on-scene emergency response activity. When an urgent UAS operation supports a critical public good and requires expedited FAA action, the agency may request a Special Government Interest waiver.

The agency shall document:

- The emergency or public interest basis for the request.

- The operating area and requested altitude.

- The aircraft, Remote Pilot in Command, and supporting crew.

- The requested deviation or authorization.

- The airspace coordination method.

- The incident command, dispatch, or emergency management contact.

- The start time, expected duration, and termination conditions.

- The deconfliction plan for crewed aircraft and other UAS.

Optional: Unauthorized UAS Sightings During Incident Operations

This section applies only when agency personnel are supporting incident operations where unauthorized UAS activity may create a hazard to crewed aircraft, emergency responders, or the public.

If unauthorized UAS activity is observed near an incident, personnel should report the activity through the appropriate incident, dispatch, aviation, or supervisory channel. The report should include, when safely observable:

- Time of sighting.

- Location.

- Direction of flight.

- Approximate altitude.

- Size, color, or description.

- Flight pattern.

- Remote ID information if available.

- Operator location if known.

Personnel shall not create a greater hazard by attempting to locate, confront, pursue, disable, interfere with, or seize a UAS or operator unless specifically authorized by law and agency policy. If operator contact is appropriate, personnel should make a simple safety request and refer enforcement issues to law enforcement when necessary. Safety of flight and responder safety take priority over locating the operator.

Optional: BVLOS SGI TFR Deconfliction

This section applies only to agencies participating in wildfire, emergency response, or other incident aviation operations where a non-agency UAS TFR may conflict with emergency aircraft operations.

If a non-agency UAS TFR or BVLOS SGI operation conflicts with an urgent incident response, the agency shall coordinate through the appropriate dispatch, emergency management, aviation, FAA, or incident command channel before entering the affected airspace. Agency aircraft shall not enter restricted airspace until deconfliction has occurred and the appropriate authorization has been received.

## 15.5 Public Safety Data

Public safety data may involve sensitive images of injuries, homes, critical infrastructure, emergency response, disaster impacts, or vulnerable persons. Access must be restricted to personnel with a mission need. Public release should be coordinated with the Public Information Officer, incident command, records officer, and counsel. If a mission may produce evidence or support a criminal investigation, Annex B should be consulted before flight if time allows, and counsel should be notified.

# 16. Optional Annex B: Law Enforcement Operations

Law enforcement only: This annex should be adopted only by a law enforcement agency or by an agency that provides UAS support to law enforcement under a counsel-reviewed interagency agreement. Oregon law imposes specific restrictions on law enforcement UAS operations. Non-law-enforcement agencies should not use this annex for routine agency missions.

## 16.1 Oregon Law Enforcement Baseline Rule

ORS 837.310 provides that, except as otherwise provided in ORS 837.310 to 837.345, a law enforcement agency may not operate a UAS, acquire information through UAS operation, or disclose information acquired through UAS operation. Information obtained in violation of those statutes, and evidence derived from it, is not admissible in judicial, administrative, arbitration, or other adjudicatory proceedings and may not be used to establish reasonable suspicion or probable cause.

## 16.2 Authorized Law Enforcement UAS Pathways

| Pathway | Operational Limitation |
| --- | --- |
| Warrant | A law enforcement agency may operate, acquire information, or disclose information if a warrant authorizes UAS use. The warrant must specify the authorized period and may not authorize operation for more than 30 days without renewal on motion and good cause. |
| Exigent circumstances | Permitted when the agency has probable cause to believe a person has committed, is committing, or is about to commit a crime and exigent circumstances make obtaining a warrant unreasonable. Document facts supporting probable cause and exigency. |
| Written consent | Permitted to acquire information about an individual or property when that individual has given written consent for those purposes. Maintain the consent record. |
| Search and rescue | Permitted for search and rescue activities as defined in Oregon law. Coordinate through the appropriate incident structure and document mission purpose. |
| Emergency assistance to an individual | Permitted when the agency reasonably believes there is an imminent threat to life or safety, documents the factual basis, and an official files the required sworn statement with circuit court not more than 48 hours after the emergency operation begins. |
| Governor-declared emergency | Permitted during a declared state of emergency when used only for preserving public safety, protecting property, or damage/erosion/contamination assessment and only in the geographic area specified in the proclamation. |
| Crime or accident scene reconstruction | Permitted for reconstruction of a specific crime scene or accident scene, or similar physical assessment, related to a specific investigation. The operation period under this section may not exceed five days. |
| Training | Permitted for training in UAS use and information acquisition. Images or other information acquired during training, and derivative evidence, may not be used in adjudicatory proceedings or to establish reasonable suspicion or probable cause. |

## 16.3 Law Enforcement Mission Approval

Identify the statutory pathway before flight and document it in the law enforcement UAS mission record.

Obtain warrant, written consent, sworn statement, or other legal documentation as required before flight unless exigent circumstances apply.

Limit flight area, sensor use, altitude, time, and data collection to the legal purpose.

Separate training data, incidental data, evidentiary data, and public safety data in records systems.

Coordinate with prosecutor, agency counsel, evidence custodian, and public records officer as appropriate.

Do not use UAS for general surveillance, crowd monitoring, investigative fishing expeditions, or intelligence gathering outside an authorized statutory pathway.

## 16.4 Support by Non-Law-Enforcement Agencies

When a non-law-enforcement agency provides UAS personnel, aircraft, or data support to a law enforcement agency, the support must be reviewed by counsel unless immediate emergency circumstances prevent advance review. The mission record should identify the requesting law enforcement agency, statutory basis, operational control, RPIC, data custodian, data transfer method, evidence handling requirements, and public records responsibilities. Non-law-enforcement pilots should not independently decide law enforcement legal questions in the field.

When a law enforcement agency conducts UAS operations during a broader emergency, disaster, search and rescue, wildfire, or public safety incident, the agency must comply with both the law enforcement restrictions in this annex and the applicable incident airspace, SGI, TFR, privacy, reporting, and operational control provisions of this manual. Nothing in the incident operations annex expands law enforcement authority beyond what is permitted by Oregon law, federal law, court order, warrant, consent, emergency authority, or other applicable legal authorization.

# 17. Appendices and Forms

The following forms may be used as written, converted into agency electronic systems, or replaced with equivalent agency forms. Forms should be kept simple enough for field use and complete enough to support annual reporting, safety assurance, records retention, and legal compliance.

## Appendix A: Customization Matrix

| Section | Decision Needed | Agency Selection / Notes |
| --- | --- | --- |
| Cover | Agency name, version, effective date, custodian. | [Insert] |
| 1.4 Authority | Add agency-specific authority and remove authorities that do not apply. | [Insert] |
| 3 Roles | Identify role holders and alternates. | [Insert] |
| 4 Mission categories | Select authorized mission categories. | [Insert] |
| 5.1 Operating authority | Part 107, COA, ECOA, contractor, or combination. | [Insert] |
| 5.6 Annual reporting | Assign ODAV annual report owner and due date workflow. | [Insert] |
| 7 Currency | Set platform currency and annual refresher requirements. | [Insert] |
| 12 Data | Set retention periods, storage systems, and website posting location. | [Insert] |
| 15 Annex A | Adopt, delete, or mark not adopted. | [Insert] |
| 16 Annex B | Adopt, delete, or mark not adopted. | [Insert] |

## Appendix B: Legal Requirements Quick Checklist

| Requirement | Completed By | Due / Frequency | Evidence of Completion |
| --- | --- | --- | --- |
| FAA registration | [UAS Program Manager] | Before flight and before expiration | FAA registration certificate and fleet tracker |
| FAA marking | [RPIC / Program Manager] | Before flight | Aircraft label / photo / inspection |
| Remote ID compliance | [RPIC] | Before each flight | Preflight checklist and fleet tracker |
| ODAV UAS registration | [UAS Program Manager] | Before operating in Oregon and annual renewal as required | ODAV registration record |
| ODAV annual report | [UAS Program Manager / Records Officer] | January 31 for preceding calendar year | Submitted report |
| ORS 837.362 data policy | [Program Manager / Records Officer] | Before operations and annually reviewed | Website posting or public availability record |
| Text of ORS 192.345 made available | [Records Officer] | Before operations and after legal updates | Website posting or public availability record |
| Public records retention schedule assigned | [Records Officer] | Before operations | Retention matrix |
| Part 107 pilot certificate and recency | [RPIC / Training Officer] | Before assignment and recurrently | Pilot master file |
| FAA airspace authorization | [RPIC] | Before controlled airspace operation | LAANC/manual authorization record |
| FAA/NTSB reporting | [RPIC / Program Manager] | As required by event | Report confirmation and incident file |

## Appendix C: Agency UAS Program Information Sheet

| Field | Agency Entry |
| --- | --- |
| Agency name | [Insert] |
| Program owner/division | [Insert] |
| UAS Program Manager | [Name / phone / email] |
| Flight Operations Coordinator / Chief Pilot | [Name / phone / email] |
| Records Officer | [Name / phone / email] |
| IT/Security Lead | [Name / phone / email] |
| Public Information Officer | [Name / phone / email] |
| Risk/Insurance Contact | [Name / phone / email] |
| Counsel | [Name / phone / email] |
| ODAV annual report owner | [Name / phone / email] |
| Public UAS policy website location | [URL] |

## Appendix D: UAS Fleet Inventory and Registration Tracker

| Aircraft ID | Make/Model | Serial No. | Weight Class | Color | FAA Reg./Exp. | ODAV Reg./Exp. | Remote ID SN | Assigned Unit | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [UAS-001] | [Make/Model] | [Serial] | [<55 / >=55] | [Color] | [Number / Date] | [Number / Date] | [Serial] | [Unit] | [Active / Maint / Retired] |

## Appendix E: Personnel Credential and Currency Tracker

| Name | Role | FAA Cert. | FAA Recency | Agency Training | Platform Checkout | 90-Day Currency | Annual Review | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Name] | [RPIC/VO/etc.] | [No./Date] | [Date] | [Date] | [Aircraft] | [Date] | [Date] | [Notes] |

## Appendix F: Mission Request and Approval Form

| Field | Entry |
| --- | --- |
| Mission title | [Insert] |
| Requesting program/project | [Insert] |
| Mission objective and public purpose | [Insert] |
| Requested date/time | [Insert] |
| Location / coordinates / map attached | [Insert] |
| Operating authority | [Part 107 / COA / ECOA / Contractor] |
| Aircraft / payload requested | [Insert] |
| RPIC / VO / crew | [Insert] |
| Expected data products | [Insert] |
| Data storage and retention | [Insert] |
| Known hazards | [Insert] |
| Permissions or coordination needed | [Insert] |
| PASP required? | [Yes / No] |
| Risk rating before mitigation | [Low / Medium / High / Critical] |
| Approval decision and conditions | [Approved / Denied / Conditions] |

## Appendix G: Project Aviation Safety Plan Template

| Element | Entry |
| --- | --- |
| Project / mission name | [Insert] |
| Mission objective | [Insert] |
| Date(s) and operational period | [Insert] |
| Location, coordinates, map | [Insert] |
| RPIC and crew | [Insert] |
| Aircraft, payload, Remote ID, registration | [Insert] |
| Airspace class and authorization | [Insert] |
| Airports/heliports/CTAF/ATC contacts | [Insert] |
| NOTAMs/TFRs/special use airspace | [Insert] |
| Weather minimums and forecast | [Insert] |
| Launch/recovery and emergency landing zones | [Insert] |
| VO locations and communications plan | [Insert] |
| Public/site control plan | [Insert] |
| Privacy/data collection boundaries | [Insert] |
| Hazard analysis and mitigation | [Insert] |
| Emergency response plan and contacts | [Insert] |
| Postflight data and records plan | [Insert] |

## Appendix H: Site Survey and Risk Assessment Worksheet

| Hazard | Present? | Risk Level | Mitigation / Notes |
| --- | --- | --- | --- |
| Controlled airspace / airport / heliport | [Y/N] | [L/M/H/C] | [Insert] |
| TFR / NOTAM / special use airspace | [Y/N] | [L/M/H/C] | [Insert] |
| Manned aircraft / emergency aircraft | [Y/N] | [L/M/H/C] | [Insert] |
| People, vehicles, public access | [Y/N] | [L/M/H/C] | [Insert] |
| Private property / sensitive areas | [Y/N] | [L/M/H/C] | [Insert] |
| Wires, towers, trees, cranes, structures | [Y/N] | [L/M/H/C] | [Insert] |
| Terrain, water, cliffs, roads, rail | [Y/N] | [L/M/H/C] | [Insert] |
| RF/magnetic/GNSS interference | [Y/N] | [L/M/H/C] | [Insert] |
| Weather, wind, smoke, visibility | [Y/N] | [L/M/H/C] | [Insert] |
| Wildlife/livestock/environmental sensitivity | [Y/N] | [L/M/H/C] | [Insert] |
| Data/privacy/public records sensitivity | [Y/N] | [L/M/H/C] | [Insert] |
| Crew fatigue/proficiency/workload | [Y/N] | [L/M/H/C] | [Insert] |

## Appendix I: Crew Briefing Checklist

Mission objective and public purpose.

RPIC, VO, payload operator, mission lead, trainee, and public contact roles.

Aircraft, payload, Remote ID, registration, battery plan, and expected flight profile.

Airspace, NOTAMs, TFRs, weather, local traffic, and emergency aircraft concerns.

Launch/recovery area, emergency landing zones, avoid areas, VO positions, and public control.

Communications plan, callouts, lost link, abort, emergency, and stop-flight procedures.

Data collection limits, privacy-sensitive areas, and data storage plan.

Crew questions, concerns, and final go/no-go decision.

## Appendix J: Preflight, In-Flight, and Postflight Checklists

| Phase | Checklist |
| --- | --- |
| Preflight | Mission approved; RPIC current; crew briefed; aircraft registered; Remote ID verified; airspace/TFR/NOTAM checked; weather checked; aircraft inspected; batteries checked; RTH/lost link set; launch area clear; data plan confirmed. |
| In-flight | Maintain VLOS; monitor battery, telemetry, aircraft, airspace, people, vehicles, weather, mission boundaries, VO calls, and payload collection; yield to aircraft; pause or abort if safety changes. |
| Postflight | Land safely; power down; inspect aircraft and batteries; secure data; complete flight log; document maintenance issues; debrief crew; report incidents or concerns; store equipment. |

## Appendix K: Emergency Response Checklist and Contact Roster

| Action | Completed / Notes |
| --- | --- |
| Start log of times, calls, and actions. | [ ] |
| Protect people. Call 911 if needed. | [ ] |
| Secure area and prevent additional injury or damage. | [ ] |
| Preserve aircraft, batteries, controller, memory cards, and scene evidence if safe. | [ ] |
| Notify supervisor and UAS Program Manager. | [ ] |
| Determine FAA Part 107 reportability. | [ ] |
| Determine NTSB Part 830 reportability. | [ ] |
| Notify property owner or facility owner if appropriate. | [ ] |
| Notify IT/security if data, device, or account issue exists. | [ ] |
| Complete agency incident report. | [ ] |
| Remove aircraft from service pending inspection if required. | [ ] |

| Contact | Name | Phone | Email |
| --- | --- | --- | --- |
| Emergency | 911 | 911 | N/A |
| UAS Program Manager | [Name] | [Phone] | [Email] |
| Agency Supervisor | [Name] | [Phone] | [Email] |
| Records Officer | [Name] | [Phone] | [Email] |
| IT/Security | [Name] | [Phone] | [Email] |
| Counsel | [Name] | [Phone] | [Email] |
| NTSB Response Operations Center | [Insert current number] | [Insert] | [Insert] |
| FAA UAS Support / Reporting | [Insert] | [Insert] | [Insert] |

## Appendix L: UAS Incident / Accident Report

| Field | Entry |
| --- | --- |
| Date/time/location | [Insert] |
| Mission name and operating authority | [Insert] |
| RPIC and crew | [Insert] |
| Aircraft ID and payload | [Insert] |
| Description of event | [Insert] |
| Injuries or medical response | [Insert] |
| Property damage | [Insert] |
| Aircraft damage | [Insert] |
| Lost link/flyaway/fire/battery event? | [Insert] |
| Weather/airspace conditions | [Insert] |
| Immediate actions taken | [Insert] |
| Notifications made | [Insert] |
| FAA report required/submitted? | [Insert] |
| NTSB report required/submitted? | [Insert] |
| Corrective actions | [Insert] |
| Return-to-service decision | [Insert] |

## Appendix M: UAS Data Intake, Retention, and Disposition Log

| Mission | Data Type | Storage Location | Access Limits | Retention Schedule | Disposition Date | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| [Mission] | [Photos/video/thermal/etc.] | [System/path] | [Roles] | [Schedule] | [Date] | [Notes] |

## Appendix N: Maintenance Log

| Date | Aircraft ID | Maintenance / Inspection | Parts / Firmware | Performed By | Test Flight | Return to Service |
| --- | --- | --- | --- | --- | --- | --- |
| [Date] | [ID] | [Description] | [Parts/version] | [Name] | [Y/N/Notes] | [Approved/Not approved] |

## Appendix O: ODAV Annual Report Worksheet

Use this worksheet to prepare the annual report due to ODAV by January 31 for the preceding calendar year. Agencies may use an ODAV-provided form or an equivalent internal report if accepted by ODAV.

| Reporting Element | Agency Entry |
| --- | --- |
| Reporting calendar year | [Year] |
| Public body name | [Agency] |
| Registered UAS used during year | [List or attach fleet report] |
| Frequency of use | [Number of flights / hours / missions by aircraft or program] |
| Purposes of use | [Summary by category: inspection, mapping, emergency, training, etc.] |
| Public access to ORS 837.362 policies | [Website URL or method of public availability] |
| Contact for report | [Name / title / phone / email] |
| Submitted to ODAV on | [Date] |

## Appendix P: Public Website Posting Template

[AGENCY NAME] uses unmanned aircraft systems for authorized public purposes, including [insert mission categories]. The agency operates UAS in compliance with applicable FAA requirements, Oregon law, and agency policies. The agency has adopted policies and procedures for the use, storage, access, sharing, and retention of UAS data. The public may access the agency UAS policy at [URL] and may access the text of ORS 192.345 at [URL or posted text]. Public records requests may be submitted through [agency public records process].

## Appendix Q: Contractor UAS Services Checklist

Contract or purchase order approved.

Scope of work and mission objective defined.

Operational control assigned.

Part 107 certificate verified.

Aircraft FAA registration and Remote ID verified.

Insurance verified.

Safety plan/PASP reviewed.

Data ownership and public records terms included.

Storage and cybersecurity terms included.

Incident reporting requirements included.

Deliverables and retention requirements included.

Public communications plan established if needed.

# 18. Sources Consulted and Legal References

This template synthesizes the uploaded agency and public safety UAS documents together with current public legal sources. The source list is included to help reviewers understand the policy lineage and to support agency-specific legal review.

## 18.1 Source Documents

Oregon Department of Forestry Air Operations Policy.

Oregon Department of Transportation UAS Operations Manual, September 2024.

DHS NUSTL Small Unmanned Aircraft System Program Documentation for Public Safety: Recommendations and Templates, November 2025.

Department of the Interior.

## 18.2 Key Legal References Reviewed

14 CFR Part 107, Small Unmanned Aircraft Systems: https://www.ecfr.gov/current/title-14/chapter-I/subchapter-F/part-107

14 CFR Part 89, Remote Identification of Unmanned Aircraft: https://www.ecfr.gov/current/title-14/chapter-I/subchapter-F/part-89

FAA Remote ID: https://www.faa.gov/uas/getting_started/remote_id

FAA LAANC: https://www.faa.gov/uas/getting_started/laanc

FAA DroneZone: https://faadronezone-access.faa.gov/

49 CFR Part 830 and NTSB UAS advisory: https://www.ntsb.gov/investigations/process/Documents/NTSB-Advisory-Drones.pdf

ORS 837.300 to 837.390 and ORS 837.995: https://www.oregonlegislature.gov/bills_laws/ors/ors837.html

OAR 738-080-0045 and OAR 738-080-0050: https://secure.sos.state.or.us/oard/displayDivisionRules.action?selectedDivision=3467

ODAV aircraft and public use UAS registration and renewal: https://www.oregon.gov/aviation/doing-business-with-odav/pages/registration-renewal.aspx

ORS 192.345 and Oregon Public Records Law: https://www.oregonlegislature.gov/bills_laws/ors/ors192.html

# Integrated Supplemental Material from Unified UAS / DFR Policy, Operations, Training, and Test Bank Manual

The following supplemental material is incorporated into this template without replacing the base Oregon public agency policy framework. It is included to preserve and add all DFR, public safety, law enforcement, mutual aid, BVLOS/COA, remote operations, training, proficiency, practical evaluation, and test bank elements from the Unified UAS / DFR manual. Answer keys and evaluator material should be removed before preparing a student or public-facing version.

[AGENCY NAME]

Unified UAS / DFR Policy, Operations, Training, and Test Bank Manual

Public Agency Template for Routine UAS, Drone as First Responder, Fire/EMS, SAR, Emergency Management, Law Enforcement, Mutual Aid, BVLOS/COA, Remote Operations, Training, Proficiency, Practical Evaluation, and Test Bank Administration

Administrative Version - Includes Answer Keys and Evaluator Material

Remove answer keys, evaluator notes, and sensitive scenario injects before distributing a student copy.

Version: [DRAFT / ADOPTED VERSION]
Effective Date: [DATE]
Review Cycle: [ANNUAL / BIENNIAL]

# Document Control

| Field | Agency Entry |
| --- | --- |
| Adopting agency | [AGENCY NAME] |
| Governing body or approval authority | [CITY COUNCIL / COUNTY COMMISSION / FIRE DISTRICT BOARD / SHERIFF / CHIEF / DIRECTOR / OTHER] |
| Program owner | [OFFICE / DIVISION / PROGRAM] |
| UAS Program Manager | [NAME / TITLE] |
| Training Officer | [NAME / TITLE] |
| Chief Pilot / Flight Operations Coordinator | [NAME / TITLE] |
| Records / Data Steward | [NAME / TITLE] |
| IT / Security Lead | [NAME / TITLE] |
| Effective date | [DATE] |
| Version | [DRAFT / ADOPTED VERSION] |
| Review cycle | [ANNUAL / BIENNIAL / OTHER] |
| Legal review | [COUNSEL NAME / DATE] |
| Public posting location | [URL / PUBLIC RECORDS LOCATION] |

# How to Use This Unified Manual

This manual is written as a single controlled document that combines policy, operating procedures, training standards, curriculum content, test-bank material, practical evaluation standards, and field forms. It is intended for customization by a state, local, tribal, territorial, fire, EMS, law-enforcement, emergency management, transportation, public works, or other public agency UAS program.

Bracketed text must be completed before adoption. Optional annexes should be deleted, marked not adopted, or modified when the agency does not conduct the mission type. DFR, BVLOS, shielded operations, remote operations, operations over people, operations over moving vehicles, controlled-airspace operations, law-enforcement support, and contractor-supported operations require counsel, risk, IT/security, records, procurement, and FAA-authorization review before use.

This document is a template and training-administration tool. It is not legal advice, does not itself grant FAA authority, and does not create law-enforcement authority. Each adopting agency must verify federal, state, local, tribal, grant, procurement, public-records, privacy, labor, evidence, medical privacy, and FAA authorization requirements before approval.

## Student Copy Preparation Note

The test-bank and practical-evaluation sections contain answer keys and evaluator material. Before distributing a student copy, remove answer keys, scoring notes that reveal injects, sensitive DFR/BVLOS route information, tactical law-enforcement scenario details, and any local security information.

# Contents at a Glance

| Part | Title | Purpose |
| --- | --- | --- |
| Front Matter | Document Control, Use Notes | Explains agency use notes. |
| Part I | Program Policy and Operations Manual | Unified UAS and DFR policy, legal and FAA authority, governance, standard operations, data, maintenance, mutual aid, fire/EMS annex, law-enforcement annex, and operational checklists. |
| Part II | Training and Proficiency Program | Governance, role-based qualification tracks, competency domains, training delivery, testing standards, recurrent training, instructor/evaluator controls, records, DFR/BVLOS training, and RPCS carding integration. |
| Part III | Detailed Curriculum, Test Banks, and Practical Standards | Lesson content for core UAS modules, written examination program, core question bank and answer key, practical tests, DFR and BVLOS question banks, RPCS task-based standards, and DFR/BVLOS evaluation plans. |
| Part IV | Supplemental Public-Safety DFR Test Bank and Scenario Practicals | Additional 98-question public-safety DFR knowledge bank, answer key, practical rubrics, and scenario exercises for fire/EMS, SAR, law enforcement, mutual aid, and DFR operations. |
| Part V | Supplemental Training Forms and Records | Additional checklists and records fields for initial qualification, VO, payload, platform checkout, NIST events, DFR, Fire/EMS/SAR, law enforcement, annual recurrent training, AAR, and tracker fields. |
| Part VI | Sources Consulted and Local Review Checklist | Combined source list, legal review reminders, and adoption checklist. |

Part I. Program Policy and Operations Manual

Consolidated operating policy, standard procedures, DFR/public-safety annexes, and operational checklists.

# 1. Policy Statement, Scope, and Authority

## 1.1 Policy Statement

[AGENCY NAME] uses Unmanned Aircraft Systems only for authorized public purposes and only under appropriate operational control. DFR and other public-safety UAS operations are aviation activities that must be planned, briefed, documented, and reviewed in proportion to mission risk. The agency will operate UAS in a manner that protects life safety, responder safety, the National Airspace System, privacy, civil liberties, public records obligations, cybersecurity, evidence integrity, and public trust.

The Remote Pilot in Command has final authority over the safe conduct of each flight. The RPIC may cancel, postpone, modify, or terminate a flight when safety, regulatory compliance, aircraft condition, weather, airspace, crew readiness, public interaction, data handling, or mission conditions require that decision. Command staff, dispatchers, incident commanders, RTIC/RTCC staff, and supervisors may request or assign a mission, but they may not compel unsafe flight.

## 1.2 Purpose

This manual establishes a common framework for DFR and public-safety UAS governance, mission approval, dispatch integration, flight operations, safety management, DFR COA and waiver compliance, pilot and crew training, data handling, maintenance, records, mutual aid, and continuous improvement. It is intended to support fire/rescue, EMS, emergency management, search and rescue, disaster response, law-enforcement, and joint public-safety operations while allowing local customization.

## 1.3 Scope

This manual applies to all employees, volunteers, interns, agents, contractors, mutual-aid personnel, and partner agencies who plan, approve, request, launch, pilot, observe, operate payloads, maintain, procure, store data from, or otherwise support UAS operations conducted by or for [AGENCY NAME]. It applies to aircraft owned, leased, borrowed, loaned to, docked for, contracted for, or operated on behalf of the agency, and it applies to joint operations when the agency has operational control, shares operational control, provides personnel, requests a mission, receives UAS data, or uses live video for public-safety decisions.

## 1.4 Authority

Each UAS mission must have an aviation authority and an agency mission authority. The aviation authority may be Part 107, a Part 107 waiver, controlled-airspace authorization, a public-aircraft COA, a Part 91 public-safety waiver, an emergency COA, an SGI authorization, a Section 927 waiver when applicable, or another FAA authorization. The agency mission authority comes from state law, local charter or ordinance, emergency-management authority, fire/rescue authority, law-enforcement authority, mutual-aid agreement, intergovernmental agreement, contract, incident command assignment, or other agency-specific authorization.

| Authority type | Local customization / operational effect |
| --- | --- |
| 14 CFR Part 107 | Default civil small-UAS operating pathway for many public-safety flights when all Part 107 requirements and any applicable waivers or authorizations are satisfied. |
| Public Aircraft COA / Part 91 public-aircraft operation | Available only when the agency and flight qualify under public-aircraft criteria and the COA or waiver expressly covers the operation. |
| Part 107 waiver / Part 91 waiver / Section 927 waiver | Used only for operations outside standard rule limits when FAA approval, safety case, responsible person, conditions, and limitations are documented. |
| Emergency COA / SGI authorization | Used for qualifying urgent public-safety missions when standard processing cannot meet the operational need and the FAA process authorizes the deviation. |
| State and local authority | [INSERT STATE UAS LAW, PRIVACY LAW, PUBLIC RECORDS LAW, LAW-ENFORCEMENT AUTHORITY, FIRE/EMS AUTHORITY, EMERGENCY MANAGEMENT AUTHORITY, LOCAL POLICY, AND RECORDS RETENTION CITATIONS.] |
| Mutual-aid or intergovernmental authority | [INSERT AUTOMATIC-AID, MUTUAL-AID, INTERLOCAL, REGIONAL TASK FORCE, EOC, RTIC/RTCC, OR INCIDENT COMMAND AGREEMENTS.] |

## 1.5 Relationship to Other Policies

This manual supplements agency policies on records management, information security, privacy, procurement, vehicle use, risk management, emergency management, media relations, public records, workplace safety, incident command, evidence, use of force, community engagement, and contract administration. When a law, regulation, FAA authorization, court order, contract, or higher-level agency policy conflicts with this manual, the higher authority controls. The UAS Program Manager must coordinate with counsel when a mission raises a legal conflict, uncertain operational control, data-sharing question, or law-enforcement authority issue.

## 1.6 Program Exceptions

Exceptions require approval by [APPROVAL AUTHORITY] after consultation with the UAS Program Manager. Exceptions involving BVLOS, operations over people, operations over moving vehicles, controlled airspace, emergency response aircraft, law-enforcement support, public records, privacy, data retention, cybersecurity, procurement, public-aircraft operation, or COA/waiver compliance require counsel or subject-matter review before flight unless an emergency requires immediate action. Any emergency deviation must be documented within [72 HOURS OR LOCAL REQUIREMENT].

# 2. Definitions and Acronyms

Customization note: Agencies should preserve definitions drawn from federal and state law. Local definitions may be added, but local wording should not change the legal meaning of statutory or FAA terms.

| Term | Definition |
| --- | --- |
| Agency UAS | A UAS owned, leased, borrowed, operated, contracted for, docked for, or otherwise used for an authorized [AGENCY NAME] mission. |
| Airspace authorization | FAA authorization, including LAANC or manual authorization, permitting UAS operations in controlled airspace or other airspace where authorization is required. |
| Airspace Coordinator / Airspace Monitor | A trained person assigned to monitor crewed and uncrewed aircraft activity, maintain deconfliction information, and advise the RPIC and incident command on airspace conflicts. |
| Certificate of Waiver or Authorization (COA) | An FAA authorization issued to a public operator or eligible entity for specific UAS operations subject to stated provisions and limitations. |
| DFR | Drone as First Responder, a dispatch-integrated model in which a prepositioned UAS responds to selected calls for service to provide timely situational awareness. |
| Operational control | The authority over initiating, conducting, tasking, changing, or terminating a flight. Operational control must be clear before launch. |
| Remote Pilot in Command (RPIC) | The person designated before or during flight who is directly responsible for and has final authority over the UAS operation. |
| Remote ID | FAA-required broadcast identification and location information for UAS required to comply with Part 89 unless a lawful exception or authorization applies. |
| RTIC/RTCC | Real-Time Information Center or Real-Time Crime Center, a public-safety coordination location that may support DFR launch decisions, video routing, dispatch coordination, and incident intelligence under approved policy. |
| Visual Observer (VO) | A trained crewmember who assists the RPIC with see-and-avoid, obstacle awareness, ground safety, communications, and mission awareness. |

# 3. Program Governance, Roles, and Responsibilities

The UAS program must have a clear chain of responsibility. A small agency may assign multiple responsibilities to one person, but the manual must still identify accountability for policy, flight safety, training, records, data, cybersecurity, maintenance, procurement, and annual program review. When several agencies participate in a mission, the participating agencies must identify which entity has operational control before flight or as soon as practical during an emergency.

| Role | Narrative standard |
| --- | --- |
| Agency Head or Designee | Approves the UAS program, adopts the manual, assigns program authority, resolves policy issues, and ensures resources exist for safe operations. |
| UAS Program Manager | Owns the program, maintains this manual, approves pilots and aircraft for agency use, manages audits, coordinates FAA and state/local compliance, and serves as the policy point of contact. |
| DFR Operations Coordinator | Manages day-to-day DFR workflow, launch matrix, dock readiness, CAD/dispatch integration, RTIC/RTCC coordination, video routing, shift coverage, and operational readiness. |
| Chief Pilot / Flight Operations Coordinator | Oversees pilot standardization, proficiency, mission planning quality, platform checkouts, flight logs, operational risk management, and remedial training. |
| Aviation Safety Officer / Airspace Coordinator | Tracks hazards, near misses, airspace conflicts, crewed-aircraft encounters, TFR/NOTAM issues, deconfliction procedures, and corrective actions. |
| Remote Pilot in Command | Has final authority for safe flight; verifies aircraft condition, crew readiness, airspace, weather, Remote ID, registration, mission authorization, site suitability, emergency procedures, and postflight reporting. |
| Visual Observer / Airspace Monitor | Maintains airspace and ground scan, supports obstacle and traffic avoidance, communicates changes to the RPIC, and remains focused on safety during flight. |
| Payload Operator / Sensor Operator | Operates cameras, thermal sensors, mapping payloads, and other sensors only under the mission plan, RPIC direction, privacy rules, and data-handling requirements. |
| Mission Lead / Incident Commander / Project Lead | Defines the public purpose, coordinates site access and incident objectives, determines how data will be used, and integrates the UAS into incident command or agency operations. |
| Records Officer / Data Steward | Ensures flight logs, imagery, video, maps, incident records, maintenance logs, training records, authorizations, and annual reports are retained and disposed according to law and policy. |
| IT/Security Lead | Reviews systems used to store, transmit, process, or share UAS data; approves cloud, dock, RTIC/RTCC, and vendor systems; and supports incident response for data exposure. |
| Procurement / Contract Administrator | Ensures aircraft and UAS services are procured through approved channels and that contracts address safety, insurance, data, cybersecurity, public records, and operational control. |
| Public Information Officer | Coordinates public-facing information, website posting, media inquiries, complaints, community outreach, and mission notifications where appropriate. |

## 3.1 Operational Control

Operational control must be documented for each routine mission and clearly assigned during emergencies. The lead entity is ordinarily the entity that requested or ordered the aircraft, funds or provides the aircraft or operator, directs the mission objectives, controls the flight boundaries, and receives or uses the mission data. When those factors point to more than one entity, the agencies must identify the lead entity and the flight-safety chain of command before launch. Agency personnel must not direct an aircraft outside the agency's operational control unless a mutual-aid agreement, incident command assignment, contract, intergovernmental agreement, or other lawful authority permits that role.

# 4. Authorized Missions and Prohibited Uses

UAS may be used only for authorized public purposes consistent with agency mission, law, policy, FAA authorization, and the approved mission plan. The adopting agency should select mission categories, delete categories that do not apply, and mark public-safety and law-enforcement annexes as adopted, not adopted, or adopted with modifications.

| Mission category | Examples | Adopt? |
| --- | --- | --- |
| DFR - fire/rescue and EMS | Initial scene size-up, smoke/fire conditions, route assessment, water rescue support, hazardous materials standoff assessment, patient access support, air-medical LZ awareness, and rescue overwatch. | [Yes / No / Modify] |
| DFR - law enforcement | Selected calls involving risk to responders or the public, perimeter information, missing persons, vehicle crashes, barricades, in-progress crimes, suspect location when lawful, and scene safety support. | [Yes / No / Modify] |
| Emergency management and disaster response | Flood, storm, earthquake, wildfire, landslide, evacuation route, shelter, infrastructure, and access assessment. | [Yes / No / Modify] |
| Search and rescue | Lost persons, wilderness search, water search, thermal screening, search-area documentation, and support to incident command. | [Yes / No / Modify] |
| Fire investigation / accident reconstruction / incident documentation | Post-incident documentation, structure or wildland fire perimeter, crash scene documentation, and damage assessment when legally authorized. | [Yes / No / Modify] |
| Training and proficiency | Pilot currency, platform checkout, NIST-style drills, tabletop exercises, airspace deconfliction drills, emergency procedures, and data-handling exercises. | [Yes / No / Modify] |
| Public information and outreach | Public project documentation, education, and media support when approved by command and privacy/public-records rules. | [Yes / No / Modify] |
| Infrastructure, facility, and utility support | Bridges, roads, rooftops, towers, utilities, stormwater, communications, public works, and other facility inspections. | [Yes / No / Modify] |

## 4.1 Prohibited Uses

The agency shall not use UAS for personal, recreational, political, campaign, or non-agency purposes. UAS shall not be used for general surveillance, investigative fishing expeditions, intimidation, harassment, or observation of constitutionally protected activity unrelated to an approved public-safety mission. The agency shall avoid collecting data beyond the approved mission objective when feasible, and shall not use UAS in a way that interferes with crewed aircraft, emergency responders, fire aviation, medevac operations, airports, heliports, wildlife, or lawful activity.

Operations over people, operations over moving vehicles, BVLOS operations, operations from moving vehicles, controlled-airspace operations, multi-aircraft operations by one pilot, weaponized or projectile operations, item drops, and other advanced operations may occur only when permitted by law, FAA rule, FAA waiver or COA, local policy, and written approval. Privately owned UAS may not be used for agency business unless the UAS Program Manager, risk management, and counsel approve registration, insurance, maintenance, data, and operational-control requirements.

# 5. Federal and State/Local Legal Compliance

Legal compliance is a flight-planning requirement, not a postflight documentation exercise. Before launch, the RPIC or authorized designee must confirm the operating pathway, aircraft registration, Remote ID status, pilot qualifications, airspace, TFRs, NOTAMs, mission authority, privacy rules, data retention category, and any law-enforcement legal process or fire/EMS confidentiality restrictions.

## 5.1 FAA Operating Pathways

Government, law-enforcement, and public-safety entities operating small drones under 55 pounds generally use either Part 107 or the statutory public-aircraft pathway with a COA. The agency may hold both a Part 107 capability and a public-aircraft COA capability, but a particular flight must be assigned to a single pathway before launch. A mission may not assume that a public purpose automatically exempts it from FAA requirements.

| Pathway | Use when | Key program requirements |
| --- | --- | --- |
| Part 107 | The operation can comply with Part 107 or with an approved Part 107 waiver and airspace authorization. | FAA remote pilot certificate, aircraft registration, Remote ID when required, preflight, VLOS unless waived, right-of-way, Part 107 limits, FAA reporting, and airspace authorization when required. |
| Public Aircraft COA / Part 91 | The agency and flight qualify as public aircraft and the COA or waiver covers the location, aircraft, crew, mission, and limitations. | Public-aircraft eligibility, COA compliance, agency-defined pilot qualification and airworthiness, operational control, reporting, and compliance with COA provisions. |
| Emergency COA / SGI | An urgent qualifying emergency or public-interest mission cannot wait for standard processing and requires temporary authorization. | Authorized requester, urgent mission basis, defined operating area and altitude, crew and aircraft identification, deconfliction plan, incident contact, and post-operation documentation. |
| Contractor Part 107 | A vendor operates the aircraft as a civil Part 107 operator under contract to the agency. | Contract must address operational control, certificate, registration, Remote ID, insurance, maintenance, data ownership, public records, cybersecurity, privacy, and incident reporting. |

## 5.2 Part 107 Baseline Requirements

When operating under Part 107, the RPIC must satisfy remote pilot certification and recency requirements, conduct a preflight inspection, determine that the aircraft is in condition for safe operation, maintain required visual line of sight unless waived, yield right-of-way to crewed aircraft, comply with applicable limits on altitude, speed, cloud clearance, operations over people, moving vehicles, night operations, and airspace, and report qualifying safety events to the FAA. A waiver or airspace authorization must be obtained before operating outside applicable limits.

## 5.3 Public Aircraft COA and Part 91 Public-Safety Operations

When operating as a public aircraft, the agency must document that the organization and the flight meet public-aircraft criteria, that the aircraft is owned or exclusively leased as required, that the mission is a governmental function, and that the operation is noncommercial. The COA or waiver provisions must be available to the RPIC, DFR Operations Coordinator, airspace coordinator, and UAS Program Manager. The agency remains responsible for pilot qualification, airworthiness, training, maintenance, reporting, and operational control.

## 5.4 DFR COA, BVLOS, and Shielded Operations

Routine DFR often creates pressure to operate beyond visual line of sight, launch from remote docks, operate over streets, fly near buildings, or provide video before responders arrive. Those operational benefits do not create authority by themselves. BVLOS is not permitted for routine public-safety operations unless the agency has a Part 107 waiver, a specific COA provision, a Part 91 public-safety waiver, or another FAA authorization that covers the operation. The agency shall maintain a DFR authorization binder that contains the approved concept of operations, maps, operational areas, altitude limits, lost-link procedures, crew roles, technology assumptions, and waiver conditions.

For qualifying public-safety organizations, the FAA has published Part 91 public-aircraft and Part 107 public-safety shielded-operations BVLOS checklists for 200-foot AGL operations that use obstruction shielding as a primary anti-collision mitigation. This manual does not grant that authority. The agency must obtain and comply with the applicable FAA authorization, confirm whether the operation is under Part 107 or Part 91, and ensure that all required lighting, Remote ID, airspace awareness, ADS-B or approved air-traffic awareness resources, responsible-person accountability, and lost-link/RTH provisions are implemented.

## 5.5 Emergency COA and Special Governmental Interest

The SGI process is reserved for urgent public-interest and emergency operations such as firefighting, search and rescue, law enforcement, utility restoration, and disaster damage assessment. It is not a convenience tool for routine planning. When SGI is used, the agency must document the emergency basis, operating area, altitude, requested deviation, aircraft, RPIC, supporting crew, airspace coordination method, incident command or dispatch contact, start time, expected duration, termination conditions, and deconfliction plan.

## 5.6 Remote ID, Registration, Airspace, NOTAMs, and TFRs

Each UAS must be registered and marked as required. The RPIC must verify Remote ID compliance before flight unless a lawful exception or authorization applies. Before each flight, the RPIC or airspace monitor must review airspace class, airports, heliports, LAANC or manual authorization status, NOTAMs, TFRs, stadium restrictions, wildfire restrictions, special-use airspace, parachute activity, military training routes when relevant, and known public-safety aviation. UAS operations inside or near incident TFRs must be authorized and coordinated with the official responsible for the on-scene emergency response activity.

## 5.7 State and Local Legal Customization

The adopting agency shall insert state and local law before adoption. At minimum, the legal review should address state UAS statutes, state constitutional search-and-seizure rules, law-enforcement warrant or exception requirements, fire/EMS confidentiality, medical information, public records, records retention, private property, critical infrastructure, local preemption, park rules, public-notice requirements, tribal or federal lands, emergency powers, procurement rules, insurance, and intergovernmental sharing. Oregon agencies should add Oregon-specific registration, reporting, public body UAS data policy, and law-enforcement UAS requirements from the Oregon template or current counsel-reviewed sources.

# 6. Acquisition, Inventory, Registration, and Contractor Services

No aircraft, dock, payload, Remote ID module, controller, battery system, command-and-control link, software platform, cloud service, RTIC/RTCC integration, or mission-critical accessory may be purchased, leased, donated, borrowed, or placed into service without approval by the UAS Program Manager and compliance with procurement, IT/security, records, risk-management, and legal requirements. The acquisition review must consider mission need, lifecycle cost, pilot training, maintenance support, Remote ID, cybersecurity, data ownership, public records, foreign-manufactured technology restrictions when applicable, and compatibility with existing fleet and authorization documents.

The agency must maintain a fleet and configuration inventory for each aircraft and dock. The inventory should track make, model, serial number, aircraft type, weight, color, FAA registration, state registration when required, Remote ID serial number, battery sets, payloads, assigned pilots, dock location, software/firmware versions, maintenance status, and disposal status. The RPIC must have access to required documents during operations, including registration, aircraft manuals, checklists, COAs, waivers, authorizations, and emergency contacts.

## 6.1 Contractor and Vendor-Operated DFR

Contracted UAS services require careful operational-control review. A vendor may supply hardware, software, docks, live-stream tools, maintenance, training, cloud services, or flight operations, but the agency remains responsible for ensuring that the mission complies with law, policy, records obligations, and public trust. The contract, purchase order, task order, or intergovernmental agreement must specify who has operational control, who serves as RPIC, which FAA pathway applies, who obtains airspace authorization, who maintains the aircraft, who owns and stores data, who responds to public records requests, who reports incidents, who pays costs, and who can stop the operation for safety.

# 7. Personnel Qualifications, Training, and Currency

Training must be role-based, mission-based, and documented. A person may not act as RPIC for agency UAS missions until the person has satisfied FAA certification requirements, completed agency policy and legal training, demonstrated platform-specific proficiency, reviewed emergency procedures, and been appointed by the agency. A person may not act as a visual observer, airspace monitor, teleoperator, payload operator, or DFR supervisor until the person has completed the training required for that role.

## 7.1 Pilot and Crew Standards

The RPIC must hold a current FAA remote pilot certificate when operating under Part 107 and must satisfy any FAA, COA, waiver, or agency qualification standard applicable to public-aircraft operations. A trainee may manipulate the controls only under the direct supervision of an approved RPIC in an approved training environment or as otherwise permitted by the applicable FAA authorization. All crewmembers must self-ground when fatigue, medication, illness, stress, distraction, workload, or another condition could interfere with safe operation.

DFR creates additional role-specific requirements. A DFR teleoperator must understand dock operations, remote launch and recovery, C2 monitoring, video latency, geofencing, RTH settings, emergency landing zones, live-stream access, and RTIC/RTCC workflow. A remote airspace monitor must understand airspace displays, local airports and heliports, ADS-B limitations, NOTAM/TFR checks, crewed-aircraft priority, deconfliction phraseology, and emergency landing authority. Fire/EMS and law-enforcement endorsements should be issued only after mission-specific training.

| Role or endorsement | Initial standard | Recurrent standard |
| --- | --- | --- |
| RPIC | FAA certificate or COA-defined qualification, agency manual review, legal/privacy/data training, platform checkout, emergency procedures, and supervised operational experience. | FAA recurrent compliance, annual policy/legal update, annual platform proficiency, emergency drill, and review after any incident, long break, or major software/aircraft change. |
| Visual Observer / Airspace Monitor | Airspace awareness, scanning technique, aircraft recognition, crew communication, lost-link procedure, public-safety aviation hazards, and local airport/heliport orientation. | Annual airspace refresher, deconfliction drill, scenario-based crew resource management practice, and requalification after material procedure changes. |
| DFR Teleoperator | Dock console, launch matrix, C2 link, RTH/lost-link settings, RTIC/RTCC workflow, sterile cockpit, emergency procedures, and role-based privacy/data controls. | Quarterly console currency, annual DFR emergency drill, annual privacy/data update, and proficiency after dock, software, or authorization changes. |
| Fire/EMS DFR Endorsement | ICS basics, fireground hazards, smoke/wind behavior, air-medical LZ awareness, wildfire/WUI hazards, hazmat standoff concepts, patient imagery controls, and coordination with fire command. | Annual fire/EMS scenario, seasonal wildfire/TFR refresher, air-medical deconfliction drill, and after-action integration with fire leadership. |
| Law-Enforcement DFR Endorsement | Legal process, privacy, civil liberties, evidence handling, RTIC/RTCC workflow, tactical deconfliction, pursuit/containment policy, community-care/emergency-aid concepts, and data minimization. | Annual legal/privacy update, evidence/data drill, tactical deconfliction scenario, and review after any complaint, court ruling, or policy update. |
| BVLOS / DFR COA Endorsement | FAA authorization review, ConOps, route/altitude limits, shielded-operation assumptions, airspace awareness, detect-and-avoid mitigations, lost-link/RTH, emergency landing, and remote crew coordination. | Semiannual BVLOS drill, annual authorization review, quarterly lost-link tabletop or live drill, and requalification after FAA authorization, software, dock, or route changes. |

## 7.2 Training Model and Proficiency Testing

The agency should use DRONERESPONDERS and Law Enforcement Drone Association (LEDA) materials as a public-safety benchmarking resource, especially for program management, safety management, waiver navigation, airspace deconfliction, thermal imaging, search and rescue, and tactical mission training. The agency should also use objective proficiency tasks, such as NIST aerial drone test methods, to measure manual flight, precision, payload orientation, thermal or visual inspection skill, and mission-specific operator proficiency. NIST methods are not a substitute for FAA certification or agency appointment, but they provide repeatable measures for skill development and documentation.

The DRONERESPONDERS DFR Working Group should be treated as an external benchmarking and learning resource. Participation does not create authority to fly, but it can help the agency compare launch matrices, FAA authorization models, deconfliction practices, community-engagement approaches, and technology workflows with other current and prospective DFR programs.

# 8. Safety Management System

The agency UAS program will use a scalable Safety Management System built around safety policy, safety risk management, safety assurance, and safety promotion. The purpose is to identify hazards, reduce risk, learn from operations, and create a safety culture in which personnel can raise concerns without fear of retaliation for good-faith reporting. SMS applies to routine training flights, dock-launched DFR flights, fire/EMS incidents, tactical law-enforcement incidents, mutual aid, and contractor operations.

| SMS component | Agency practice |
| --- | --- |
| Safety policy | Adopt this manual, define authority, assign roles, set training expectations, establish stop-flight authority, and make safety a program responsibility. |
| Safety risk management | Use mission screening, site surveys, airspace checks, weather review, crew briefing, risk assessment, Check 3 GPS, and mitigation before flight. |
| Safety assurance | Review flight logs, maintenance logs, battery records, incidents, near misses, crewed-aircraft conflicts, complaints, data incidents, audits, and corrective actions. |
| Safety promotion | Share lessons learned, provide recurrent training, brief seasonal hazards, encourage reporting, review legal and FAA changes, and update procedures. |

## 8.1 Check 3 GPS Field Habit

Crews should use the Check 3 GPS field habit before launch. Gear means the aircraft, batteries, dock, controller, payload, Remote ID, lighting, memory cards, firmware, first aid kit, fire extinguisher, PPE, and required documents. Plan means the mission objective, airspace, weather, NOTAMs, TFRs, launch and recovery, emergency landing zones, communications, data plan, public interaction, and deconfliction. Skills means RPIC currency, VO or airspace-monitor readiness, platform proficiency, fatigue, workload, crew roles, abnormal procedures, and stop-flight authority.

## 8.2 Risk Assessment and Stop-Flight Authority

Each mission must be screened for risk. The agency may use a brief risk worksheet for routine operations and a Project Aviation Safety Plan for higher-risk operations. Hazards should be identified across aircraft and equipment, operating environment, and human factors. Any crewmember may call for a pause or stop when they observe a condition that may affect safety. The RPIC must evaluate the concern and may not continue until the concern has been resolved, mitigated, or documented as acceptable.

| Risk level | Operational meaning | Approval expectation |
| --- | --- | --- |
| Low | Standard mission with ordinary hazards controlled by routine procedures. | RPIC may proceed after required checks. |
| Medium | Mission has additional complexity such as night, public area, high workload, emergency incident, limited landing zones, or sensitive data. | RPIC and mission lead must document mitigation before launch. |
| Serious | Mission includes elevated airspace, public, tactical, fireground, BVLOS, mutual-aid, or data/legal complexity. | UAS Program Manager or designee approval required unless emergency conditions require immediate action. |
| High / Critical | Mission creates significant risk of injury, crewed-aircraft conflict, major property damage, legal violation, or major public-trust impact. | Mission may not proceed until risk is reduced or an emergency command decision documents why the benefit outweighs remaining risk. |

# 9. Mission Request, Authorization, and Planning

Every operational mission must begin with a defined public purpose. Routine training in approved areas may be authorized through a standing training plan. Operational missions, public-safety incidents, mutual-aid missions, contractor missions, controlled-airspace missions, and DFR deployments must be documented through a mission request, CAD record, incident action plan, PASP, launch log, or approved electronic equivalent.

## 9.1 Launch Matrix and Mission Request

The agency should maintain a launch matrix that assigns call types to approval tiers. Low-risk public-safety calls may be approved by dispatch, RTIC/RTCC, or preauthorized criteria when the launch matrix is satisfied. Higher-risk calls should require supervisor, incident commander, or mission lead approval. Each launch must answer the public-safety problem being addressed, the decision the aircraft is expected to support, the legal and FAA pathway, the people who may view the live feed, the data that will be retained, and the condition that ends the mission.

## 9.2 Project Aviation Safety Plan

A PASP is required for missions outside designated practice areas, missions involving contractors or mutual aid, operations near controlled airspace or airports, public events, multiple aircraft or crews, public-safety incidents, fire or law-enforcement tactical operations, BVLOS, and any mission rated Medium or higher after initial screening.

| PASP element | Required information |
| --- | --- |
| Mission objective | Clear statement of public purpose, incident or project identifier, intended decision support, and expected deliverables. |
| Operating authority | Part 107, Part 107 waiver, public-aircraft COA, Part 91 waiver, SGI, ECOA, contractor authority, or other FAA authorization. |
| Crew and roles | RPIC, VO, airspace monitor, teleoperator, payload operator, trainee, mission lead, incident command contact, data steward, contractor. |
| Aircraft and payload | Make/model, registration, Remote ID, dock, battery, sensor, live-stream platform, software version, and maintenance status. |
| Airspace | Class, authorization, LAANC/manual record, NOTAMs, TFRs, airports, heliports, special-use airspace, air-medical pads, routes, altitude limits, and deconfliction contacts. |
| Site hazards | Wires, towers, terrain, trees, traffic, pedestrians, crowds, responders, smoke, water, wildlife, RF interference, magnetic interference, weather, lighting, and public interaction. |
| Data plan | Data to collect, live-feed recipients, storage system, access limits, retention schedule, evidence or incident status, intergovernmental sharing, and public-records handling. |
| Emergency plan | Lost link, flyaway, crewed aircraft, low battery, emergency landing, crash, injury, fire, unauthorized UAS, public complaint, and data incident procedures. |

## 9.3 Site Survey and Crew Briefing

The RPIC or designated planner should conduct a remote site assessment before the mission and an onsite survey before flight when feasible. The survey should identify launch and recovery sites, emergency landing zones, VO positions, public access controls, crew staging, obstacles, RF or magnetic interference, privacy-sensitive areas, roads, rails, water, overhead utilities, heliports, airports, and areas to avoid. Before each operational period, the RPIC must brief roles, objective, aircraft, payload, airspace, weather, boundaries, lost-link plan, emergency procedures, communications, sterile cockpit expectations, public interaction, data handling, stop-flight authority, and crew concerns.

# 10. Standard Flight Operations Procedures

Standard flight operations are organized as a sequence of day-before or pre-mission review, site setup, preflight, launch, flight, landing, postflight, and abnormal procedures. The agency may implement these procedures through electronic checklists, CAD fields, dock software, or paper forms, but the required decision points must remain visible to the RPIC and UAS Program Manager.

## 10.1 Pre-Mission Review

Before the mission, the RPIC or designee must confirm mission approval, public purpose, site access, operating authority, registration, Remote ID, pilot and crew assignment, aircraft status, insurance or contract coverage, and data-handling plan. The review must include airspace, LAANC or manual authorization, NOTAMs, TFRs, weather, obstacles, launch/recovery area, emergency landing zones, air-medical pads, fire aviation, law-enforcement aviation, and communications.

## 10.2 Site Setup and Preflight

At the site or dock, the crew must establish a safe launch and recovery area, identify emergency landing zones, position visual observers or airspace monitors, verify public access controls, check weather and airspace, and inspect the aircraft. The preflight inspection must include airframe, propellers, motors, landing gear, payload, gimbal, lights, Remote ID, batteries, dock, controller, mobile device, antennas, memory card, GNSS, compass/IMU, home point, RTH altitude, geofence, obstacle sensing, lost-link settings, and positive control response.

## 10.3 Launch, Flight, and Landing

The RPIC or teleoperator must announce launch, confirm crew readiness, climb to a safe check altitude, verify stability, control link, video link, telemetry, battery, GNSS, Remote ID, and airspace status, and remain within approved mission boundaries. The crew must yield to crewed aircraft, maintain sterile cockpit during high-workload periods, avoid unnecessary collection outside the mission area, monitor people and vehicles, watch battery and weather, and land when safety, policy, or the mission objective no longer supports continued flight. After landing, the crew must power down, inspect equipment, secure data and batteries, complete logs, record discrepancies, and debrief hazards or lessons learned.

| Abnormal condition | Expected response |
| --- | --- |
| Crewed aircraft or emergency aircraft observed | Descend or land immediately when required to maintain separation; announce to the crew; do not relaunch until deconfliction is assured. |
| Lost link or command-and-control anomaly | Follow the pre-briefed lost-link procedure and RTH settings; monitor expected behavior; notify airspace coordinator, incident command, dispatch, or ATC contact as applicable. |
| Low battery or battery warning | Return and land before critical battery; use a contingency landing zone if the primary zone is not reachable; isolate damaged or overheated batteries. |
| GNSS, compass, or navigation anomaly | Stop mission tasks, maintain safe control if possible, transition only to a mode the RPIC is trained to use, and land as soon as practical. |
| Flyaway or loss of control | Warn crew and nearby personnel, attempt recovery if safe, notify affected airspace contacts, preserve logs, and complete internal and regulatory reports. |
| Public enters launch or recovery area | Pause launch or abort landing when safe, assign a non-flying crewmember to clear the area, and document any safety concern or complaint. |
| Weather deterioration or smoke obscuration | Return and land before limits are exceeded or visibility is insufficient; reassess mission risk before relaunch. |

# 11. Emergency Response, Mishaps, and Reporting

When an accident, incident, crash, flyaway, injury, fire, serious anomaly, unauthorized UAS encounter, privacy complaint, or data incident occurs, life safety takes priority over aircraft, data, and property. The crew should start a log of actions, protect people, prevent additional harm, preserve evidence when safe, notify required internal and external contacts, recover equipment only when safe, and complete corrective action before returning the aircraft or crew to mission status.

| Event | Immediate notification standard |
| --- | --- |
| Injury, serious injury, fire, hazardous material, threat to public, or emergency | Call 911 or request emergency assistance first, then notify the agency supervisor and UAS Program Manager. |
| FAA Part 107 reportable safety event | Notify the FAA within required timelines when criteria are met; preserve logs and related evidence. |
| NTSB accident or listed serious incident | Notify the NTSB by the most expeditious means when Part 830 criteria are met. |
| Lost aircraft, flyaway, unsafe lost link, collision, or substantial property damage | Notify supervisor, UAS Program Manager, property owner when applicable, affected airspace contacts, and FAA/NTSB if reportable. |
| Public complaint or privacy concern | Notify the UAS Program Manager, Public Information Officer or privacy/records officer, and supervisor. |
| Data loss, device loss, unauthorized disclosure, or suspected compromise | Notify IT/security, records officer, UAS Program Manager, and follow agency incident-response procedures. |
| Law-enforcement operation or evidence concern | Notify law-enforcement chain of command, evidence custodian, records officer, counsel, and Annex B contacts. |
| Mutual-aid, incident command, wildfire, or aviation-managed incident | Notify the designated operational contact, airspace coordinator, incident command, and any required aviation-safety reporting channel such as SAFECOM when applicable. |

## 11.1 Return to Service

An aircraft involved in a crash, hard landing, propeller strike, motor anomaly, battery event, flyaway, abnormal lost link, water exposure, structural concern, or significant software anomaly may not be returned to service until inspected, repaired if needed, test flown in a safe environment if appropriate, and released by the UAS Program Manager or Maintenance Coordinator. The release must be documented in the maintenance log.

# 12. Data Management, Privacy, Cybersecurity, and Public Records

DFR and public-safety UAS data must be governed before launch. The agency shall collect only what is reasonably necessary for the authorized mission, identify expected data products, assign a data steward when data will be retained or shared, store data in approved systems, restrict access based on mission need, retain and dispose of data under the applicable schedule, and document third-party storage, contractor platforms, cloud systems, and intergovernmental sharing.

## 12.1 Privacy-by-Design Operating Rules

Camera direction, altitude, zoom, route, geofencing, masking, and post-processing should be used to avoid unnecessary collection of uninvolved persons, license plates, homes, fenced yards, medical scenes, protected activity, or unrelated private activity. While en route, the camera should be pointed toward a public travel corridor, horizon, or other approved non-sensitive direction unless immediate life-safety needs require otherwise. Random surveillance is prohibited.

## 12.2 Law-Enforcement Legal Process and Evidence

Before using DFR for criminal investigation, evidence gathering, search, surveillance, tracking, or monitoring of protected activity, the supervisor must identify the legal basis. Depending on state law and the facts, legal authority may require a warrant, consent, emergency-aid exception, exigency, search-and-rescue authority, community-care authority, court order, incident-specific statutory authority, or another counsel-approved basis. Evidentiary data must be segregated, preserved, and transferred under evidence procedures.

## 12.3 Fire/EMS and Sensitive Incident Data

Fire/EMS DFR may capture patient images, injuries, medical treatment, fatality scenes, rescue extrication, disaster victims, or vulnerable persons. Such data should be treated as sensitive even when it is not evidence. Access should be limited to personnel with a mission need, public release should be coordinated with the PIO, incident command, records officer, and counsel, and training use should remove or protect identifiable private information whenever practical.

## 12.4 Retention and Public Transparency

The agency must distinguish live view, transitory recordings, operational recordings, evidence, training data, maintenance logs, flight logs, annual reports, and public transparency data. Retention periods must be inserted in Appendix A. The agency should publish a public version of this policy, authorized mission categories, privacy protections, public records instructions, complaint process, annual report location, and summary statistics when permitted by law.

| Data type | Suggested handling | Local retention placeholder |
| --- | --- | --- |
| Live view not recorded | Access limited to authorized personnel with mission need; viewing should be logged when technology permits. | [INSERT] |
| Routine DFR recording with no evidentiary value | Retain only if required for accountability, incident review, training, or local transparency; delete under schedule. | [INSERT] |
| Public-safety incident data | Handle under incident, emergency-management, fire, EMS, or evidence-related schedules as applicable. | [INSERT] |
| Law-enforcement evidence | Handle under evidence rules, court orders, discovery obligations, public records, and law-enforcement retention schedules. | [INSERT] |
| Training recordings | Use de-identified or controlled data where feasible; do not use law-enforcement training data as evidence unless legally permitted. | [INSERT] |
| Flight, maintenance, and authorization records | Retain enough to support compliance, safety assurance, aircraft maintenance, pilot currency, annual reporting, and audits. | [INSERT] |

# 13. Maintenance and Continued Airworthiness

The agency must operate only UAS that are in a condition for safe operation. The RPIC must perform or verify a preflight inspection before each flight. Aircraft, docks, batteries, payloads, controllers, mobile devices, Remote ID modules, chargers, and accessories must be maintained in accordance with manufacturer guidance, FAA authorization, agency policy, and conservative inspection practices when manufacturer guidance is incomplete.

Each UAS must have a maintenance log that records inspections, damage, repairs, firmware/software changes, hardware changes, dock changes, battery events, payload changes that affect performance, test flights, anomalies, and return-to-service approvals. Firmware and software changes can alter aircraft behavior, failsafe settings, Remote ID, geofencing, flight modes, sensor function, or data handling; therefore, updates should be reviewed before installation when feasible, documented, and followed by controlled testing when they affect flight performance or critical systems.

| Maintenance record field | Required content |
| --- | --- |
| Aircraft identification | Make, model, serial number, agency ID, FAA registration, Remote ID serial number, dock location, and assigned unit. |
| Inspection or maintenance action | Date, time, person performing the action, reason for action, parts replaced, firmware/software versions, battery information, and test results. |
| Battery management | Battery serial number, cycle count, condition, swelling, damage, temperature, charge/discharge status, storage condition, and retirement decision. |
| Return to service | Name and title of approving person, operational limitations, test flight results, and date aircraft was released. |

# 14. Interagency Operations, Mutual Aid, and Airspace Deconfliction

Interagency operations require clear authority, clear operational control, compatible communications, common safety expectations, and shared understanding of data ownership and public-records responsibilities. Before a joint mission, participating agencies should identify who owns the mission, who controls the flight, who may task the aircraft, who may receive data, who handles public records requests, who reports incidents, and how the resource will be demobilized.

## 14.1 Mutual-Aid Policy

A DFR or UAS resource may provide or receive mutual aid only through a documented request, automatic-aid agreement, mutual-aid agreement, emergency management request, regional task force, incident command assignment, or lawful verbal emergency request that is later documented. Self-dispatch is prohibited. The agreement or mission record should address ordering authority, reimbursement, liability, insurance, pilot qualification, aircraft registration, Remote ID, FAA pathway, operational control, data custody, public records, evidence, cybersecurity, safety reporting, and demobilization.

## 14.2 Multiagency Airspace Deconfliction

At any incident involving more than one UAS, crewed aircraft, a helipad, an air-medical landing zone, wildfire aviation, a TFR, an airport or heliport, or a high-consequence target, the incident commander or operations lead should designate a UAS Airspace Coordinator or Airspace Monitor. That role does not replace the RPIC. It creates a single airspace coordination point to track aircraft, altitude blocks, sectors, launch and recovery zones, lost-link routes, emergency landing areas, radio or phone channels, TFR/NOTAM status, and "land now" procedures.

Crewed aircraft have priority. If deconfliction cannot be assured, UAS operations must be suspended. Deconfliction should use plain language, altitude limits, lateral sectors, hold points, route assignments, no-fly zones, launch windows, emergency return corridors, and a shared incident airspace picture. The airspace plan should identify local airports, hospital pads, private heliports, helitack bases, law-enforcement aviation, news helicopters, utility helicopters, fire aircraft, and military or state aviation resources when relevant.

## 14.3 Fire, EMS, Wildfire, and Air-Medical Deconfliction

Fire and EMS incidents create airspace hazards that may not appear on a map. Air-medical helicopters may arrive quickly, law-enforcement aircraft may orbit, utility aircraft may inspect lines, and firefighting aircraft may operate low over smoke and terrain. UAS crews must coordinate through incident command and, when applicable, the air operations function before launching. For wildland and WUI incidents, UAS crews must follow incident airspace instructions, TFR restrictions, and Fire Traffic Area communication and clearance practices before operating near fire aviation.

Unauthorized or uncoordinated UAS near wildland fire operations can create serious collision risk because fire aircraft often operate at low altitudes similar to UAS operating altitudes. When unauthorized UAS activity is observed, personnel should report the time, location, direction, approximate altitude, description, flight pattern, Remote ID information if available, and operator location if known through appropriate incident, dispatch, aviation, or supervisory channels. Personnel should not create a greater hazard by pursuing, disabling, or seizing a UAS unless specifically authorized by law and agency policy.

## 14.4 Law-Enforcement Tactical Deconfliction

Law-enforcement incidents may involve tactical teams, perimeter units, negotiators, fire/EMS staging, medevac, law-enforcement aviation, news aircraft, and neighboring agencies. The tactical commander should identify one UAS tasking point and one airspace coordination point. When a UAS supports a barricade, hostage, active violence, search, warrant service, pursuit, or perimeter mission, the mission plan must address privacy, legal process, evidence, tactical radio discipline, officer safety, fire/EMS staging, air-medical contingency, and video-sharing limits.

# 15. Optional Annex A: Fire/EMS, Public Safety, and Emergency Operations

Customization note: Adopt this annex only if the agency conducts DFR, fire/rescue, EMS, search and rescue, disaster, emergency management, hazmat, incident command, or related public-safety missions. Agencies that do not conduct these missions should delete or mark this annex "Not adopted."

This annex provides supplemental procedures for fire/rescue, EMS, search and rescue, emergency management, hazardous materials, disaster response, and public-safety incident support. It does not create law-enforcement authority. If a public-safety mission may become a criminal investigation or produce evidence, the mission lead should coordinate with law enforcement, counsel, and records/evidence personnel as soon as practical.

## 15.1 Fire/Rescue and EMS DFR Model

Fire/EMS DFR is centered on incident command, life safety, and responder safety. Typical uses include early scene size-up, smoke and fire observation from a safe altitude, structure-fire exposure assessment, wildfire/WUI reconnaissance, hazardous-materials standoff assessment, roof or collapse hazard assessment, water-rescue support, missing-person support, evacuation route assessment, and disaster damage assessment. The aircraft should provide information that improves decisions about apparatus placement, evacuation, rescue entry, hazard zones, air-medical landing, mutual aid, and public warnings.

## 15.2 Incident Command Integration

UAS operations at incidents should be requested, assigned, and deconflicted through the Incident Commander or designated operations or air function. The RPIC retains final flight-safety authority even when operating under incident command tasking. UAS data should be delivered through approved incident data pathways and retained under applicable incident records schedules. Live-stream access should be limited to incident personnel and agency staff with a mission need.

## 15.3 Fireground, Wildfire, Hazmat, SAR, and Disaster Standards

Fireground operations require the UAS crew to avoid interfering with ladders, aerial devices, hose streams, rotor wash, utility hazards, collapse zones, and suppression tactics. Wildfire/WUI operations require coordination with incident aviation and awareness of TFRs, Fire Traffic Area procedures, smoke, wind, terrain, and rapidly changing hazards. Hazmat operations require safe standoff, appropriate sensor limits, responder PPE coordination, and avoidance of prop wash that could spread contaminants. Search and rescue operations require a clear search plan, communications with incident command, data logging, and careful management of victim imagery. Disaster operations require coordination with EOC, public works, utilities, shelter operations, transportation, and public information.

# 16. Optional Annex B: Law-Enforcement Operations

Customization note: Adopt this annex only for a law-enforcement agency or for a non-law-enforcement agency that provides UAS support to law enforcement under a counsel-reviewed interagency agreement. Local law may impose restrictions beyond this generic template.

Law-enforcement DFR is centered on lawful situational awareness, de-escalation, responder safety, public safety, evidence integrity, and community trust. DFR may not be used to bypass legal-process requirements. The mission supervisor must identify the lawful purpose and legal basis before flight when time allows, and must document the basis as soon as practical during exigent or emergency circumstances. State law may require a warrant, consent, emergency-aid documentation, exigency findings, search-and-rescue authority, declared-emergency authority, or other specific statutory pathway.

## 16.1 Law-Enforcement Mission Approval

Before launch, the mission record should identify the call type, public-safety objective, supervisor or command authority, legal basis, requested flight area, expected sensor use, live-stream recipients, recording decision, retention category, evidence custodian if any, and termination criteria. The aircraft should be flown and the camera should be oriented in a way that minimizes unnecessary viewing of homes, fenced yards, protected activity, and unrelated private activity while still satisfying the lawful public-safety purpose.

## 16.2 Tactical and Evidence Standards

When DFR supports tactical operations, the UAS must be integrated into the tactical plan. The mission supervisor should address officer safety, suspect and bystander safety, fire/EMS staging, air-medical contingencies, live-feed access, radio discipline, launch and recovery safety, tactical deconfliction, and perimeter assignments. When the mission may produce evidence, the agency must preserve original media, metadata, flight logs, chain of custody, viewer access records when available, and any related CAD or RTIC/RTCC notes.

## 16.3 Support by Non-Law-Enforcement Agencies

When a non-law-enforcement agency provides aircraft, pilots, teleoperators, airspace monitors, or data to law enforcement, the support must be governed by an interagency agreement or emergency command assignment reviewed by counsel when time allows. The mission record should identify the requesting law-enforcement agency, legal basis, operational control, RPIC, data custodian, evidence handling, public-records responsibilities, and who may direct sensor use. Non-law-enforcement pilots should not independently decide law-enforcement legal questions in the field.

# 17. Appendices and Forms

The following appendices may be used as written, converted into electronic systems, or replaced with equivalent agency forms. Forms should be simple enough for field use and complete enough to support annual reporting, safety assurance, records retention, FAA compliance, and legal review.

## Appendix A - Customization Matrix

| Section | Decision needed | Agency selection / notes |
| --- | --- | --- |
| Cover | Agency name, version, effective date, custodian, program owner, and review cycle. | [INSERT] |
| 1. Authority | Add state, local, tribal, regional, and agency-specific authority; remove authorities that do not apply. | [INSERT] |
| 3 Roles | Identify role holders, alternates, and succession for 24/7 DFR coverage. | [INSERT] |
| 4 Mission categories | Select authorized mission types and delete or mark unused categories not adopted. | [INSERT] |
| 5 FAA pathway | State when flights will use Part 107, public-aircraft COA, Part 91 waiver, SGI, contractor Part 107, or a combination. | [INSERT] |
| 5 State law | Insert state UAS statutes, privacy law, public records law, law-enforcement authority, fire/EMS confidentiality, and data retention schedules. | [INSERT] |
| 7 Training | Set agency-specific flight currency, annual review, platform checkout, DFR/BVLOS endorsement, and mission specialty requirements. | [INSERT] |
| 12 Data | Set retention periods, storage systems, live-feed access rules, public posting location, third-party terms, and intergovernmental disclosure policy. | [INSERT] |
| 14 Mutual aid | Insert mutual-aid agreements, regional airspace conventions, incident command contacts, airport/heliport contacts, and demobilization procedures. | [INSERT] |
| 15 Annex A | Adopt, delete, or modify fire/EMS and public-safety annex. | [INSERT] |
| 16 Annex B | Adopt, delete, or modify law-enforcement annex. | [INSERT] |

## Appendix B - FAA and DFR Authorization Binder Checklist

| No. | Checklist item | Local note / initials |
| --- | --- | --- |
| 1 | Place the current public-safety operating pathway memorandum in the binder and identify whether each mission category uses Part 107, a Part 107 waiver, public-aircraft COA, Part 91 waiver, SGI, contractor authority, or another FAA authorization. | [ ] |
| 2 | Insert copies of current remote pilot certificates, recurrent training completion, local appointments, and role-specific endorsements for all RPICs and teleoperators. | [ ] |
| 3 | Insert aircraft FAA registration, Remote ID serial numbers, declarations of compliance when applicable, state registrations when required, and fleet inventory records. | [ ] |
| 4 | Insert all Part 107 waivers, public-aircraft COAs, Part 91 waivers, Section 927 waivers, SGI approvals, operating limitations, maps, grids, routes, and approved operating areas. | [ ] |
| 5 | Insert the approved DFR concept of operations, launch matrix, dispatch workflow, RTIC/RTCC workflow, live-feed access rules, and termination criteria. | [ ] |
| 6 | Insert controlled-airspace authorizations, LAANC records, manual authorizations, airport/heliport contacts, TFR/NOTAM procedures, and local airspace deconfliction protocols. | [ ] |
| 7 | Insert emergency procedures, lost-link/RTH profiles, emergency landing sites, crewed-aircraft conflict procedures, and "land now" phraseology. | [ ] |
| 8 | Insert training curriculum, proficiency standards, NIST-style test records, DFR/BVLOS scenario records, and annual exercise documentation. | [ ] |
| 9 | Insert vendor contracts, data-processing agreements, cybersecurity approvals, insurance certificates, and operational-control assignments for contractor-supported DFR. | [ ] |
| 10 | Insert annual program review, audit findings, corrective actions, community transparency report, complaints, and policy update records. | [ ] |

## Appendix C - DFR Launch Checklist

| No. | Checklist item | Local note / initials |
| --- | --- | --- |
| 1 | Confirm the call type or mission is authorized under the launch matrix and document the mission purpose in CAD, RTIC/RTCC, or the approved mission log. | [ ] |
| 2 | Confirm the RPIC, teleoperator, airspace monitor, visual observer if required, and mission supervisor are assigned and fit for duty. | [ ] |
| 3 | Confirm the aircraft, dock, battery, payload, Remote ID, anti-collision lighting, C2 link, live-stream link, and geofence/RTH settings are ready. | [ ] |
| 4 | Confirm the FAA pathway, airspace authorization, altitude limit, route, launch area, emergency landing zones, and deconfliction plan. | [ ] |
| 5 | Confirm weather, visibility, wind, precipitation, smoke, lighting, temperature, and local hazards are within limits. | [ ] |
| 6 | Set camera direction and sensor mode to minimize unnecessary collection while still meeting the public-safety objective. | [ ] |
| 7 | Confirm live-feed recipients and recording/retention category before launch or as soon as practical during emergencies. | [ ] |
| 8 | Announce launch or log launch in the approved system and maintain sterile cockpit until the aircraft is stable and the mission is underway. | [ ] |
| 9 | Terminate the flight when the mission objective is complete, safety requires landing, deconfliction cannot be assured, or the authorized period ends. | [ ] |

## Appendix D - Project Aviation Safety Plan Checklist

| No. | Checklist item | Local note / initials |
| --- | --- | --- |
| 1 | Define the mission objective and the decision the aircraft is expected to support. | [ ] |
| 2 | Identify agency, division, incident or project number, mission lead, RPIC, crew, data steward, and approving authority. | [ ] |
| 3 | Identify aircraft, payload, Remote ID status, FAA registration, state registration when required, maintenance status, dock, and software version. | [ ] |
| 4 | State the operating authority, including Part 107, waiver, COA, SGI, ECOA, Part 91 waiver, Section 927 waiver, or contractor authority. | [ ] |
| 5 | Attach maps showing operating area, coordinates, launch/recovery, emergency landing zones, obstructions, sensitive areas, and avoid zones. | [ ] |
| 6 | Review nearby airports, heliports, helipads, fire aviation, air-medical LZs, special-use airspace, TFRs, NOTAMs, UASFM grids, and known aviation hazards. | [ ] |
| 7 | Evaluate weather, lighting, terrain, wires, towers, roadways, railways, waterways, public access, wildlife, RF interference, and magnetic interference. | [ ] |
| 8 | Document communications plan, incident command or dispatch contact, airspace coordinator, aviation frequencies if lawful and appropriate, and emergency contacts. | [ ] |
| 9 | Document data objective, live-feed recipients, storage location, retention category, public-records considerations, and privacy controls. | [ ] |
| 10 | Document emergency procedures for lost link, flyaway, low battery, crewed aircraft, crash, injury, fire, intrusion, public complaint, and data incident. | [ ] |

## Appendix E - Mutual Aid and Multiagency Airspace Deconfliction Checklist

| No. | Checklist item | Local note / initials |
| --- | --- | --- |
| 1 | Verify the request is authorized through dispatch, incident command, automatic aid, mutual aid, EOC, regional task force, or another lawful channel. | [ ] |
| 2 | Identify the requesting agency, providing agency, incident identifier, mission objective, operational period, and reimbursement or cost status. | [ ] |
| 3 | Document which agency has operational control and which RPIC controls each aircraft. | [ ] |
| 4 | Confirm FAA pathway, airspace authorization, Remote ID, pilot qualification, aircraft registration, insurance, and data handling for each agency or contractor aircraft. | [ ] |
| 5 | Designate a UAS Airspace Coordinator or Airspace Monitor when more than one UAS, crewed aircraft, helipad, TFR, wildfire aircraft, or air-medical resource may be involved. | [ ] |
| 6 | Establish operating area, lateral sectors, altitude limits, launch and recovery zones, emergency landing areas, lost-link routes, and emergency return corridors. | [ ] |
| 7 | Identify all known aircraft, including law-enforcement aviation, fire aircraft, air-medical helicopters, utility aircraft, military aircraft, news aircraft, and other UAS. | [ ] |
| 8 | Publish the radio, phone, CAD, RTIC/RTCC, EOC, UTM, chat, or other channel used for airspace coordination. | [ ] |
| 9 | Establish "hold," "land now," "lost link," "flyaway," and "aircraft conflict" phraseology in plain language. | [ ] |
| 10 | Document demobilization, data custody, records transfer, evidence transfer, public-information coordination, and after-action reporting. | [ ] |

## Appendix F - Fire/EMS DFR Checklist

| No. | Checklist item | Local note / initials |
| --- | --- | --- |
| 1 | Confirm the incident command structure, fire/EMS mission objective, staging location, and ground contact before launch or as soon as practical. | [ ] |
| 2 | Confirm whether the mission involves structure fire, wildfire/WUI, rescue, hazardous materials, water incident, disaster assessment, patient access, or air-medical LZ awareness. | [ ] |
| 3 | Determine whether air-medical, law-enforcement aviation, fire aircraft, utility aircraft, news aircraft, or other crewed aviation is operating or expected. | [ ] |
| 4 | Coordinate with incident command before overflying responders, patients, hose lines, ladders, aerial devices, collapse zones, or hazmat zones. | [ ] |
| 5 | Set sensor use to support life safety, responder safety, hazard assessment, evacuation, or incident stabilization and avoid unnecessary patient or private imagery. | [ ] |
| 6 | Maintain deconfliction with crewed aviation and land immediately when required by incident command, air operations, RPIC judgment, or loss of deconfliction. | [ ] |
| 7 | Route live video and data only to incident personnel with a mission need and protect sensitive patient, fatality, or victim imagery. | [ ] |
| 8 | Close the mission with incident command, record operational value, and identify whether data is transitory, incident record, training material, or evidence. | [ ] |

## Appendix G - Law-Enforcement DFR Checklist

| No. | Checklist item | Local note / initials |
| --- | --- | --- |
| 1 | Confirm the call type, lawful mission objective, supervisor or command authority, and whether the mission is an emergency, investigation, search, tactical event, or public-safety support mission. | [ ] |
| 2 | Determine whether a warrant, consent, emergency aid, exigency, search and rescue authority, declared emergency, community-care authority, or other legal basis is required. | [ ] |
| 3 | Set camera direction, zoom, route, altitude, and sensor settings to minimize unnecessary viewing of homes, fenced yards, protected activity, and unrelated private activity. | [ ] |
| 4 | Limit live-feed access to personnel with a mission need and record viewers when technology and policy permit. | [ ] |
| 5 | Coordinate with tactical command, fire/EMS staging, airspace coordinator, evidence custodian, and public-information officer when applicable. | [ ] |
| 6 | Document whether recording is off, transitory, operational record, or evidence, and preserve media and metadata when evidence is reasonably anticipated. | [ ] |
| 7 | Terminate or modify the flight when the legal basis changes, the mission objective is complete, safety requires landing, or deconfliction cannot be assured. | [ ] |
| 8 | Complete postflight mission record, evidence transfer if applicable, public-records category, privacy concern notes, and after-action observations. | [ ] |

## Appendix H - BVLOS / Shielded Operations Readiness Checklist

| No. | Checklist item | Local note / initials |
| --- | --- | --- |
| 1 | Confirm the agency has written FAA authority for the exact BVLOS or shielded operation and that the operation is within the authorized FAA pathway. | [ ] |
| 2 | Confirm whether the operation is Part 107, public-aircraft/Part 91, SGI, Section 927, or another FAA authorization and ensure the required responsible person is identified. | [ ] |
| 3 | Confirm approved ConOps, route, altitude, obstruction-shielding assumptions, contingency procedures, emergency landing zones, and lost-link/RTH settings. | [ ] |
| 4 | Confirm aircraft Remote ID, anti-collision lighting, position lighting if required, ADS-B or approved air-traffic awareness resources if required, C2 link, battery reserve, and health status. | [ ] |
| 5 | Confirm airspace review, TFR/NOTAM check, local airport/heliport contacts, hospital pad contacts, fire/air-medical/law-enforcement aviation deconfliction, and no-fly areas. | [ ] |
| 6 | Confirm remote crew roles, teleoperator, RPIC, airspace monitor, visual observer if required, dispatch/RTIC/RTCC contact, and sterile cockpit procedures. | [ ] |
| 7 | Confirm that emergency procedures, lost-link notifications, flyaway reporting, crewed-aircraft conflict response, and immediate landing authority are understood. | [ ] |
| 8 | Document the flight and any deviation, hazard, or communication issue for authorization compliance and safety assurance. | [ ] |

## Appendix I - Training Record and Proficiency Sign-Off

| No. | Checklist item | Local note / initials |
| --- | --- | --- |
| 1 | Verify FAA remote pilot certificate or role-specific prerequisite, as applicable. | [ ] |
| 2 | Complete agency manual orientation, legal/privacy, public records, cybersecurity, evidence, data-retention, and community-engagement training. | [ ] |
| 3 | Complete aircraft-specific, dock-specific, payload-specific, battery, maintenance, and emergency-procedure training. | [ ] |
| 4 | Complete dispatch/RTIC/RTCC workflow, launch matrix, communications, sterile cockpit, and crew resource management training. | [ ] |
| 5 | Complete visual observer or airspace monitor training when assigned to that role. | [ ] |
| 6 | Complete fire/EMS, law-enforcement, SAR, hazmat, disaster, or BVLOS endorsement training when assigned to those missions. | [ ] |
| 7 | Demonstrate basic flight proficiency, precision maneuvers, emergency landing, lost-link response, and payload control using agency standards or NIST-style tests. | [ ] |
| 8 | Complete supervised operational experience and obtain Chief Pilot or UAS Program Manager sign-off before unsupervised operational duty. | [ ] |

## Appendix J - Emergency Procedures Checklist

| No. | Checklist item | Local note / initials |
| --- | --- | --- |
| 1 | For crewed-aircraft conflict, announce the conflict, descend or land if safe and appropriate, and do not relaunch until deconfliction is assured. | [ ] |
| 2 | For lost link, follow the aircraft-specific lost-link procedure, monitor telemetry or last known position, notify airspace contacts as applicable, and prepare for recovery or reporting. | [ ] |
| 3 | For flyaway, notify dispatch, incident command, airspace coordinator, nearby aircraft contacts if applicable, and the UAS Program Manager; preserve logs and report as required. | [ ] |
| 4 | For battery emergency, return or land at the safest available location, keep people clear, isolate the battery, and use fire response procedures if thermal runaway occurs. | [ ] |
| 5 | For crash or injury, protect life safety first, request emergency assistance, preserve evidence when safe, and notify internal and external reporting contacts. | [ ] |
| 6 | For data incident, stop unnecessary access, preserve logs, notify IT/security and records, and follow agency breach and public-records procedures. | [ ] |
| 7 | For public complaint, assign a non-flying contact when possible, listen respectfully, avoid argument, document the concern, and notify the UAS Program Manager. | [ ] |
| 8 | For unauthorized UAS at an incident, report observable information through incident/dispatch/aviation channels and do not create greater hazard by confronting or disabling the UAS unless authorized by law and policy. | [ ] |

## Appendix K - Annual Program Review Checklist

| No. | Checklist item | Local note / initials |
| --- | --- | --- |
| 1 | Review changes to FAA rules, FAA guidance, FAA authorizations, state law, local ordinance, case law, public-records schedules, and agency policies. | [ ] |
| 2 | Summarize total flights, flight hours, DFR launches, fire/EMS missions, law-enforcement missions, SAR/disaster missions, training missions, and mutual-aid deployments. | [ ] |
| 3 | Summarize Part 107 waivers, public-aircraft COAs, Part 91 waivers, controlled-airspace authorizations, SGI requests, BVLOS missions, and any deviations. | [ ] |
| 4 | Review mishaps, near misses, airspace conflicts, crewed-aircraft encounters, lost links, flyaways, privacy complaints, evidence issues, and data incidents. | [ ] |
| 5 | Review pilot currency, training records, DFR/BVLOS endorsements, NIST-style proficiency records, and role authorizations. | [ ] |
| 6 | Review aircraft maintenance, battery retirements, firmware changes, dock performance, Remote ID performance, and vendor service issues. | [ ] |
| 7 | Review mutual-aid agreements, airspace deconfliction contacts, airport/heliport lists, fire aviation contacts, and emergency "land now" procedures. | [ ] |
| 8 | Review public transparency materials, website posting, annual report, public complaints, community engagement, and policy updates. | [ ] |
| 9 | Assign corrective actions, responsible persons, target dates, and management approval for the next review period. | [ ] |

Part II. Training and Proficiency Program

Role-based qualification, competency model, testing standards, recurrent training, records, DFR/BVLOS module, and RPCS integration.

# 1. Training Policy Statement, Scope, and Authority

## 1.1 Training Policy Statement

[AGENCY NAME] will train, qualify, authorize, and periodically evaluate UAS personnel before they perform agency UAS duties. UAS training is aviation training, public-service training, and mission training. A person may be technically capable of flying an aircraft and still be unqualified for agency missions unless the person also understands agency policy, FAA authority, airspace, mission approval, operational control, risk assessment, crew coordination, privacy, public records, cybersecurity, incident command, public interaction, and the specific role being assigned.

The agency will not rely on an FAA Remote Pilot Certificate alone as proof of operational readiness. The certificate establishes federal aeronautical knowledge eligibility for Part 107 operations, but the agency must also verify local policy knowledge, aircraft-specific proficiency, mission judgment, role-specific skills, and continuing currency. The Remote Pilot in Command retains final authority over the safe conduct of flight, and this training program ensures that every crewmember understands how to support that authority and how to stop, delay, or modify an unsafe training or operational event.

## 1.2 Purpose

The purpose of this training program is to create a repeatable pathway from candidate selection to independent operational authorization. The program establishes how personnel learn the UAS policy manual, how they demonstrate knowledge, how they demonstrate practical skill, how they receive mission endorsements, how they remain current, and how the agency documents training. The program also gives supervisors a transparent basis for assigning personnel to routine missions, contractor-supported missions, mutual-aid operations, incident response, DFR shifts, law-enforcement support, fire/EMS operations, and advanced operations such as night, operations over people, operations over moving vehicles, public-aircraft COA operations, or BVLOS/DFR operations when authorized.

## 1.3 Scope

This training program applies to employees, volunteers, interns, reserve personnel, contractors when required by contract, mutual-aid personnel when accepted by agreement, and any person who acts as a UAS crewmember or makes operational decisions for an agency UAS mission. It applies to RPICs, visual observers, payload operators, launch and recovery personnel, dock technicians, airspace coordinators, mission requesters, dispatch or RTIC/RTCC staff, incident commanders who request UAS support, maintenance designees, UAS instructors, UAS evaluators, and UAS proctors.

This program does not certify private pilots for personal or commercial UAS work outside agency operational control. The agency may recognize external training as evidence toward a qualification, but only the UAS Program Manager or authorized designee may approve agency mission qualification.

## 1.4 Authority and Relationship to Operating Policies

Training shall be aligned with the agency UAS operations and policy manual, FAA rules and authorizations, Oregon public body UAS registration and reporting requirements, public records and records retention policies, data and privacy policies, IT/security requirements, procurement requirements, insurance requirements, labor or volunteer agreements, and incident command requirements. Training for Part 107 operations shall include Part 107 rules, certification, recurrent training, airspace authorization, accident reporting, aircraft condition, Remote ID, registration, and operating limitations. Training for public-aircraft COA operations shall include the applicable COA, the agency self-certification responsibilities, pilot qualification requirements, reporting requirements, operational-control provisions, and limitations in the FAA authorization.

## 1.5 Program Exceptions

Exceptions to this training program require written approval by [APPROVAL AUTHORITY] after consultation with the UAS Program Manager and Training Officer. Exceptions involving FAA compliance, BVLOS, public-aircraft COA, operations over people, operations over moving vehicles, controlled airspace, DFR launch from docks, law-enforcement support, fire/wildfire operations, evidence handling, medical privacy, cybersecurity, or public records require counsel or subject-matter review before approval unless an emergency requires immediate temporary action. Any emergency training or qualification deviation must be documented within [72 hours] and reviewed before the person performs unsupervised missions.

# 2. Definitions and Acronyms

Definitions should be consistent with the agency operations and policy manual. The adopting agency may add local terms but should not redefine terms drawn from federal law, state law, or FAA authorization. For training purposes, the following terms are emphasized because they drive role qualification and evaluation.

| Term | Training Program Meaning |
| --- | --- |
| Agency Qualification | A documented agency approval allowing a person to perform a defined UAS role after completing required training, evaluations, and currency requirements. |
| Endorsement | A mission-specific approval added to a base role qualification, such as night, thermal search, mapping, dock operations, DFR teleoperation, evidence collection, fire/EMS support, wildland fire support, or BVLOS/COA operations. |
| Evaluator | A person authorized by the UAS Program Manager to administer checkrides, scenario evaluations, oral boards, NIST-based proficiency events, or role assessments. |
| Instructor | A person authorized to teach agency UAS curriculum and supervise trainees. An instructor may also be an evaluator only if specifically designated. |
| NIST-Based Proficiency Event | A local agency evaluation using NIST-developed sUAS standard test methods. The agency must not imply NIST certification unless that credential is issued through an authorized mechanism. |
| Role Track | The training pathway for a specific role such as RPIC, visual observer, payload operator, launch/recovery technician, airspace coordinator, DFR teleoperator, dock technician, or law-enforcement UAS operator. |
| Training Flight | A flight conducted for qualification, recurrent proficiency, platform checkout, remedial training, scenario evaluation, or instructor/evaluator calibration. |
| Unsupervised Operational Authorization | Agency approval allowing a person to perform a role without direct instructor supervision, subject to mission approval, FAA compliance, currency, and local limitations. |

| Acronym | Meaning |
| --- | --- |
| AAR | After-Action Review |
| AGL | Above Ground Level |
| BVLOS | Beyond Visual Line of Sight |
| COA | Certificate of Waiver or Authorization |
| CRM | Crew Resource Management |
| DFR | Drone as First Responder |
| ECOA | Emergency Certificate of Waiver or Authorization |
| FAA | Federal Aviation Administration |
| FEMA | Federal Emergency Management Agency |
| ICS | Incident Command System |
| LAANC | Low Altitude Authorization and Notification Capability |
| NIMS | National Incident Management System |
| NIST | National Institute of Standards and Technology |
| NOTAM | Notice to Air Missions |
| NWCG | National Wildfire Coordinating Group |
| PASP | Project Aviation Safety Plan |
| RPIC | Remote Pilot in Command |
| RTIC/RTCC | Real-Time Information Center / Real-Time Crime Center |
| SGI | Special Governmental Interest |
| SMS | Safety Management System |
| TEM | Threat and Error Management |
| TFR | Temporary Flight Restriction |
| UAS | Unmanned Aircraft System |
| VO | Visual Observer |

# 3. Training Governance, Roles, and Responsibilities

Training governance must be clear enough that a supervisor can determine who may train, who may evaluate, who may authorize a person for operations, and who maintains records. A single person may hold multiple training roles in a small agency, but the roles should still be named so accountability is not lost.

| Role | Training Responsibilities |
| --- | --- |
| Agency Head or Designee | Approves the training program, assigns authority, provides resources, and resolves policy issues that affect UAS training or qualification. |
| UAS Program Manager | Owns the training program, approves role tracks and endorsements, approves instructors and evaluators, reviews training records, and ensures the training program remains aligned with policy and FAA authorization. |
| Training Officer / Training Coordinator | Schedules training, maintains curriculum, coordinates instructors, tracks completion, maintains the master training calendar, and monitors expiration dates. |
| Flight Operations Coordinator / Chief Pilot | Sets flight standardization, approves platform checkout profiles, supervises checkrides, manages RPIC standardization, and evaluates operational readiness. |
| Instructor | Teaches approved curriculum, supervises trainees, documents attendance and performance, identifies remedial needs, and stops training that becomes unsafe. |
| Evaluator / Check Pilot / Proctor | Administers written exams, oral boards, practical checkrides, NIST-based proficiency events, scenario evaluations, and requalification events according to objective criteria. |
| Records Officer / Data Steward | Ensures training records, proficiency scores, training videos, certificates, and training data are retained and disclosed according to agency policy and law. |
| IT/Security Lead | Approves training systems, simulation platforms, cloud-based training accounts, DFR software sandboxes, credential systems, and training-data storage. |
| Risk Manager / Safety Officer | Reviews high-risk training, training mishaps, student supervision controls, insurance considerations, and corrective-action trends. |
| Agency Counsel | Reviews state/local law training, law-enforcement modules, public records/privacy modules, evidence training, DFR data-sharing, and any training involving legal authority or constitutional concerns. |

## 3.1 Training Review Group

The agency should establish a UAS Training Review Group or include training in the UAS Safety Review Group. The group should meet at least annually and after a significant mishap, failed evaluation trend, regulatory change, new aircraft, new DFR platform, new sensor, new COA/waiver, or new mission type. The group should review completion rates, checkride pass rates, NIST-based proficiency scores, currency lapses, instructor calibration, training mishaps, public complaints, privacy concerns, and corrective actions.

## 3.2 Training Records Control

Training records must be complete enough to prove that a person was qualified for the assigned role at the time of mission. Records should include FAA certificate information, recurrent training dates, agency curriculum completion, platform checkouts, practical evaluations, scenario evaluations, NIST-based test results, role endorsements, currency logs, remedial training, mishap involvement, instructor/evaluator approvals, and expiration dates. The training file should also record limitations, such as daylight-only, no DFR teleoperation, no thermal payload, supervised-only, no law-enforcement support, no COA operations, or no BVLOS operations.

## 3.3 Performance Tracking

The training program should support program performance tracking. The UAS Program Manager or Training Officer should track mission count, training flights, flight hours, avoided hazardous work hours, cost savings, response-time improvements when applicable, safety events, near misses, training failures, remedial plans, currency lapses, aircraft utilization, and compliance trends. Metrics should be used to improve training and to support transparent program management, not to pressure pilots to launch or complete missions that should be delayed or canceled.

# 4. Training Philosophy and Competency Model

The agency training model is competency-based. Personnel must demonstrate knowledge, skill, judgment, communication, and policy compliance before independent assignment. The model combines established aviation certification, standardized practical skills testing, scenario-based training, incident command training when applicable, and local policy evaluation. The model intentionally separates classroom knowledge from practical proficiency because a person can pass a written test and still lack the field judgment needed for a mission.

## 4.1 Five Competency Domains

| Domain | What the trainee must demonstrate | Examples of evidence |
| --- | --- | --- |
| Regulatory and Policy Knowledge | The trainee understands the FAA operating pathway, agency manual, Oregon and local rules, privacy, public records, data policy, mission approval, procurement restrictions, and prohibited uses. | Written exam, oral board, policy acknowledgement, supervisor review, scenario discussion. |
| Aircraft and Systems Proficiency | The trainee can inspect, configure, launch, control, recover, and secure the assigned aircraft, payload, controller, Remote ID, batteries, dock system, and software. | Platform checkout, preflight evaluation, emergency procedure drill, battery safety evaluation. |
| Airmanship and Risk Management | The trainee can plan a safe mission, identify hazards, use the risk matrix, brief the crew, manage airspace, respond to abnormal events, and stop unsafe operations. | PASP exercise, risk worksheet, crew briefing evaluation, checkride, tabletop drill. |
| Mission Integration | The trainee can apply UAS safely and legally to the agency mission, such as inspection, mapping, emergency management, DFR, fire/EMS, SAR, law enforcement, or mutual aid. | Scenario evaluation, incident command exercise, DFR dispatch simulation, project mission endorsement. |
| Professional Conduct and Data Stewardship | The trainee uses UAS in a manner that protects privacy, civil liberties, community trust, public records duties, cybersecurity, evidence, and sensitive data. | Data handling exercise, public interaction scenario, evidence transfer drill, privacy/data exam. |

## 4.2 Existing Training and Testing Methods Integrated into This Program

The training program uses established external methods where they fit the role. The FAA Remote Pilot Certificate and recurrent training remain the baseline for Part 107 RPICs. NIST-developed sUAS test methods provide repeatable practical skill evaluations. FEMA/EMI ICS and NIMS courses provide incident-command and mutual-aid literacy. DRONERESPONDERS and LEDA training and coordination models provide practitioner-led public-safety UAS benchmarking. NFPA 2400 concepts support public-safety program, personnel qualification, operational procedures, safety, and job-performance expectations. NWCG materials provide wildfire and incident aviation concepts for agencies operating in wildland or WUI environments.

| Method | Used For | Agency Use |
| --- | --- | --- |
| FAA Part 107 initial knowledge test and recurrent training | Aeronautical knowledge, certification, and legal baseline for civil small UAS operations. | Required for RPICs operating under Part 107. Recurrent training is tracked every 24 calendar months unless a more restrictive agency standard applies. |
| NIST sUAS standard test methods | Objective measurement of aircraft capability and remote pilot proficiency. | Used for base pilot proficiency, platform checkout, annual evaluation, sensor/payload qualification, and advanced mission endorsements. Agency sets local pass thresholds. |
| Scenario-based practical evaluations | Judgment, crew coordination, mission integration, and pressure-tested decision making. | Used for inspections, mapping, DFR, fire/EMS, SAR, law enforcement, mutual aid, incident airspace deconfliction, and emergency procedures. |
| FEMA/EMI ICS/NIMS courses | Incident command, unified command, resource management, emergency management, and mutual aid. | Required by role for public safety, DFR, fire/EMS, emergency management, mutual aid, airspace coordination, and supervisor roles. |
| DRONERESPONDERS and LEDA training and working groups | Public-safety practitioner training, program management, safety management, fleet management, waiver applications, airspace deconfliction, and discipline-specific training. | Used as recognized continuing education, benchmarking, and curriculum enrichment when approved by the UAS Program Manager. |
| NFPA 2400 public-safety UAS concepts | Public-safety program criteria, operational procedures, personnel qualifications, safety, maintenance, and job-performance requirements. | Used as a public-safety standards framework and as a reference for testable job performance in fire, EMS, SAR, and law-enforcement programs. |
| NWCG wildfire UAS materials | Wildland fire UAS operations, UAS pilot role, Fire Traffic Area, airspace coordination, aviation deconfliction, and incident integration. | Required when UAS personnel support wildland fire, WUI, prescribed fire, wildfire damage assessment, or incident aviation operations. |
| Vendor platform training | Aircraft-specific capabilities, controller workflow, payloads, software, firmware, dock operation, maintenance, and known limitations. | Required before platform checkout and whenever a new aircraft, payload, dock, or major software change is placed into service. |

## 4.3 Training Delivery Methods

The agency should use a blend of classroom instruction, online learning, self-study, instructor-led discussion, tabletop exercises, simulator practice, controlled flight training, standardized practical tests, field scenarios, supervised operational shifts, after-action review, and continuing education. Online modules may be used for policy, legal, privacy, records, cybersecurity, and Part 107 review, but flight skills and incident performance must be evaluated in person or through an approved simulator only when the simulator has a defined training objective and evaluator rubric.

The preferred sequence is orientation, knowledge training, observed practice, coached flight, standardized evaluation, supervised operational experience, endorsement review, and independent authorization. Trainees should not be advanced based only on hours. Hours document exposure. Evaluations document competence.

# 5. Role-Based Qualification Tracks

The agency will qualify personnel by role. A person may be fully qualified as a visual observer but not as an RPIC, as a payload operator but not as a DFR teleoperator, or as an RPIC for routine daylight missions but not for law-enforcement DFR or fireground operations. Each role track must have prerequisites, training modules, evaluations, currency standards, and a written authorization record.

| Role Track | Minimum Qualification Standard | Common Endorsements |
| --- | --- | --- |
| Remote Pilot in Command (RPIC) | FAA Remote Pilot Certificate when operating under Part 107 or agency-approved public-aircraft qualification when operating under COA; agency policy exam; platform checkout; practical evaluation; emergency procedures; data/privacy training. | Night, operations over people, moving-vehicle exposure, thermal, mapping, public works inspection, fire/EMS, law enforcement, mutual aid, instructor, DFR/BVLOS/COA when separately authorized. |
| Visual Observer (VO) | Agency VO training; airspace scan technique; crew communications; ground hazard monitoring; lost-link and emergency procedures; mission environment briefing. | Night VO, public works inspection VO, fireground VO, law-enforcement tactical VO, DFR airspace monitor, multiagency airspace observer. |
| Payload / Sensor Operator | Policy training, payload-specific training, camera/thermal/zoom/recording controls, data minimization, sensor limitations, privacy/data handling, RPIC coordination. | Thermal search, fire thermal, mapping/photo documentation, evidence imagery, hazmat standoff, public information support. |
| Launch/Recovery Technician | Aircraft setup, manual or dock launch, battery handling, launch/recovery zone control, Remote ID check, weather/airspace confirmation, emergency landing support, handoff to remote operator or RPIC. | Dock technician, rooftop operations, mobile launch, battery technician, public-safety staging. |
| Airspace Coordinator / Airspace Monitor | Airspace, LAANC, NOTAM, TFR, crewed-aircraft priority, incident aviation, mutual-aid deconfliction, altitude/sector plan, radio/phone coordination, emergency land-now authority. | Fire Traffic Area, medevac/helipad coordination, airport liaison, multi-UAS operations, DFR BVLOS monitor. |
| Mission Requester / Project Lead | Mission approval, public purpose, PASP request, site access, privacy/data objective, deliverables, public interaction, post-mission data intake. | Inspection/mapping project lead, incident command requester, DFR dispatch decision-maker. |
| DFR Teleoperator / Remote Operator | This is not a base qualification. Requires RPIC qualification if designated as RPIC or supervised-operator qualification if not RPIC; DFR software training; dock/remote launch procedures; C2 monitoring; video latency; geofencing; lost link; dispatch integration; RTIC/RTCC communications; FAA authorization conditions. | DFR/BVLOS, dock operations, RTIC/RTCC integration, law-enforcement DFR, fire/EMS DFR, night, operations over people/moving vehicles. |
| Maintenance / Inventory Designee | Fleet inventory, battery logs, firmware/software control, maintenance records, return-to-service after incident, manufacturer guidance, configuration management. | Dock maintenance, battery program, sensor calibration, post-crash inspection coordinator. |
| Instructor / Evaluator | Current qualification in the role taught; instructor training; evaluator standardization; objective scoring; remediation process; training safety oversight; records completion. | NIST proctor, DFR evaluator, fire/EMS evaluator, law-enforcement evaluator, platform check pilot. |

## 5.1 Qualification Status Categories

Each person should have a current status in the training record. Candidate means the person is approved to begin training. Trainee means the person may participate under instructor supervision. Supervised Operational means the person may participate in real missions only under direct supervision and only if the mission risk supports training. Qualified means the person may perform the role independently within stated limitations. Restricted means the person is qualified only under listed conditions. Suspended means the person may not perform UAS duties until the suspension is lifted. Expired means the person has not maintained currency or recurrent requirements and must complete requalification before operational assignment.

# 6. Core Curriculum Requirements

The core curriculum is required for all operational UAS personnel at the level appropriate to their role. The agency may deliver modules individually or as a combined academy. Each module must have learning objectives, instructor qualifications, course materials, attendance records, evaluation method, and retention location. The Training Officer may credit equivalent external training only when the external training satisfies the local objective and the completion record is placed in the trainee file.

| Module | Required For | Core Content | Evaluation |
| --- | --- | --- | --- |
| T-01 Program Orientation and Manual Review | All personnel | Agency mission, governance, roles, operational control, approved missions, prohibited uses, stop-flight authority, public trust, and how the training program connects to the operations manual. | Policy acknowledgement and orientation quiz. |
| T-02 FAA Rules and Operating Authority | RPIC, DFR, airspace, supervisors; awareness for others | Part 107, public-aircraft COA, waivers, SGI/ECOA, Remote ID, aircraft registration, operations over people, night, moving vehicles, accident reporting, and FAA inspection/records expectations. | Written exam and oral scenario. |
| T-03 Oregon and Local Legal Requirements | All personnel; detailed for law enforcement and records staff | Oregon public body UAS registration and annual reporting, public records, data policies, privacy, local ordinances, park/facility rules, critical infrastructure, law-enforcement limitations, and agency counsel guidance. | Policy exam and case scenarios. |
| T-04 Privacy, Civil Liberties, Records, and Data Stewardship | All personnel | Data minimization, retention, disclosure, public records routing, third-party storage, cybersecurity, social media limits, evidence/medical data, and intergovernmental sharing. | Data handling exercise and written quiz. |
| T-05 Safety Management, Risk Assessment, and PASP | All personnel; advanced for RPIC/mission leads | SMS, hazard identification, risk matrix, weighted risk score option, project aviation safety plan, stop-work authority, voluntary/mandatory reporting, and after-action learning. | Risk worksheet exercise and PASP review. |
| T-06 Airspace, Weather, NOTAMs, TFRs, and LAANC | RPIC, VO, airspace coordinator, DFR staff | Airspace classes, UAS Facility Maps, controlled airspace, LAANC/manual authorization, NOTAM/TFR checks, airports, heliports, hospital pads, weather minimums, and crewed-aircraft priority. | Airspace planning exercise and practical briefing. |
| T-07 Aircraft Systems, Preflight, Batteries, and Maintenance | RPIC, launch/recovery, maintenance, payload operators as needed | Aircraft configuration, preflight, Remote ID, battery safety, firmware/software changes, payload attachment, dock system health, return-to-service requirements, maintenance documentation, and covered UAS/federal funding restrictions when applicable. | Platform checkout and maintenance log exercise. |
| T-08 Basic Flight Skills and Emergency Procedures | RPIC and supervised operators | Manual and automated flight modes, takeoff/landing, hover, orbit, traverse, inspection, recon, lost link, low battery, GNSS/compass anomaly, flyaway, crash, fire, and public intrusion. | NIST-based evaluation and checkride. |
| T-09 Crew Resource Management and Threat/Error Management | All crewmembers | Roles, sterile cockpit, standard callouts, assertive communication, workload management, handoffs, fatigue, distractions, pressure to launch, error traps, and debriefing. | Crew simulation and evaluator observation. |
| T-10 Mission Request, Planning, and Field Operations | RPIC, mission leads, payload, VO, DFR staff | Mission request, mission justification, site survey, launch/recovery, emergency landing zones, privacy-sensitive areas, communications plan, data plan, public interaction, and postflight documentation. | Mission planning package and field scenario. |
| T-11 Contractor and Consultant UAS Operations | Project leads, UAS managers, procurement, contractors as required | Operational control, scope of work, pilot qualification review, insurance, FAA registration/Remote ID, data handling, maintenance records, flight logs, security, public records, and stop-flight authority. | Contractor review exercise. |
| T-12 Public Interaction and Communications | All field personnel | Plain-language explanation of mission, handling complaints, media/public interaction, signage, public safety around launch/recovery areas, and routing public records or privacy questions. | Public-contact scenario. |

Agencies should preserve the sequence of foundation modules before advanced mission endorsements. A trainee should not enter DFR, fireground, law-enforcement, BVLOS, or mutual-aid scenarios until the trainee has demonstrated policy knowledge, basic safety, airspace awareness, and role communication. Advanced modules may be taught early for awareness, but endorsement requires evaluation under the role standard.

# 7. Evaluation, Testing, and Proficiency Standards

The evaluation program must test knowledge, skill, and judgment. Written tests measure policy and regulatory understanding. Oral boards measure decision making and the ability to explain safety choices. Practical tests measure aircraft control, system operation, and emergency response. Scenario evaluations measure mission integration under realistic workload. NIST-based proficiency events provide objective repeatable performance metrics for pilot and platform skills. Tabletop and functional exercises measure incident integration, mutual aid, and DFR workflow.

## 7.1 Written Knowledge Exams

Written exams should be drawn from an agency-controlled question bank that maps questions to learning objectives. Questions should include scenario-based judgment questions, not only recall. The baseline passing score should be [80 percent] for routine role qualification and [90 percent] for DFR/BVLOS/COA, law-enforcement, or high-risk public-safety endorsements unless the agency sets a different threshold. A critical safety or legal failure may require remediation even if the overall score is passing.

| Exam Area | Recommended Question Types | Minimum Passing Standard |
| --- | --- | --- |
| Agency policy and prohibited uses | Multiple-choice, short-answer, and scenario judgment. | [80 percent] and no missed critical prohibited-use question. |
| FAA rules and operating authority | Multiple-choice and scenario questions using Part 107/COA/waiver examples. | [80 percent] for routine roles; [90 percent] for DFR/BVLOS/COA endorsements. |
| Oregon public body registration/reporting | Scenario questions about aircraft registration, public body reporting, and records required for reporting. | [80 percent] and no missed annual reporting question. |
| Privacy, records, data, and public interaction | Scenario questions involving incidental capture, live video, public records, and external sharing. | [80 percent] and no missed critical data-release question. |
| Public-safety/fire/EMS | Scenario questions involving ICS, crewed aircraft, thermal search, incident command, and EMS privacy. | [80 percent] or higher local standard. |
| Law enforcement | Scenario questions involving legal pathway, evidence, public release, and data restrictions. | [90 percent] or counsel-approved standard. |
| DFR/BVLOS/remote operations | Authorization-specific scenarios involving launch criteria, live video, crewed aircraft, lost link, dock status, remote PIC decisions, and mission termination. | [90 percent] and no missed FAA authorization, privacy, or land-now critical item. |

## 7.2 Oral Boards and Decision-Making Evaluation

Oral boards are useful for testing judgment that cannot be captured by a written exam. An evaluator should present realistic situations, such as pressure from command to launch in deteriorating weather, a request to use live video for a non-approved purpose, a medevac helicopter entering the area, a lost-link event, a public complaint at a launch site, a law-enforcement request without a clear legal pathway, a mutual-aid request with unclear operational control, or a contractor requesting permission to fly outside the approved scope. The trainee must explain the decision, cite the governing policy, identify who must be notified, and state when the aircraft must remain grounded.

## 7.3 Practical Flight Evaluations

Practical flight evaluations should be standardized across instructors. Every practical evaluation must start with a preflight brief, risk review, aircraft inspection, Remote ID/registration/authorization check, launch/recovery zone assessment, communications check, and emergency procedure review. The evaluation must include normal flight, precision control, situational awareness, safe recovery, postflight documentation, and at least one abnormal or emergency procedure. A trainee fails the evaluation if the evaluator must intervene to prevent unsafe flight, if the aircraft contacts a person or obstacle, if the trainee violates airspace or operational limitations, if the trainee loses situational awareness and cannot recover, or if the trainee disregards stop-flight direction.

## 7.4 NIST-Based Proficiency Testing

The agency should use NIST-developed sUAS standard test methods to objectively measure practical proficiency and platform capability. NIST methods are not a substitute for FAA certification, legal authority, or agency endorsement. They are a standardized local evaluation tool. The agency should avoid implying that NIST has certified the pilot unless the certificate is issued under an authorized NIST-compatible program. Local certificates should state that the proficiency evaluation included aerial drone test methods developed by the National Institute of Standards and Technology.

| Proficiency Level | Agency Use | Recommended Local Threshold |
| --- | --- | --- |
| NIST Level 1 Basic Proficiency / Safety Checkride | Initial RPIC practical gate, remedial evaluation, annual skill check, or qualification for low-risk routine missions. | Complete the trial without collision, unsafe deviation, or evaluator intervention; achieve [local score] on target identification or task completion. |
| NIST Level 2 Open Test Lane Maneuvering | Baseline operational proficiency for routine agency missions, inspection, mapping, and platform checkout. | Complete position, traverse, orbit, inspect, and recon tasks within [local time/score] and without safety fault. |
| NIST Level 3 Open Test Lane with Payload Functionality | Payload readiness for zoom/thermal/search tasks, inspection, public works, DFR sensor operations, fire/EMS support, and law-enforcement observation. | Complete maneuvering and payload tasks with [local score] on visual/thermal targets and no uncontrolled exposure/focus/zoom deficiencies. |
| NIST Level 4 Obstructed Test Lane | Advanced operations near structures, trees, urban canyons, collapsed structures, fire/EMS scenes, or tactical overwatch environments. | Required only for assigned missions. Complete with no contact, no lost-situational-awareness event, and [local score]. |
| NIST Level 5 Confined Test Lane | Indoor, GPS-denied, confined-space, hazardous access, or special response missions when authorized by policy and aircraft capability. | Required only with aircraft and policy allowing confined operations. Complete with no contact or unsafe condition and [local score]. |
| Sensor Test Lane / Scenario Add-ons | Thermal, visual, identification, search, or payload capability evaluation. | Meet sensor-specific scoring threshold and document camera settings, lighting, distance, altitude, and payload configuration. |

The Training Officer should retain NIST-based scoresheets, platform details, date, location, weather/lighting, evaluator name, aircraft make/model, payload, camera settings when relevant, and any faults. The same method should be repeated for annual calibration so that performance trends can be compared over time.

## 7.5 Scenario-Based Evaluations

Scenario evaluations should be designed to reveal whether personnel can apply policy under realistic workload. A scenario may be tabletop, simulator-based, field-based without flight, or live-flight. Each scenario should have a scenario script, safety plan, objectives, injects, expected actions, disqualifying errors, evaluator notes, and after-action questions. Routine scenarios should include mapping/inspection, mission request, data handling, public interaction, weather/airspace changes, and contractor coordination. DFR scenarios should include dispatch timing and video-sharing decisions. Fire/EMS scenarios should include incident command, thermal interpretation, and air medical deconfliction. Law-enforcement scenarios should include legal pathway, evidence, and privacy limits. Mutual-aid scenarios should include operational control and airspace deconfliction.

## 7.6 Remediation and Retesting

A failed evaluation should produce a remedial training plan rather than an informal retest. The plan should identify the failed objective, root cause, remedial instruction, practice requirement, retest date, evaluator, and limitations pending retest. A trainee who fails due to a critical safety, legal, or integrity issue may be removed from the training program until reviewed by the UAS Program Manager. A trainee who fails due to correctable skill deficiency may continue under supervision after remedial training.

# 8. Initial Qualification Pathways

Initial qualification should be modular but sequenced. The suggested hours are a baseline for planning and may be adjusted based on agency mission risk, prior experience, aircraft complexity, and external course credit. The agency should not reduce evaluation standards even when it reduces classroom hours for experienced personnel.

| Track | Suggested Minimum Training | Required Evaluations Before Independent Use |
| --- | --- | --- |
| RPIC - Routine Agency Missions | FAA Part 107 preparation or certificate; 8 hours agency policy/legal/data; 8 hours aircraft systems and safety; 8 hours supervised flight; 4 hours mission planning and emergency procedures; NIST Level 1/2 practice. | Policy exam, FAA certificate verification, platform checkout, practical checkride, NIST Level 1 or Level 2 local threshold, records/data exercise. |
| Visual Observer | 4 hours agency policy and crew coordination; 2 hours airspace/scan technique; 2 hours field practice; annual refresher. | VO oral/practical evaluation, crew communications drill, emergency/lost-link drill. |
| Payload / Sensor Operator | 4 hours policy/data/privacy; 4 hours payload controls; 4 hours sensor interpretation and mission-specific data handling; supervised missions as needed. | Payload practical evaluation, data handling exercise, sensor-specific scenario. |
| Mission Requester / Project Lead | 2 hours mission request process; 2 hours privacy/data objective; 2 hours operational control and contractor coordination. | Mission request package exercise and oral review. |
| Launch/Recovery / Dock Technician | 8 hours platform and launch setup; 4 hours batteries and maintenance; 4 hours site control and emergency procedures; supervised launch/recovery events. | Launch practical, battery safety evaluation, emergency landing zone drill. |
| Airspace Coordinator / Airspace Monitor | 8 hours airspace/LAANC/NOTAM/TFR; 4 hours incident aviation and crewed-aircraft deconfliction; 4 hours mutual-aid/ICS coordination; annual exercise. | Airspace planning exercise, multiagency deconfliction tabletop, live communications drill. |
| Maintenance / Inventory Designee | 4 hours inventory, maintenance records, battery program, firmware/software, repair documentation, and return-to-service decision support. | Maintenance checklist and records exercise. |
| Instructor / Evaluator | Current qualification in role taught; 8 hours adult learning and safety; 8 hours evaluator scoring/calibration; teach-back; supervised evaluation. | Instructor teach-back, evaluator calibration, proctored evaluation observed by Chief Pilot/Training Officer. |

## 8.1 Prior Experience Credit

Prior experience may reduce training time but should not waive evaluation. The Training Officer may credit FAA certificates, FEMA/EMI ICS certificates, DRONERESPONDERS courses, LEDA courses, NIST-based proficiency records, vendor training, public-safety academy training, NWCG training, aviation experience, or prior agency UAS qualification when the records are current, relevant, and verified. The agency should record what was credited, what local training remains, and what evaluations are still required.

## 8.2 Supervised Operational Experience

Supervised operational experience bridges training and independent operations. A supervised trainee may participate in a real mission only when the RPIC, supervisor, and mission lead agree that training will not compromise safety or mission outcome. High-risk missions, law-enforcement evidentiary missions, DFR emergency launches, hazardous materials scenes, active fires, or crowded public events should not be used as trainee experience unless the trainee has already met prerequisite competencies and the supervisor can safely intervene.

# 9. Recurrent Training, Currency, and Continuing Education

Currency standards must be clear and auditable. Recurrent training should maintain the knowledge and skills required for the role actually performed. A person who flies only routine mapping missions should not be considered current for DFR or fireground operations unless the person has maintained the corresponding endorsement. A person whose FAA aeronautical knowledge recency, platform checkout, annual review, or mission endorsement expires must be placed in expired or restricted status until requalified.

| Requirement | Recommended Frequency | Applies To | Evidence |
| --- | --- | --- | --- |
| FAA aeronautical knowledge recency | Every 24 calendar months or current FAA requirement. | Part 107 RPICs. | FAA recurrent training certificate or applicable FAA record. |
| Agency policy and legal update | Annual and after major policy/legal change. | All operational UAS personnel. | Annual review acknowledgement and quiz. |
| Platform currency | At least three takeoffs and landings or one supervised proficiency flight every 90 days for each aircraft family, unless a stricter local standard applies. | RPICs and assigned operators. | Flight log and currency tracker. |
| NIST-based proficiency refresh | Annual for RPICs; every 6 months for DFR/BVLOS/high-risk mission endorsements unless modified by agency. | RPICs and assigned operators. | Scoresheet and evaluator record. |
| Emergency procedures drill | Annual minimum; semiannual for DFR/BVLOS and public-safety crews. | RPIC, VO, launch/recovery, airspace coordinators, DFR personnel. | Drill record and AAR. |
| DFR shift proficiency | [One] supervised or evaluated DFR shift/simulation every [90] days for DFR personnel. | DFR teleoperators, dispatch, RTIC/RTCC, launch/dock staff. | DFR shift evaluation. |
| Mutual-aid and airspace exercise | Annual; more frequently for agencies with regular multiagency response. | Public-safety UAS personnel and supervisors. | Exercise roster, deconfliction plan, and AAR. |
| Instructor/evaluator calibration | Annual and after major curriculum change. | Instructors, evaluators, proctors. | Calibration records and sample scored evaluations. |

## 9.1 Continuing Education

Continuing education may include FAA seminars, FAA Safety Team materials, DRONERESPONDERS training, LEDA training, FEMA/EMI courses, NWCG training, NFPA public-safety UAS training, vendor platform training, state academy training, legal updates, cybersecurity training, and regional exercises. The UAS Program Manager should approve continuing education credit only when the training supports a specific role or program need and the completion record is placed in the training file.

## 9.2 Requalification After Lapse or Suspension

A person whose qualification lapses due to currency, recurrent training, leave, suspension, or extended inactivity must complete requalification before operational assignment. Requalification should include policy/legal updates, review of recent incidents and lessons learned, platform checkout, emergency procedures, and any mission endorsement evaluation that has expired. A person suspended for safety, legal, integrity, or policy reasons may require a review board, remedial plan, and approval by [APPROVAL AUTHORITY] before returning to training or operations.

# 10. Instructor, Evaluator, and Proctor Standards

Instructor and evaluator quality controls protect the credibility of the entire training program. An instructor teaches and coaches. An evaluator measures performance against an objective standard. The same person may teach and evaluate in a small agency, but the agency should separate teaching from final checkrides when staffing allows. Evaluators must apply standards consistently and must document both passing and failing performance.

## 10.1 Instructor Qualifications

An agency UAS instructor should be currently qualified in the role being taught, have operational experience in the mission environment, understand the training curriculum, complete instructor orientation, demonstrate safe student supervision, and be approved by the UAS Program Manager. A flight instructor should have a demonstrated record of safe operations and should not instruct advanced operations unless personally qualified or paired with a qualified subject-matter instructor.

## 10.2 Evaluator and Proctor Qualifications

An evaluator should be trained to use the applicable rubric, document objective results, manage evaluation safety, identify disqualifying errors, explain remedial needs, and protect scoring integrity. A NIST-based proficiency proctor should know how to set up the test apparatus, brief the test, score consistently, record environmental and equipment variables, and avoid implying NIST certification when the agency is using the methods as a local evaluation tool.

| # | Done | Instructor/Evaluator Authorization Checklist Item | Evidence / Initials |
| --- | --- | --- | --- |
| 1 | [ ] | Currently qualified in the role or mission area to be taught or evaluated. | [Initials / Date] |
| 2 | [ ] | Completed agency instructor/evaluator orientation. | [Initials / Date] |
| 3 | [ ] | Completed adult learning, coaching, safety supervision, and documentation training. | [Initials / Date] |
| 4 | [ ] | Demonstrated a teach-back using approved lesson plan and safety controls. | [Initials / Date] |
| 5 | [ ] | Observed at least [number] training events and [number] evaluations before independent approval. | [Initials / Date] |
| 6 | [ ] | Calibrated scoring with Chief Pilot, Training Officer, or designated senior evaluator. | [Initials / Date] |
| 7 | [ ] | Understands remediation process, retest standards, and trainee appeal process. | [Initials / Date] |
| 8 | [ ] | Approved in writing by UAS Program Manager for specific courses, roles, and evaluation types. | [Initials / Date] |

# 11. Training Safety, Risk Management, and Student Supervision

Training must never create unnecessary risk to people, property, aircraft, responders, or public trust. The agency should select training sites that reduce exposure to nonparticipating people, vehicles, sensitive property, wildlife, crewed aircraft, and privacy-sensitive areas. Training flights must be authorized under the correct FAA authority and must follow the same preflight, airspace, Remote ID, registration, risk assessment, and emergency procedures as operational flights.

## 11.1 Student Supervision

A trainee may manipulate flight controls only under conditions permitted by FAA rules, agency policy, and the instructor ability to immediately intervene. When operating under Part 107, any non-certificated person manipulating the controls must be directly supervised by a certificated remote pilot who has the ability to immediately take direct control when required by Part 107. When operating under a public-aircraft COA or agency self-certification model, trainee supervision must follow the COA, agency policy, and documented training plan.

## 11.2 Training Risk Controls

The Training Officer should require a training safety plan for initial flight academies, NIST-based events, multi-aircraft training, public-safety scenarios, night training, fire/EMS training, tactical law-enforcement scenarios, DFR dock training, and any training in controlled airspace or near incident aviation. The plan should identify training objectives, training area boundaries, launch/recovery zones, emergency landing sites, instructor-to-student ratio, aircraft separation, weather minimums, airspace checks, public contact plan, and stop-training authority.

| # | Done | Training Event Safety Checklist Item | Evidence / Initials |
| --- | --- | --- | --- |
| 1 | [ ] | Training objective, roles, instructor/evaluator, and trainee status are documented. | [Initials / Date] |
| 2 | [ ] | FAA operating authority, airspace authorization, Remote ID, registration, and local permissions are verified. | [Initials / Date] |
| 3 | [ ] | Training site survey identifies obstacles, public access, traffic, privacy-sensitive areas, RF/magnetic interference, wildlife, and emergency landing zones. | [Initials / Date] |
| 4 | [ ] | Weather, lighting, wind, visibility, and aircraft limitations support the training objective. | [Initials / Date] |
| 5 | [ ] | Instructor-to-student ratio allows immediate safety intervention. | [Initials / Date] |
| 6 | [ ] | Sterile launch/recovery area, cones/signage, fire extinguisher, first aid kit, and battery safety controls are in place. | [Initials / Date] |
| 7 | [ ] | Emergency procedures for lost link, low battery, flyaway, crash, injury, fire, and crewed-aircraft conflict are briefed. | [Initials / Date] |
| 8 | [ ] | Training data storage, deletion, and privacy plan are established before flight. | [Initials / Date] |
| 9 | [ ] | After-action debrief and training records are completed before participants are released. | [Initials / Date] |

# 12. Training Records, Credentialing, and Quality Assurance

Training records should be treated as operational compliance records. If a mission is later reviewed by the FAA, a court, a public records officer, an insurer, a safety board, a prosecutor, or an internal auditor, the agency should be able to show who was qualified, under what standard, on what date, by whom, with what limitations, and with what recurrent requirements.

## 12.1 Credentialing

The agency should issue a credential record or roster entry for each qualified person. The credential should state role, platform family, mission endorsements, limitations, issue date, expiration date, approving authority, and required recurrent training. A credential should not imply FAA certification or NIST certification unless the person actually holds that credential. The credential is an agency operational authorization only.

## 12.2 Quality Assurance

The Training Officer should audit training records at least quarterly during program buildout and at least annually after the program stabilizes. The audit should compare the roster of active personnel to FAA certificates, recurrent training, platform currency, role endorsements, incident involvement, remedial training, and operational assignments. The UAS Program Manager should review any mission performed by a person whose qualification was expired or incomplete and should take corrective action before further assignment.

| QA Measure | Review Question | Corrective Action |
| --- | --- | --- |
| Completion audit | Are all required modules, exams, and evaluations recorded for each active role? | Restrict role until missing records are completed or verified. |
| Currency audit | Are FAA recency, 90-day flight currency, annual review, and mission endorsements current? | Expire or restrict credential until requalification. |
| Evaluation calibration | Do evaluators score similar performance similarly? | Conduct evaluator calibration and revise rubrics if necessary. |
| Incident learning | Did training address hazards observed in missions, incidents, near-misses, or complaints? | Update curriculum, add drill, issue safety bulletin, or require remedial training. |
| External training credit | Were external certificates verified and mapped to local objectives? | Remove unsupported credit and require local training/evaluation. |
| Legal/privacy updates | Were statutory, policy, prosecutor, FAA, or public-records changes integrated into training? | Issue update module and require acknowledgement or retest. |

# 13. Mutual Aid and Multiagency Airspace Deconfliction Training

Mutual-aid and multiagency UAS operations require training beyond ordinary flight skill. A qualified pilot from one agency can still create risk if operational control, airspace, data custody, communications, and incident command are unclear. Training must prepare personnel to integrate into unified command, identify the lead agency, determine who controls the flight, avoid self-dispatch, coordinate with crewed aircraft, and document data custody and incident reporting.

## 13.1 Mutual Aid Competencies

| Competency | Training Explanation | Evaluation Method |
| --- | --- | --- |
| Operational control | Trainee identifies who requested the aircraft, who funds or owns the operation, who tasks the aircraft, who can terminate the mission, and who receives data. | Operational-control worksheet using mutual-aid scenarios. |
| Credentialing and role recognition | Trainee can present qualifications, FAA certificate, platform qualification, mission endorsements, and limitations to the host agency or incident supervisor. | Credential packet review and check-in drill. |
| Incident communications | Trainee uses assigned command, tactical, airspace, phone, and data channels and avoids freelancing or unapproved video sharing. | Communications drill. |
| Airspace deconfliction | Trainee establishes altitude blocks, sectors, launch/recovery zones, emergency landing areas, and crewed-aircraft priority with the designated airspace coordinator. | Multi-UAS airspace tabletop. |
| Data custody and public records | Trainee identifies who owns, stores, retains, releases, and protects UAS data when more than one agency is involved. | Data-custody scenario. |
| Demobilization and after-action review | Trainee completes logs, transfers data, reports hazards, resolves equipment status, and participates in AAR/improvement planning. | Demobilization drill and AAR form. |

## 13.2 Airspace Deconfliction Training

Airspace deconfliction training must assume that multiple hazards can exist at the same time: agency drones, police drones, fire drones, public works drones, media helicopters, air ambulances, law-enforcement aircraft, firefighting aircraft, airport traffic, private aircraft, and unauthorized drones. The training should use a map-based exercise requiring trainees to assign altitude blocks, routes, sectors, launch/recovery zones, emergency landing sites, lost-link routes, airspace monitoring positions, and land-now triggers. Crewed aircraft must always have priority. If deconfliction cannot be assured, UAS operations must be delayed, modified, or stopped.

| # | Done | Multiagency Airspace Deconfliction Drill Checklist Item | Evidence / Initials |
| --- | --- | --- | --- |
| 1 | [ ] | Identify Incident Commander, Operations Section, Air Operations, UAS Airspace Coordinator, and each RPIC. | [Initials / Date] |
| 2 | [ ] | Identify all crewed aircraft, helipads, hospital pads, airports, private strips, TFRs, NOTAMs, and special use airspace relevant to the incident. | [Initials / Date] |
| 3 | [ ] | Assign launch/recovery zones, sterile areas, routes, sectors, altitude blocks, emergency landing zones, and lost-link/RTH defaults. | [Initials / Date] |
| 4 | [ ] | Confirm radio, phone, CAD, RTIC/RTCC, EOC, and dispatch communications between agencies. | [Initials / Date] |
| 5 | [ ] | Confirm land-now phraseology and who has authority to issue it. | [Initials / Date] |
| 6 | [ ] | Confirm data-sharing limits, live-feed recipients, evidence/public-records custody, and public information release channel. | [Initials / Date] |
| 7 | [ ] | Conduct a simulated crewed-aircraft intrusion and verify that all UAS crews respond correctly. | [Initials / Date] |
| 8 | [ ] | Conduct a simulated lost-link or flyaway and verify notification, search, and reporting procedures. | [Initials / Date] |
| 9 | [ ] | Conduct AAR and document improvement items, responsible persons, and due dates. | [Initials / Date] |

# 14. Optional Fire/EMS, SAR, and Emergency Management Training Module

This module should be adopted only by agencies that perform public safety, fire, EMS, search and rescue, emergency management, disaster response, hazardous access, or emergency DFR missions. Fire/EMS training must emphasize incident command, crewed aircraft priority, safety officer coordination, thermal-camera limitations, firefighter and patient safety, data minimization, and communication with command. This module does not create law-enforcement authority and should not be used to avoid law-enforcement legal restrictions.

## 14. Fire and EMS Mission Competencies

| Endorsement | Training Content | Evaluation |
| --- | --- | --- |
| Structure Fire Support | Incident command integration, fireground communications, thermal limitations, roof/exposure assessment, collapse zones, air medical and law-enforcement aviation awareness, public/involved-person privacy. | Structure fire tabletop and thermal target scenario. |
| Wildland/WUI Support | NWCG/NIMS awareness, Fire Traffic Area, TFR/airspace, aviation briefing, wildfire aviation priority, mapping perimeter concepts, hotspot documentation, UAS incursion hazards. | Wildfire airspace tabletop and map/thermal exercise. |
| Search and Rescue | Search patterns, missing-person behavior awareness, thermal/visual search limitations, evidence preservation, volunteer coordination, family/public interaction, night and terrain hazards. | SAR field scenario with target detection and communications evaluation. |
| EMS / Air Medical Support | Patient privacy, landing zone safety, medevac coordination, responder safety, crowd control, crash scene documentation limits, and handoff to incident command. | Air medical deconfliction drill and privacy scenario. |
| Hazmat / Dangerous Scene | Standoff assessment, sensor limitations, contamination/decontamination, explosive/flammable atmosphere awareness, safety officer approval, aircraft loss considerations. | Hazmat tabletop and standoff sensor scenario. |
| Disaster Damage Assessment | Rapid mapping/photo documentation, public works coordination, data management, resource prioritization, critical infrastructure coordination, public information release limits. | Damage assessment field or tabletop exercise. |

## 14.1 Fire/EMS DFR Awareness

Fire/EMS DFR awareness training should use dispatch simulations that include structure fire, smoke investigation, vehicle crash, water rescue, hazmat report, cardiac arrest with access issues, technical rescue, storm damage, and vegetation fire. The trainee must determine whether launch is authorized, whether the aircraft will help before arrival of units, what video should be shared, whether any EMS privacy or patient data issue exists, whether crewed aircraft or medevac operations are likely, and when the drone must hold, land, or relocate.

# 15. Optional Law-Enforcement UAS Training Module

This module should be adopted only by a law-enforcement agency or by a non-law-enforcement agency that provides UAS support to law enforcement under a counsel-reviewed agreement. Law-enforcement training must be jurisdiction-specific. A law-enforcement UAS operator must understand the legal pathway for each mission, the limits of UAS surveillance, evidence handling, public release, investigative data, data minimization, constitutional considerations, and community trust. Non-law-enforcement pilots should not make legal authorization decisions in the field unless that authority has been assigned and reviewed by counsel.

## 15.1 Law-Enforcement Mission Competencies

| Topic | Training Requirement | Evaluation |
| --- | --- | --- |
| Legal pathway and approval | Oregon and local law, warrant, consent, exigency, emergency assistance, search and rescue, declared emergency, crime/accident reconstruction, and training-data restrictions. | Counsel-approved written exam and oral board. |
| Call screening | Eligible call types, non-eligible call types, launch/recording triggers, dispatch documentation, termination criteria, and community transparency. | Call-review simulation. |
| Tactical overwatch | Command approval, officer/responder safety, perimeter support, de-escalation, video sharing, field communications, and avoiding interference with tactical operations. | Tactical tabletop and live/simulated scenario. |
| Evidence and records | Evidence capture, chain of custody, metadata, retention, public records, prosecutor coordination, legal hold, and segregation of training/incidental/evidentiary data. | Evidence/data transfer exercise. |
| Privacy, bias, and community trust | No general surveillance, no discriminatory or retaliatory use, data minimization, public-facing explanations, complaint routing, and auditability. | Scenario discussion and written acknowledgement. |
| Joint public-safety incidents | Interaction with fire/EMS/SAR, incident command, air medical, public works, mutual aid, and emergency management while preserving law-enforcement restrictions. | Unified-command scenario. |

## 15.2 Law-Enforcement Scenario Requirements

Before a law-enforcement UAS operator is authorized for independent law-enforcement duty, the operator should successfully complete scenarios involving a priority disturbance, missing person, vehicle crash, perimeter search, false alarm, public event near protected activity, domestic violence call, tactical containment, and a request for video outside the approved mission purpose. The evaluator should require the trainee to identify the legal pathway, decide whether recording is permitted, state who may view the feed, state how the data will be stored, and explain how the mission protects privacy and civil liberties.

# 16. Additional DFR, BVLOS, COA, and Remote Operations Training Module

This is an additional module to the regular UAS training program. It should be adopted only when the agency has, or is actively preparing for, a specific DFR operating model and the FAA authorization, COA provision, waiver, emergency authorization, dispatch process, RTIC/RTCC workflow, dock/remote launch procedures, community transparency, and data governance required for the operation. DFR training must not be treated as a routine RPIC endorsement unless the agency has approved the operational model.

DFR training must be treated as a specialized mission endorsement. DFR often involves rapid dispatch, remote launch, dock-based operations, live video to responders, operations near streets and people, night operations, and potential BVLOS or waiver/COA conditions. Training must therefore test not only aircraft control, but also dispatch judgment, public-safety communications, system monitoring, authorization compliance, privacy limits, and termination decision making.

## 16.1 DFR Prerequisites

| Prerequisite | Required Before DFR Module |
| --- | --- |
| Core UAS training | Completion of T-01 through T-10, or documented equivalent credit approved by the UAS Program Manager. |
| FAA authority | Training on the actual Part 107 waiver, public-aircraft COA, BVLOS authorization, SGI/ECOA process, or other operating authority used by the agency. |
| Platform qualification | Platform checkout for the aircraft, dock, payload, DFR software, controller, C2 link, command station, or remote operations system used by the agency. |
| Data and privacy training | Completion of DFR-specific live video, recording, retention, access control, redaction, public records, and prohibited-use training. |
| Dispatch/RTIC/RTCC workflow | Completion of call screening, mission approval, launch decision, response monitoring, communications, escalation, and termination training. |
| Supervisor approval | Written authorization from the UAS Program Manager, Training Officer, and relevant operational supervisor before supervised DFR shifts. |

## 16.2 FAA Authorization and COA/Waiver Training

Personnel assigned to DFR, BVLOS, public-aircraft COA, or waiver operations must be trained on the actual authorization under which the agency operates. The training must identify who may act as RPIC, who may operate remotely, what airspace is authorized, what altitudes are allowed, whether operations over people or moving vehicles are allowed, what visual observer or airspace monitoring configuration is required, what detect-and-avoid or shielded-operation mitigation applies, what weather minimums apply, what reporting is required, and when the mission must terminate. The trainee should be tested using the agency authorization rather than a generic summary.

## 16.3 DFR Dispatch and Launch Decision Training

The DFR launch decision should be trained as both a public-service decision and an aviation decision. Dispatch or RTIC/RTCC staff may identify eligible calls, but the launch must also satisfy FAA authorization, policy, weather, airspace, aircraft status, staffing, privacy, and mission-purpose criteria. The training program should use dispatch simulations with real or anonymized call types, including calls that should not launch. A trainee must be able to explain why a drone should be launched, why it should not be launched, when recording may begin, who may view the feed, and when the flight must be terminated.

## 16.4 Remote Operations and Dock Training

Remote operations training must include dock preflight, aircraft health checks, camera checks, command-and-control link status, telemetry quality, Remote ID status, weather at the dock and mission area, obstruction maps, geofence status, lost-link settings, return-to-home altitude, emergency landing points, alternate launch/recovery options, and crew communications. A remote operator or RPIC must demonstrate the ability to pause or cancel launch when the dock camera, weather sensor, telemetry, or local observer indicates a hazard.

## 16.5 BVLOS and Shielded Operations Training

BVLOS training must be authorization-specific. If the agency relies on a shielded-operations model, training must define the shielding objects, lateral boundaries, altitude limits, crewed-aircraft deconfliction, detect-and-avoid limitations, route planning, command-and-control monitoring, emergency descent/land procedures, and documentation requirements. If the agency relies on a detect-and-avoid system, training must include system capabilities, limitations, failure modes, false alerts, non-cooperative aircraft, and crew response. The agency must not train personnel to conduct routine BVLOS unless a waiver, COA provision, or other FAA authorization permits the operation.

## 16.6 DFR Test Battery

DFR candidates must complete the regular training evaluations before the DFR test battery. The DFR test battery adds authorization-specific knowledge, DFR workflow judgment, remote system proficiency, dispatch/RTIC/RTCC integration, privacy/data controls, and emergency termination drills. The test battery should be repeated during recurrent qualification and after material changes to the FAA authorization, dispatch criteria, software, dock hardware, aircraft, payload, or data workflow.

| DFR Test | Purpose | Minimum Passing Standard |
| --- | --- | --- |
| DFR Written Exam | Tests FAA authorization conditions, DFR policy, call eligibility, live-video limits, data handling, Remote ID, airspace, weather, over-people/moving-vehicle limits, and mission termination. | [90 percent] and no missed critical FAA authorization, privacy, or land-now item. |
| DFR Oral Board | Tests judgment under pressure, including no-launch calls, conflicting supervisor direction, crewed-aircraft conflicts, loss of video, public complaints, and non-mission use of video. | Evaluator consensus that the trainee makes safe, lawful, explainable decisions. |
| Dispatch Simulation | Tests call screening, launch timing, CAD/RTIC/RTCC logging, handoff to field units, escalation, and cancellation. | Correctly approve/deny at least [local number] call types with no critical prohibited launch. |
| Dock/Remote Preflight Practical | Tests dock camera review, weather, system health, aircraft status, battery, firmware, Remote ID, C2 link, geofence, RTH altitude, and emergency site selection. | Complete checklist without missed safety-critical item. |
| Remote Flight / Simulator Evaluation | Tests DFR navigation, remote situational awareness, camera use, route selection, no-fly areas, latency management, and recovery. | No unsafe deviation, no unapproved data use, no evaluator intervention. |
| Crewed-Aircraft Intrusion Drill | Tests detection, communication, land-now procedures, and coordination with airspace monitor/dispatch. | Immediate correct response and documented notification. |
| Lost Link / Loss of Video Drill | Tests lost-link procedure, system monitoring, dispatch notification, field safety, search/recovery, and report initiation. | Correct emergency procedure and documentation. |
| Privacy/Data Release Scenario | Tests live-feed recipient limits, recording decision, retention category, public records routing, evidence handling, and prohibited secondary use. | No unauthorized disclosure or retention error. |
| Final DFR Scenario | End-to-end realistic DFR mission from dispatch to launch, response, data handling, debrief, and log closeout. | Passing performance with no critical safety, legal, privacy, authorization, or operational-control error. |

## 16.7 DFR Remote Operations Qualification Checklist

| # | Done | Checklist Item | Evidence / Initials |
| --- | --- | --- | --- |
| 1 | [ ] | Trainee completed agency DFR policy, FAA authorization, privacy, records, and data-sharing modules. | [Initials / Date] |
| 2 | [ ] | Trainee can identify eligible and prohibited DFR call types under [AGENCY NAME] policy. | [Initials / Date] |
| 3 | [ ] | Trainee can complete dock/remote preflight and explain each system-health indicator. | [Initials / Date] |
| 4 | [ ] | Trainee can verify weather, airspace, TFR/NOTAM status, Remote ID, and authorization conditions before launch. | [Initials / Date] |
| 5 | [ ] | Trainee can communicate with dispatch, RTIC/RTCC, RPIC, VO/airspace monitor, and field responders using approved channels. | [Initials / Date] |
| 6 | [ ] | Trainee can operate live video without unnecessary collection or unauthorized disclosure. | [Initials / Date] |
| 7 | [ ] | Trainee can execute lost-link, loss-of-video, battery, geofence, crewed-aircraft, and land-now procedures. | [Initials / Date] |
| 8 | [ ] | Trainee completed [number] supervised DFR simulations and [number] supervised operational shifts or agency-approved equivalents. | [Initials / Date] |
| 9 | [ ] | Evaluator documented final DFR scenario as passing with no critical safety, privacy, FAA authorization, or operational-control error. | [Initials / Date] |
| 10 | [ ] | UAS Program Manager approved DFR endorsement in writing with any limitations. | [Initials / Date] |

## 16.8 DFR Recurrent and Remedial Training

DFR personnel should complete authorization-specific recurrent training at least annually and whenever the agency changes the FAA authorization, DFR software, dock hardware, operating area, dispatch criteria, live-feed access, data retention, or aircraft. DFR personnel should also complete at least one supervised or evaluated DFR shift or simulation every [90] days. A DFR crewmember who fails a critical DFR scenario, makes an unauthorized launch, improperly shares video, violates the FAA authorization, or fails to respond to a crewed-aircraft conflict should be placed in restricted status until remedial training and re-evaluation are complete.

# 17. Implementation Plan

The agency may implement this training program in phases. The goal is to avoid launching a broad UAS, DFR, or public-safety program before instructors, evaluators, training records, policy modules, safety procedures, and role qualifications are ready. The UAS Program Manager should tailor the timeline to program maturity and mission risk.

| Phase | Purpose | Expected Outputs |
| --- | --- | --- |
| Phase 1 - Program Setup | Customize the training program, identify legal modules, approve instructors/evaluators, create question banks, adopt forms, and build records system. | Approved training manual, instructor roster, evaluator roster, training calendar, LMS/record system, and baseline curriculum. |
| Phase 2 - Core UAS Qualification | Train RPICs, VOs, payload operators, mission requesters, and maintenance/inventory personnel for routine missions. | Qualified core crews, platform checkouts, NIST-based baseline scores, training records, and routine mission authorization. |
| Phase 3 - Agency Mission Endorsements | Train personnel for mapping, inspection, emergency management, public works, natural resources, and contractor-supported missions. | Mission-specific endorsements, project scenario evaluations, data workflow training, and mission request process. |
| Phase 4 - Public Safety and Incident Integration | Train fire/EMS/SAR/emergency management personnel, incident command users, and mutual-aid partners if adopted. | Public-safety endorsements, ICS/NIMS records, fire/EMS scenarios, mutual-aid exercise, and airspace deconfliction plan. |
| Phase 5 - Law Enforcement Support | Implement counsel-approved law-enforcement training, legal pathways, evidence/data procedures, and scenarios if adopted. | Authorized law-enforcement operators or support personnel, legal exam records, evidence exercises, and community/data training records. |
| Phase 6 - DFR/BVLOS/COA Advanced Operations | Train under the actual FAA authorization, DFR software, dock workflows, airspace monitoring, emergency procedures, and live/simulated shifts if adopted. | DFR/BVLOS endorsements, supervised shift records, authorization-specific exams, DFR tabletop/live drills, and readiness review. |
| Phase 7 - Sustainment and QA | Maintain recurrent training, annual evaluations, instructor calibration, corrective actions, and program review. | Annual training report, updated curriculum, currency audit, AAR/improvement plan, and revised training calendar. |

# 18. Appendices and Forms

The appendices may be used as paper forms, electronic forms, LMS records, spreadsheet trackers, or fields in a records-management system. Forms should remain simple enough for field use and complete enough to prove training, qualification, currency, and corrective action.

## Appendix A: Training Program Customization Matrix

| Section | Decision Needed | Agency Selection / Notes |
| --- | --- | --- |
| Cover | Agency name, version, effective date, training custodian. | [Insert] |
| 1. Authority | Part 107, public-aircraft COA, ECOA/SGI, DFR waiver, contractor, or combination. | [Insert] |
| 3 Roles | Role holders and alternates for UAS Program Manager, Training Officer, Chief Pilot, instructors, evaluators, records, IT/security, counsel. | [Insert] |
| 4 Testing Methods | Which NIST levels, written tests, oral boards, and scenarios are required for each role. | [Insert] |
| 5 Role Tracks | Which roles and endorsements the agency will use. | [Insert] |
| 6 Curriculum | Local modules, external training credit, and required Oregon/local content. | [Insert] |
| 7 Passing Standards | Written exam scores, NIST thresholds, scenario passing standards, critical-fail items. | [Insert] |
| 14 Fire/EMS/SAR | Adopt, delete, or mark not adopted. | [Insert] |
| 15 Law Enforcement | Adopt, delete, or mark not adopted; insert counsel-approved legal module. | [Insert] |
| 16 DFR/BVLOS | FAA authorization, DFR software, dispatch criteria, RTIC/RTCC workflow, dock locations, airspace monitoring plan. | [Insert] |
| Records | Storage location, retention period, public records handling, access controls. | [Insert] |

## Appendix B: Written Exam Blueprint

| Exam | Question Count | Critical-Fail Topics | Passing Standard |
| --- | --- | --- | --- |
| Core UAS Policy Exam | [40] | FAA authority, stop-flight authority, prohibited uses, Oregon registration/reporting, privacy/data release. | [80 percent] and no missed critical item. |
| RPIC Practical Oral Board | [10 scenarios] | Airspace, weather, emergency, public interaction, operational control, contractor coordination. | Evaluator pass/fail. |
| VO Exam | [20] | Crewed-aircraft callout, scan duties, loss of visual contact, emergency communication. | [80 percent]. |
| Payload/Data Exam | [25] | Data minimization, public records, evidence, EMS/privacy, unauthorized sharing. | [80 percent] and no critical data-release error. |
| Fire/EMS/SAR Module Exam | [30] | Incident command, air medical, wildfire aviation, thermal limitations, EMS privacy. | [80 percent]. |
| Law-Enforcement Module Exam | [40] | Legal pathway, warrant/consent/exigency, evidence, retention, privacy/civil liberties. | [90 percent] or counsel-approved standard. |
| DFR Supplemental Exam | [50] | FAA authorization, call eligibility, remote launch, live video, lost link, crewed-aircraft intrusion, land-now, privacy/data handling. | [90 percent] and no critical safety/legal/authorization error. |

## Appendix C: Practical Flight Evaluation Rubric

| Task Area | Satisfactory Performance | Unsatisfactory / Critical Failure |
| --- | --- | --- |
| Preflight and setup | Completes aircraft inspection, battery check, Remote ID verification, registration/authorization check, controller setup, payload configuration, launch site survey, and crew briefing. | Misses safety-critical inspection item, launches with defective aircraft, fails to verify authorization, or ignores hazards. |
| Basic flight control | Maintains positive control, stable hover, precise lateral/vertical movement, heading control, safe distance, and situational awareness. | Contacts object, loses orientation, violates boundary, or requires evaluator intervention. |
| Mission maneuvering | Completes orbit, traverse, inspection, recon, camera/sensor use, and safe altitude/route selection. | Fails mission task due to unsafe handling, uncontrolled camera use, or loss of situational awareness. |
| Emergency procedures | Executes lost link, low battery, crewed-aircraft, GNSS anomaly, and emergency landing procedures using approved callouts. | Fails to land/hold when required, ignores crewed aircraft, or cannot execute emergency procedure. |
| Postflight and records | Completes logs, maintenance discrepancy note, data intake, battery status, incident reports if needed, and debrief. | Fails to report damage, mishap, data issue, or training deficiency. |

## Appendix D: Scenario Evaluation Form

| Field | Entry |
| --- | --- |
| Scenario Title | [Insert] |
| Scenario Type | [Routine / Inspection / Mapping / Contractor / Mutual Aid / Fire/EMS / Law Enforcement / DFR] |
| Evaluator | [Insert] |
| Trainee | [Insert] |
| Aircraft / Payload / Software | [Insert] |
| Learning Objectives | [Insert] |
| Injects Used | [Weather change / airspace conflict / public complaint / data request / lost link / crewed aircraft / legal pathway issue] |
| Critical-Fail Items | [Insert] |
| Result | [Pass / Remedial / Fail] |
| Remedial Plan | [Insert] |
| Evaluator Signature | [Insert] |

## Appendix E: DFR Scenario Evaluation Form

| DFR Evaluation Element | Pass Criteria | Result / Notes |
| --- | --- | --- |
| Call screening | Candidate correctly identifies whether the call is eligible, whether recording is permitted, and whether launch supports an approved public purpose. | [Insert] |
| Authorization check | Candidate confirms FAA authorization, airspace, weather, Remote ID, dock status, staffing, and mission limitations before launch. | [Insert] |
| Remote launch | Candidate completes remote/dock preflight and does not launch with unresolved dock, camera, telemetry, weather, or airspace hazards. | [Insert] |
| Flight monitoring | Candidate maintains situational awareness, responds to latency, protects route boundaries, and communicates with dispatch/RTIC/RTCC/field units. | [Insert] |
| Crewed aircraft | Candidate immediately executes land-now/hold/deconfliction procedure and documents the event. | [Insert] |
| Privacy/data handling | Candidate limits live-feed access, avoids unnecessary collection, follows recording/retention rules, and routes public records/evidence issues correctly. | [Insert] |
| Emergency termination | Candidate executes lost-link/loss-of-video/battery/geofence procedures and notifies required personnel. | [Insert] |
| Closeout | Candidate completes log, data intake, maintenance notes, incident report if needed, and AAR. | [Insert] |

## Appendix F: Training Record and Credential Roster Fields

| Record Field | Required Entry |
| --- | --- |
| Name / Employee ID | [Insert] |
| Role Track | [RPIC / VO / Payload / Mission Lead / Maintenance / Airspace / DFR / Instructor / Evaluator] |
| FAA Certificate / Recency | [Certificate number / recurrent date / expiration tracker] |
| Platform Family | [Aircraft model / dock / payload / software] |
| Core Modules Completed | [List] |
| Endorsements | [Night / thermal / mapping / fire / law enforcement / DFR / BVLOS / COA / instructor] |
| Practical Evaluations | [Date / evaluator / result] |
| NIST Scores | [Level / score / date / aircraft / evaluator] |
| Scenario Evaluations | [Scenario / date / evaluator / result] |
| Limitations | [Daylight only / supervised only / no DFR / no law enforcement / no COA / other] |
| Currency Due Dates | [FAA / platform / scenario / DFR / legal / instructor] |
| Remedial Training | [Plan / status / retest date] |
| Approval | [UAS Program Manager / date] |

## Appendix G: Sample Training Calendar

| Period | Training Activity | Output |
| --- | --- | --- |
| Month 1 | Program orientation, legal/privacy module, Part 107 verification, training records setup. | Candidate roster and baseline records. |
| Month 2 | Aircraft systems, preflight, maintenance, and basic controlled flight training. | Platform checkouts and supervised flight logs. |
| Month 3 | NIST Level 1/2 proficiency events, mission planning, risk assessment, and core practical evaluations. | Qualified routine RPICs and VOs. |
| Month 4 | Mission endorsements for inspection, mapping, public works, or natural resources. | Mission endorsement records. |
| Month 5 | Optional public safety, fire/EMS, SAR, or law-enforcement modules if adopted. | Optional endorsement records. |
| Month 6 | Additional DFR module if adopted: authorization exam, dispatch simulation, dock/remote operations, and supervised shifts. | DFR readiness review and restricted/supervised endorsements. |
| Ongoing | Recurrent training, annual review, NIST refresh, emergency drills, instructor calibration, and AAR integration. | Current roster and annual training report. |

# 20. Remote Pilot Certification Standards Integration Addendum

This addendum integrates the May 2026 Certification Standards for Remote Pilot Operations into the generic Oregon public agency UAS Training and Proficiency Program. Agencies may rename this addendum as an agency carding standard, remote pilot qualification standard, or mission endorsement standard.

The certification standards document reinforces the approach already used in this template: an FAA Remote Pilot Certificate is the federal baseline, but agency operations require additional training, task-based proficiency, risk management, mission-specific endorsements, evaluator oversight, and complete training records. The added language below should be treated as controlling guidance for the training and evaluation portions of this template once adopted by the agency.

## 20.1 Integration Map

| Template Area | Required Integration |
| --- | --- |
| Training governance | Add Lead Instructor, Pilot Evaluator, and Final Evaluator as named training roles. The UAS Program Manager may approve smaller-agency equivalents. |
| Role-based qualification tracks | Add a carding/endorsement model that distinguishes Basic Remote Pilot, aircraft-family qualification, Type 3/Type 4 or complex aircraft qualification, night endorsement, DFR/remote operations endorsement, ELOS/BVLOS endorsement, and non-standard operating procedure endorsement. |
| Evaluation methodology | Require a four-phase methodology: introduction, knowledge competency, flight exercise, and debrief. Demonstration flights, training flights, and evaluation flights should be documented separately. |
| Practical testing | Use the Master Task List as the required practical evaluation backbone. The practical test should not be reduced below the minimum task plan for the aircraft or mission. |
| Records | Add pilot card, current-year designation, endorsement, evaluator approval, and retraining records to the training file. |
| DFR/BVLOS addenda | Use the same task-based method for DFR, ELOS/BVLOS, and remote operations even where the base certification standards mark those areas as under development. |

## 20.2 Special Emphasis Areas

Instructors, evaluators, and final evaluators shall place continuing emphasis on the following areas during academic training, simulator events, aircraft training, scenario evaluations, practical tests, and recurrent evaluations. These areas apply even when not restated in each task sheet.

Cooperation, professionalism, and positive attitude toward aviation safety.

Risk assessment and risk management.

Aeronautical decision making.

Crew resource management.

Checklist usage and division of attention.

Ability to stop, delay, or modify an unsafe operation.

## 20.3 Agency Carding and Mission Endorsement Model

| Qualification / Endorsement | Minimum Basis | Authorizes |
| --- | --- | --- |
| Basic Remote Pilot | FAA Remote Pilot Certificate when operating under Part 107; agency policy/legal/data training; aircraft systems training; completion of Basic Remote Pilot task plan; satisfactory final evaluation. | Routine agency UAS missions within listed limitations. |
| Aircraft-Family Qualification | Basic Remote Pilot plus platform checkout for each aircraft family or payload class. | Operation of the listed aircraft family or payload. |
| Type 3/Type 4 or Complex Aircraft Endorsement | Task-based evaluation on complex or heavier agency aircraft using the Type 3/Type 4 task plan or agency equivalent. | Larger aircraft, higher-risk payloads, or aircraft that require additional launch, recovery, maintenance, or crew coordination. |
| Night Endorsement | Night module, lighting, visibility, VO, emergency procedures, and practical demonstration under agency procedures. | Night missions authorized by FAA rules, COA, waiver, or agency policy. |
| DFR / Remote Operations Endorsement | Core RPIC qualification plus DFR/remote operations addendum, dispatch simulation, remote/dock practical, live video/data module, and scenario evaluation. | Drone as First Responder, dock-based operations, remote launch, or RTIC/RTCC-supported operations. |
| ELOS/BVLOS Endorsement | Authorization-specific training, airspace monitor or VO plan, detect-and-avoid/shielding procedures, lost-link route, emergency land-now drill, and final evaluation. | Extended line of sight or beyond visual line of sight operations only within FAA authorization. |
| Non-Standard Operating Procedure Endorsement | Written approval, task-based training, operational risk review, practical demonstration, and evaluator sign-off. | Any mission, payload, aircraft, or procedure not otherwise covered by routine qualification. |

## 20.4 Evaluator Roles

| Role | Function | Minimum Standard |
| --- | --- | --- |
| Lead Instructor | Delivers standardized instruction and ensures course objectives are met. | Must be qualified in the subject taught and designated for the course offering. |
| UAS Pilot Evaluator | Conducts training flights, proficiency checks, standards checks, retraining, and assigned checkrides. Documents deviations and recommends additional training. | Current FAA Remote Pilot; current agency qualification for the aircraft, payload, or mission being evaluated; subject-matter expertise; evaluator training; approval by UAS Program Manager or designee. |
| UAS Final Evaluator | Provides oversight and quality assurance of pilot standards, coordinates final checkrides, approves or issues agency pilot cards and endorsements, and audits training records. | Current FAA Remote Pilot; current agency pilot card; subject-matter expert in UAS policy and operations; designation by UAS Program Manager or agency aviation leadership. |

## 20.5 Pilot Evaluator Minimum Experience Guide

The adopting agency should set its own final thresholds based on aircraft complexity and mission risk. The following baseline may be used as a starting point for local approval.

| Evaluator Area | Minimum Experience Guide |
| --- | --- |
| Initial agency carding / basic remote pilot | 20 hours total time as RPIC |
| ELOS, BVLOS, or night evaluation | 10 hours in the specific mission type or agency-approved equivalent |
| Aerial ignition or high-risk payload evaluation | 10 hours in the specific mission type plus payload-specific training |
| Type 3 or complex aircraft evaluation | 15 hours in that aircraft class or agency-approved equivalent |
| Type 4 or smaller aircraft evaluation | 10 hours in that aircraft class or agency-approved equivalent |

## 20.6 Training and Evaluation Phases

| Phase | Required Content |
| --- | --- |
| Phase 1 - Introduction | Evaluator confirms the purpose of the training or evaluation, explains standards, reviews the pilot training record, and verifies prerequisites before flight. |
| Phase 2 - Knowledge Competency | Evaluator conducts an academic knowledge check using the relevant aircraft, mission, policy, airspace, safety, data, and emergency procedure topics. |
| Phase 3 - Flight Exercise | Evaluator conducts demonstration, training, or evaluation flights using the appropriate task plan. Briefing, preflight, flight tasks, postflight inspection, and flight-use reporting are included. |
| Phase 4 - Debrief | Evaluator reviews tasks not performed to standard, strengths, weaknesses, pass/fail result, restrictions, improvement recommendations, and required documentation. |

## 20.7 Evaluation Ratings

| Rating | Meaning | Use |
| --- | --- | --- |
| DM | Demonstrated | Used for introduced tasks. Proficiency is not required during the demonstration phase. |
| 4 | Near optimum standard | Recognizes errors immediately, corrects properly, rarely repeats errors, and safety is never in doubt. |
| 3 | Acceptable / developing standard | Recognizes errors quickly, corrects properly, infrequently repeats errors, and safety is never in doubt. |
| 2 | Marginal training performance | Performs inconsistently, is slow to recognize or correct errors, but safety is not in doubt. |
| 1 | Unsatisfactory training performance | Lacks sufficient knowledge, skill, or ability; safety or outcome may be in doubt. |
| S / U | Final evaluation only | Final checkrides shall be marked only satisfactory or unsatisfactory. A task equivalent to a 1 is unsatisfactory. |

## 20.8 Basic Remote Pilot Evaluation Plan

The following plan should be used as the minimum practical task plan for initial agency qualification. Agencies may add tasks but should not reduce the minimum plan for an agency RPIC.

| Task # | Task Title | Training Mode | Evaluation Mode |
| --- | --- | --- | --- |
| BR 01 | Complete a Mission Aviation Safety Plan | Classroom | Classroom |
| BR 10 | Create a UAS Mission Plan | Classroom | Classroom |
| BR 15 | Participate in a Crew/Mission Brief | Aircraft / Classroom | Aircraft / Classroom |
| BR 20 | Perform System Ground Inspection | Aircraft / Classroom | Aircraft |
| BR 25 | Perform Radio Communication Procedures | Aircraft / Classroom | Aircraft / Classroom |
| BR 30 | Perform System Preflight | Aircraft / Classroom | Aircraft |
| BR 35 | Perform Aircraft Takeoff | Aircraft / Classroom | Aircraft |
| BR 40 | Perform Flight in Manual Control | Aircraft / Classroom | Aircraft |
| BR 45 | Perform Flight Utilizing Automatic Flight Mode | Aircraft / Classroom | Aircraft |
| BR 50 | Operate Optical Sensor Payload | Aircraft / Classroom | Aircraft |
| BR 55 | Perform Battery Management Procedures | Aircraft / Classroom | Aircraft |
| BR 60 | Perform Basic Flight Maneuvers | Aircraft / Classroom | Aircraft |
| BR 65 | Perform Flight at Night | Aircraft / Classroom | Aircraft |
| BR 70 | Respond to Emergencies | Aircraft / Classroom | Aircraft / Classroom |
| BR 75 | Perform Aircraft Landing | Aircraft / Classroom | Aircraft |
| BR 80 | Perform After Landing Checks | Aircraft / Classroom | Aircraft |
| BR 85 | Participate in a Crew After Action Review | Aircraft / Classroom | Aircraft / Classroom |

## 20.9 Complex Aircraft or Type 3/Type 4 Endorsement Task Plan

This task plan may be used for larger aircraft, higher-risk platforms, complex payloads, or agency-designated aircraft requiring an added endorsement. Planning tasks may be required separately through the mission planning process.

| Task # | Task Title | Training Mode | Evaluation Mode |
| --- | --- | --- | --- |
| BR 20 | Perform System Ground Inspection | Aircraft / Classroom | Aircraft |
| BR 30 | Perform System Preflight | Aircraft / Classroom | Aircraft |
| BR 35 | Perform Aircraft Takeoff | Aircraft / Classroom | Aircraft |
| BR 40 | Perform Flight in Manual Control | Aircraft / Classroom | Aircraft |
| BR 45 | Perform Flight Utilizing Automatic Flight Mode | Aircraft / Classroom | Aircraft |
| BR 50 | Operate Optical Sensor Payload | Aircraft / Classroom | Aircraft |
| BR 55 | Perform Battery Management Procedures | Aircraft / Classroom | Aircraft |
| BR 60 | Perform Basic Flight Maneuvers | Aircraft / Classroom | Aircraft |
| BR 70 | Respond to Emergencies | Aircraft / Classroom | Aircraft / Classroom |
| BR 75 | Perform Aircraft Landing | Aircraft / Classroom | Aircraft |
| BR 80 | Perform After Landing Checks | Aircraft / Classroom | Aircraft |
| BR 85 | Participate in a Crew After Action Review | Aircraft / Classroom | Aircraft / Classroom |

## 20.10 Pilot Evaluator and Final Evaluator Task Plan

| Task # | Task Title | Training Mode | Evaluation Mode |
| --- | --- | --- | --- |
| PE 165 | Develop Evaluation Plan | Classroom | Classroom |
| PE 170 | Instruct Academic Knowledge Subjects | Aircraft / Classroom | Aircraft / Classroom |
| PE 175 | Evaluate Academic Knowledge Subjects | Aircraft / Classroom | Aircraft / Classroom |
| PE 180 | Train Remote Pilot Tasks | Aircraft / Classroom | Aircraft / Classroom |
| PE 185 | Evaluate Remote Pilot Tasks | Aircraft / Classroom | Aircraft / Classroom |
| PE 190 | Administer Final Evaluation (Check Ride) | Aircraft / Classroom | Aircraft / Classroom |

## 20.11 Pilot Training and Qualification Records

The training file should be complete enough to show the pilot was qualified, current, and properly endorsed at the time of any mission. The agency should maintain a current roster or carding record for each pilot and evaluator.

| Record Category | Required Contents |
| --- | --- |
| Pilot identity and FAA record | Name, agency, contact information, FAA certificate number, initial Part 107 date, recurrent training date. |
| Flight time | Total agency UAS time, last 12 months flight time, platform time, evaluator time if applicable. |
| Aircraft qualifications | Aircraft make/model or aircraft family, payload qualification, Type 3/Type 4 or complex aircraft endorsement. |
| Mission endorsements | Night, DFR, ELOS/BVLOS, thermal, mapping, public safety, law enforcement, fire/EMS, SAR, mutual aid, NSOP, or other mission authorization. |
| Pilot designations | Lead Instructor, UAS Pilot Evaluator, UAS Final Evaluator, trainee status, restrictions, limitations, or suspension. |
| Current-year training | FAA Part 107 recurrency, aircraft refresher, UAS operations refresher, CRM, basic aviation safety, mishap review, legal/privacy updates. |
| Evaluation records | Written exams, oral boards, NIST-based proficiency sheets, task evaluations, final checkride forms, debrief notes, remediation, retests. |

## 20.12 Recommended Operations Manual Cross-References

The operations and policy manual should cross-reference this addendum wherever it describes pilot authority, mission approval, training, complex aircraft, night, DFR/BVLOS, mutual aid, and training records. The addendum does not replace agency operating procedures; it makes those procedures trainable and testable.

Source Integration Note

This document was updated to incorporate the May 2026 Certification Standards for Remote Pilot Operations model, including its task-based carding concept, special emphasis areas, evaluator roles, four-phase evaluation methodology, Master Task List, Basic Remote Pilot task plan, Type 3/Type 4 aircraft task plan, pilot evaluator task plan, and current-year training and qualification record concepts.

Part III. Detailed Curriculum, Test Banks, and Practical Standards

Instructor/administrator curriculum content, written question banks, answer keys, practical standards, DFR/BVLOS addenda, RPCS task plans, and advanced evaluation plans.

# Administrative Handling Note

This part contains instructor-controlled questions, answer keys, evaluator notes, and scenario prompts. Remove or separate the answer keys and evaluator-only material before student distribution.

# Part I. Core UAS Training Curriculum

### T-01 Program Orientation, Governance, and Approved Uses

| Item | Content |
| --- | --- |
| Duration | 2 hours |
| Audience | All UAS personnel, mission requesters, supervisors, contractors when required by contract. |
| Purpose | Establish the agency UAS program structure, the relationship between the training program and the operations manual, and the limits on agency UAS use. |
| Evaluation | Orientation quiz and signed policy acknowledgment. |

Learning objectives:

Describe the agency UAS program, chain of authority, and role-based qualification model.

Distinguish personal drone use, contractor drone use, mutual-aid drone use, and agency-controlled UAS operations.

Explain RPIC stop-flight authority and the role of the UAS Program Manager, Training Officer, Chief Pilot, instructor, evaluator, records officer, IT/security lead, risk manager, and counsel.

Identify approved mission types, prohibited uses, required approvals, and when a mission must be referred to legal, IT, records, or risk management.

Training content:

Program purpose and public trust principles.

Roles and responsibilities from the operations and policy manual.

Operational control, mission approval, and stop-flight authority.

Agency qualification status categories: candidate, trainee, supervised operational, qualified, restricted, suspended, expired.

How to use the mission request, PASP, risk assessment, checklists, flight logs, maintenance logs, and incident forms.

Learning activities:

Policy walk-through using three mission examples: public works inspection, natural-resources mapping, and public-safety support.

Short tabletop: supervisor asks for a last-minute flight that has not been approved.

### T-02 FAA Operating Authority, Part 107, COA, SGI, Remote ID, and Registration

| Item | Content |
| --- | --- |
| Duration | 4 hours core plus FAA Part 107 preparation as needed |
| Audience | RPICs, airspace coordinators, supervisors, DFR candidates, mission approvers; awareness for all crewmembers. |
| Purpose | Ensure trainees understand the federal operating pathway for each mission and do not assume that governmental status alone authorizes flight. |
| Evaluation | Written test and oral scenario. FAA certificate verification for RPICs. |

Learning objectives:

Explain the difference between Part 107 operations, public aircraft operations, public aircraft COA, emergency COA, SGI waiver, and other FAA waiver or exemption pathways.

Verify aircraft registration, aircraft marking, Remote ID status, RPIC certification, recurrent training, airspace authorization, and applicable operating limitations.

Identify when operations over people, operations over moving vehicles, night operations, controlled airspace, or BVLOS require additional authority.

Explain FAA accident reporting triggers and the relationship between internal reporting and FAA/NTSB reporting.

Training content:

Part 107 eligibility, RPIC duties, visual line of sight, right-of-way, altitude, speed, daylight/night, airspace, Remote ID, and recurrent training.

Public aircraft COA basics, operational control, self-certification, agency responsibility, and authorization limitations.

SGI and emergency authorization concepts.

Remote ID and FAA registration verification.

FAA accident reporting and agency reporting timelines.

Learning activities:

Review three sample operations and identify the operating authority.

Review aircraft registration and Remote ID status from a sample aircraft file.

### T-03 Oregon Public Body Requirements, Privacy, Public Records, and Data Policy

| Item | Content |
| --- | --- |
| Duration | 4 hours |
| Audience | All personnel; detailed version for RPICs, payload operators, mission requesters, records staff, public information staff, DFR personnel, and law-enforcement personnel. |
| Purpose | Train UAS personnel to operate in a way that satisfies Oregon public body expectations, privacy limits, public records duties, records retention, data security, and agency transparency requirements. |
| Evaluation | Written quiz and data-handling exercise. |

Learning objectives:

Identify when Oregon public body UAS registration and annual reporting requirements apply.

Explain how agency UAS data may become a public record and how records requests should be routed.

Apply data minimization practices during flight planning, sensor operation, live video viewing, storage, retention, and release.

Distinguish routine public agency missions from law-enforcement missions that may require additional legal authority.

Training content:

Oregon public body UAS registration and annual reporting concepts.

Agency UAS data policy and public notice requirements.

Data minimization, privacy-sensitive areas, incidental capture, live-stream access, and public release.

Evidence, medical data, sensitive infrastructure data, and cybersecurity responsibilities.

Law-enforcement restrictions as an optional legal module only.

Learning activities:

Review a sample dataset and identify what should be retained, deleted, restricted, or routed to records staff.

Scenario: public asks for drone video at the landing zone.

### T-04 Safety Management System, Risk Assessment, and Project Aviation Safety Plan

| Item | Content |
| --- | --- |
| Duration | 4 hours |
| Audience | All operational personnel; advanced content for RPICs, supervisors, instructors, and mission leads. |
| Purpose | Create a common safety process for identifying hazards, assessing risk, documenting controls, and stopping unsafe operations. |
| Evaluation | Risk worksheet and PASP exercise. |

Learning objectives:

Describe the agency SMS elements: policy, risk management, assurance, and promotion.

Complete a risk assessment using agency scoring or a narrative risk matrix.

Prepare a PASP for a routine flight and identify when elevated-risk approval is required.

Use the aviation risk questions, Check 3 GPS, and crew briefing to identify safety concerns before launch.

Training content:

Hazard categories: equipment, human factors, environment, airspace, mission complexity, privacy, data, and public interaction.

Risk probability, severity, mitigation, residual risk, and approval threshold.

PASP elements: mission objective, crew, aircraft, payload, site, airspace, weather, public access, emergency plan, data plan, and postflight records.

Safety reporting, near-miss reporting, and continuous improvement.

Learning activities:

Complete risk assessment for a bridge inspection, a roof inspection, and an emergency damage assessment.

Group discussion on "pressure to launch."

### T-05 Airspace, Weather, NOTAMs, TFRs, LAANC, and Local Aviation Coordination

| Item | Content |
| --- | --- |
| Duration | 4 hours |
| Audience | RPICs, VOs, airspace coordinators, supervisors, DFR candidates, launch/recovery personnel. |
| Purpose | Train personnel to identify aviation hazards, obtain required approvals, and protect crewed aircraft. |
| Evaluation | Airspace planning exercise and oral briefing. |

Learning objectives:

Classify airspace for a mission and identify controlled-airspace approval needs.

Check LAANC, manual authorizations, NOTAMs, TFRs, airports, heliports, seaplane bases, hospital pads, special use airspace, and military training routes.

Read METAR/TAF, winds, visibility, ceiling, temperature, precipitation, density altitude, and local micro-weather factors.

State the land-now procedure for crewed aircraft, airspace conflict, deteriorating weather, or loss of situational awareness.

Training content:

Airspace classes and UAS Facility Map concepts.

NOTAM, TFR, airport, heliport, helipad, and CTAF awareness.

Weather minimums and manufacturer wind/temperature limitations.

Crewed aircraft priority and local coordination.

Incident airspace and optional TFR/SGI awareness.

Learning activities:

Map exercise: plan a flight near an airport and hospital helipad.

Weather decision exercise with marginal wind and low ceiling.

### T-06 Aircraft Systems, Batteries, Remote ID, Preflight, Maintenance, and Return-to-Service

| Item | Content |
| --- | --- |
| Duration | 6 hours plus platform-specific checkout |
| Audience | RPICs, launch/recovery technicians, maintenance/inventory designees, DFR dock technicians, payload operators as needed. |
| Purpose | Ensure personnel can inspect, configure, maintain, store, and document UAS equipment safely. |
| Evaluation | Platform checkout and maintenance log exercise. |

Learning objectives:

Perform an aircraft and controller preflight inspection using the agency checklist.

Verify aircraft registration, marking, Remote ID, firmware/software status, geofencing, batteries, propellers, payload mounts, and log status.

Apply battery safety rules for charging, storage, transport, field use, swelling, fire, and damaged batteries.

Determine when an aircraft must be removed from service and how return-to-service approval is documented.

Training content:

Aircraft airframe, motors, propellers, sensors, GCS, data links, telemetry, batteries, chargers, cases, dock systems, and payloads.

Maintenance log, discrepancies, major/minor repair, firmware control, sensor calibration, and annual inspection.

Battery handling and field fire safety.

Covered UAS/federal funding procurement caution when applicable.

Learning activities:

Hands-on preflight and battery inspection.

Maintenance discrepancy documentation exercise.

### T-07 Mission Planning, Site Survey, Crew Briefing, and Field Operations

| Item | Content |
| --- | --- |
| Duration | 6 hours |
| Audience | RPICs, VOs, payload operators, mission leads, airspace coordinators, launch/recovery personnel. |
| Purpose | Train crews to convert a mission request into a safe, documented field operation. |
| Evaluation | Mission planning package and briefing evaluation. |

Learning objectives:

Complete a mission request review and determine whether the UAS mission is justified.

Conduct a remote and on-site site survey.

Select launch/recovery areas, emergency landing areas, sterile zones, signage, cones, observer positions, and public contact points.

Conduct a mission briefing with roles, callouts, weather, airspace, hazards, emergency actions, data plan, and go/no-go decision.

Training content:

Mission justification: safety, efficiency, time savings, cost savings, documentation, personnel exposure reduction, or public service value.

Remote site review using maps, charts, satellite imagery, and airspace tools.

Launch/recovery site criteria, public access, vehicle/roadway exposure, privacy-sensitive areas, wildlife, terrain, utilities, and RF/magnetic interference.

Standard callouts, sterile cockpit, battery calls, go-around, land-now, and debriefing.

Learning activities:

Prepare a mission package and conduct a mock crew briefing.

Field site survey with an evaluator.

### T-08 Basic Flight Skills, Automation Management, and NIST-Based Proficiency

| Item | Content |
| --- | --- |
| Duration | 8 hours minimum supervised flight plus NIST-based practice |
| Audience | RPICs and supervised operators. |
| Purpose | Build objective flight control skills before independent operational assignment. |
| Evaluation | Practical flight checkride and NIST-based proficiency event. |

Learning objectives:

Demonstrate safe takeoff, hover, climb, descent, yaw, lateral movement, orientation recovery, approach, landing, go-around, and return-to-home management.

Demonstrate positive control in manual and automated modes.

Complete NIST-based proficiency tasks selected by the agency.

Use automation without losing situational awareness or RPIC responsibility.

Training content:

Controls, modes, telemetry, battery management, GPS/GNSS/compass status, RTH settings, camera view limitations, and orientation recovery.

Precision flight, inspection standoff, target identification, orbit, traverse, route, search, and contingency maneuvers.

NIST-based test lane safety, scoring, and documentation.

Learning activities:

Controlled flight drills.

NIST Level 1/2 or agency equivalent practice.

### T-09 Emergency Procedures, Mishap Reporting, and Post-Incident Preservation

| Item | Content |
| --- | --- |
| Duration | 4 hours |
| Audience | All operational UAS personnel; detailed version for RPICs, instructors, supervisors, and records staff. |
| Purpose | Train crews to respond to abnormal events quickly, safely, and consistently. |
| Evaluation | Emergency scenario practical and written reporting exercise. |

Learning objectives:

Execute procedures for lost link, low battery, flyaway, loss of GPS, compass error, aircraft intrusion, public intrusion, payload failure, battery fire, crash, injury, property damage, and lost aircraft.

Identify when to notify emergency services, supervisors, dispatch, airport/ATC, FAA, NTSB, ODAV, insurer, contractor, or interagency aviation contacts.

Preserve logs, aircraft, batteries, controller, data, photos, and witness information after a mishap.

Complete internal and external reporting forms.

Training content:

Emergency priorities: protect people, protect crewed aircraft, protect property, preserve evidence, notify, recover.

Lost-link and flyaway response.

Fire and battery emergency.

Accident/incident reporting thresholds and documentation.

Learning activities:

Tabletop with flyaway toward controlled airspace.

Battery fire and crash site response drill.

### T-10 Crew Resource Management, Threat/Error Management, Fatigue, and Public Interaction

| Item | Content |
| --- | --- |
| Duration | 4 hours |
| Audience | All UAS personnel. |
| Purpose | Improve crew communication, reduce human error, and build professional public-facing behavior. |
| Evaluation | Crew simulation and evaluator observation. |

Learning objectives:

Use clear callouts, readbacks, sterile cockpit, and assertive communication.

Recognize fatigue, stress, distraction, urgency, complacency, fixation, and authority gradients.

Conduct public interaction without escalating the situation or disclosing sensitive information.

Stop or pause operations when communications break down.

Training content:

CRM principles for small UAS crews.

Threat and error management.

Fatigue and fitness for duty.

Public questions, media contact, complaints, and observer interference.

Learning activities:

Role play: visual observer sees a helicopter but RPIC is focused on camera image.

Role play: member of the public challenges the crew at the launch site.

### T-11 Payload, Sensor, Mapping, Inspection, and Data Management

| Item | Content |
| --- | --- |
| Duration | 6 hours or mission-specific |
| Audience | Payload operators, RPICs, mission leads, data stewards, inspection/mapping personnel. |
| Purpose | Train personnel to collect mission data safely, accurately, and lawfully. |
| Evaluation | Payload practical and data-handling exercise. |

Learning objectives:

Configure visual, zoom, thermal, mapping, or inspection payloads for mission objectives.

Maintain standoff from structures, utilities, traffic, people, and wildlife.

Explain limitations of thermal imagery, zoom imagery, mapping accuracy, metadata, and automated software outputs.

Transfer, label, store, retain, and dispose of UAS data under agency policy.

Training content:

Sensor setup, camera controls, exposure/focus, gimbal behavior, mission profiles, metadata, RTK/accuracy, file naming, storage, and data release.

Thermal limitations and interpretation risks.

Privacy and data minimization during sensor operation.

Learning activities:

Payload target exercise.

Data intake and file naming exercise.

### T-12 Mutual Aid, Incident Command, and Multiagency Airspace Deconfliction

| Item | Content |
| --- | --- |
| Duration | 4 hours core awareness; 8 hours for public-safety or mutual-aid personnel |
| Audience | Public-safety personnel, airspace coordinators, supervisors, mission requesters, RPICs who may support multiagency operations. |
| Purpose | Train UAS personnel to integrate with other agencies without freelancing or creating airspace conflicts. |
| Evaluation | Tabletop exercise and airspace coordination drill. |

Learning objectives:

Identify operational control during mutual aid or contractor operations.

Check in under ICS/unified command, identify role and limitations, and avoid self-dispatch.

Coordinate with crewed aircraft, medevac, fire aircraft, law-enforcement aircraft, media aircraft, and other UAS.

Assign sectors, altitude blocks, launch areas, emergency landing areas, and land-now triggers.

Training content:

ICS/NIMS awareness, incident roles, resource ordering, operational period, demobilization, and AAR.

Operational control worksheet.

Multiagency airspace deconfliction and data custody.

Optional wildfire, TFR, SGI, and Fire Traffic Area awareness.

Learning activities:

Map-based multi-UAS deconfliction exercise.

Mutual-aid check-in and credential packet drill.

### T-13 Instructor, Evaluator, Proctor, and Training Records Quality Assurance

| Item | Content |
| --- | --- |
| Duration | 8 hours for instructors/evaluators |
| Audience | Instructors, evaluators, check pilots, proctors, training officers, and program managers. |
| Purpose | Standardize how the agency teaches, tests, documents, remediates, and audits UAS qualifications. |
| Evaluation | Observed teach-back and proctored evaluation. |

Learning objectives:

Use approved lesson plans, rubrics, written exams, oral boards, NIST-based test setup, and scenario scripts.

Score practical performance consistently and identify critical failures.

Create remediation plans and retest procedures.

Audit training records and maintain credential status.

Training content:

Adult learning, safety during training, evaluator ethics, scoring calibration, NIST-based test lane setup, question bank security, records, privacy, and QA audits.

Instructor/evaluator authorization and annual calibration.

Learning activities:

Teach-back.

Sample evaluation scoring calibration.

# Part II. Core Written Examination Program

The written test should be administered after the trainee completes the core curriculum. A routine RPIC passing score should be at least 80 percent with no missed critical safety/legal questions. Agencies may require 90 percent for instructor, evaluator, public-safety, DFR, BVLOS, or law-enforcement endorsements. Failed critical questions should trigger remediation even when the total score is passing.

| Exam Area | Suggested Number of Questions | Minimum Standard |
| --- | --- | --- |
| Agency policy, operational control, approved/prohibited uses | 8-12 | 80 percent and no missed critical prohibited-use question |
| FAA authority, Part 107, COA, Remote ID, registration, accident reporting | 12-18 | 80 percent; RPIC must also maintain FAA recency |
| Oregon public body duties, privacy, public records, data management | 10-15 | 80 percent and no missed data-release question |
| SMS, risk assessment, PASP, mission planning | 10-15 | 80 percent and satisfactory PASP exercise |
| Airspace, weather, TFR/NOTAM, LAANC, local aviation | 10-15 | 80 percent and satisfactory airspace exercise |
| Aircraft systems, preflight, batteries, maintenance | 8-12 | 80 percent and platform checkout |
| Emergency procedures, CRM, public interaction, postflight records | 10-15 | 80 percent and scenario pass |

## Core UAS Written Question Bank

Use this as an instructor-controlled question bank. Agencies should remove the answer key before issuing student exams, randomize question order, and add agency-specific legal, airspace, platform, and policy questions before adoption.

C1. [T-01] Who has final authority to delay, modify, or terminate a UAS flight for safety?

A. The mission requester

B. The Remote Pilot in Command

C. The records officer

D. The visual observer

C2. [T-01] Which statement best describes agency UAS qualification?

A. It is the same thing as an FAA Remote Pilot Certificate

B. It is an agency authorization for a defined role and mission type

C. It is issued automatically after one flight

D. It applies to personal drone use

C3. [T-01] A supervisor asks an employee to fly a personally owned drone for agency work. What should the employee do under this template?

A. Fly if the employee has Part 107

B. Fly only below 100 feet

C. Decline unless the agency policy specifically authorizes it and requirements are met

D. Fly if no data will be retained

C4. [T-01] Which document connects a specific mission to hazards, crew assignments, airspace, emergency actions, and data handling?

A. PASP or mission safety plan

B. Employee time sheet

C. Procurement card log

D. Social media release

C5. [T-01] What is the safest response when operational control is unclear during a multiagency mission?

A. Launch and resolve responsibility later

B. Have the VO decide

C. Identify the lead agency and RPIC authority before launch

D. Assume the agency that owns the drone is always in charge

C6. [T-02] For most small UAS public agency missions under 55 pounds, the standard civil operating pathway is generally:

A. Part 107

B. Part 121

C. Part 135 passenger service

D. No FAA authority is needed for government work

C7. [T-02] Which item must be verified before a Part 107 RPIC operates for the agency?

A. A private pilot certificate

B. A current FAA Remote Pilot Certificate and aeronautical knowledge recency

C. A commercial driver license

D. A radio operator license in all cases

C8. [T-02] Remote ID is best described as:

A. A local agency employee badge

B. A broadcast identification and location capability for the drone in flight

C. A replacement for aircraft registration

D. A waiver allowing BVLOS operations

C9. [T-02] A public aircraft COA should be trained as:

A. A generic permission to fly anywhere

B. An authorization with specific conditions and limitations

C. A replacement for all state law obligations

D. A document needed only for recreational flights

C10. [T-02] Which operation most likely needs additional FAA approval beyond routine Part 107 authority?

A. Daylight VLOS inspection in Class G airspace

B. Routine takeoff from agency property

C. BVLOS DFR operation from a dock

D. Hovering over an empty gravel lot

C11. [T-02] When must FAA accident reporting be evaluated?

A. Only if the news media asks

B. Only for law-enforcement flights

C. After injury, loss of consciousness, or qualifying property damage under applicable FAA rules

D. Only if the drone is destroyed

C12. [T-02] What is the correct training approach for COA or waiver operations?

A. Use only a generic summary

B. Train to the actual authorization and its conditions

C. Let each pilot decide based on experience

D. Train only dispatch staff

C13. [T-02] What does an SGI request generally support?

A. Convenience flights when LAANC is slow

B. Urgent governmental operations supporting a critical public good

C. Commercial photography

D. Recreational park flights

C14. [T-02] Which answer best describes the RPIC responsibility for automation?

A. Automation eliminates RPIC responsibility

B. The RPIC remains responsible for safe operation even when automation is used

C. Only the software vendor is responsible

D. The VO becomes responsible when waypoints are used

C15. [T-03] Before adopting this template, Oregon-specific legal content should be reviewed by:

A. Agency counsel and appropriate program officials

B. Only the newest pilot

C. The vendor only

D. No one, because templates are automatically legally sufficient

C16. [T-03] Which is the best data minimization practice?

A. Record everything all the time

B. Collect only data needed for the approved agency purpose

C. Share live video with anyone who asks

D. Delete all data before records staff can review it

C17. [T-03] A member of the public requests a copy of a drone video at the launch site. The crew should:

A. Provide the file immediately from the controller

B. Refer the request through the agency public records process

C. Post it on social media

D. Ignore the person

C18. [T-03] Which mission requires special caution because Oregon law may impose additional limits?

A. Routine bridge inspection

B. Law-enforcement surveillance or evidentiary mission

C. Roof inspection of agency building

D. Training at a closed field

C19. [T-03] What is the correct response if incidental imagery of private activity is collected but is not needed for the mission?

A. Treat it under agency data policy and minimize, restrict, or dispose according to retention requirements

B. Email it to all program staff

C. Use it for training without review

D. Post a blurred screenshot publicly

C20. [T-03] Who should approve public release of UAS imagery?

A. Any crewmember

B. The agency process designated for records, communications, legal, or public information review

C. The aircraft manufacturer

D. The person who edited the video

C21. [T-03] Why should agencies define data ownership and custody before contractor UAS work?

A. To avoid records, privacy, evidence, cybersecurity, and public release confusion

B. Because contractors cannot fly under Part 107

C. Because all contractor data is automatically private

D. Because the FAA requires social media posting

C22. [T-03] Which item should be in a UAS data policy?

A. Retention, storage, access, public records routing, and deletion/disposition rules

B. Preferred drone color

C. Employee parking plan

D. Lunch arrangements

C23. [T-04] Risk management begins with:

A. Hazard identification

B. Buying the newest aircraft

C. Flying a test lap

D. Posting a public notice

C24. [T-04] Residual risk means:

A. The risk remaining after mitigation

B. The purchase price of the aircraft

C. The number of batteries remaining

D. The risk before planning starts

C25. [T-04] If a mission remains elevated or high risk after mitigation, the crew should:

A. Launch immediately before conditions change

B. Obtain required approval or delay/cancel under agency policy

C. Ask the public to move away and proceed

D. Ignore the score

C26. [T-04] Which factor belongs in a PASP?

A. Mission objective, airspace, crew, aircraft, hazards, emergency plan, and data plan

B. Only the pilot name

C. Only camera settings

D. Only a billing code

C27. [T-04] Which question is most consistent with aviation risk assessment?

A. Is the flight necessary?

B. Can the video be made dramatic?

C. Will the aircraft look impressive?

D. Can the crew skip the briefing?

C28. [T-04] What should happen if conditions change after the mission briefing?

A. Continue because the briefing is already complete

B. Reassess risk and re-brief as needed

C. Let only the payload operator know

D. Ignore changes unless the controller alarms

C29. [T-04] Which is a human-factor hazard?

A. Crew fatigue

B. Clear sky

C. Newly charged battery

D. Open field

C30. [T-04] A good mitigation for a launch site with nearby pedestrians is:

A. Use cones, sterile area, VO support, public contact plan, or select a better site

B. Launch quickly before anyone notices

C. Fly lower

D. Ignore pedestrians if below 55 pounds

C31. [T-05] Before launch, the RPIC should check:

A. Airspace, NOTAMs/TFRs, weather, nearby aviation activity, authorization, and site hazards

B. Only battery percentage

C. Only camera focus

D. Only the mission requester

C32. [T-05] LAANC is used primarily to:

A. Request near-real-time authorization in participating controlled airspace

B. Register the aircraft with Oregon

C. Post flight video

D. Replace Remote ID

C33. [T-05] A TFR is:

A. A temporary flight restriction issued for safety or security reasons

B. A battery type

C. A maintenance form

D. A privacy rule

C34. [T-05] Crewed aircraft should be treated as:

A. Having priority over UAS operations

B. Less important than UAS if the UAS launched first

C. Relevant only above 400 feet

D. A concern only near airports

C35. [T-05] If a helicopter enters the operating area, the correct action is generally to:

A. Continue if the drone is below trees

B. Announce the conflict, yield, and land or hold as briefed

C. Fly toward the helicopter for identification

D. Ignore it if the VO can see it

C36. [T-05] Weather decisions should consider:

A. Wind, gusts, visibility, ceiling, precipitation, temperature, density altitude, and aircraft limits

B. Only the local TV forecast

C. Only whether it is sunny

D. Only cloud color

C37. [T-05] What is the purpose of a VO airspace scan?

A. Assist the RPIC in seeing and avoiding other aircraft and hazards

B. Take over legal responsibility from the RPIC

C. Replace the preflight inspection

D. Approve the mission budget

C38. [T-05] A mission near a hospital helipad should include:

A. Helipad/air medical awareness and a deconfliction plan

B. No extra planning because helipads are not airports

C. A lower battery threshold only

D. Only a data plan

C39. [T-05] What should the crew do if LAANC approval is denied or unavailable for required controlled airspace?

A. Fly at a lower altitude without approval

B. Use manual authorization or postpone until lawful authority is obtained

C. Launch at night

D. Have the VO watch for FAA inspectors

C40. [T-06] Which item is part of a proper preflight?

A. Inspect airframe, propellers, batteries, payloads, controller, Remote ID, and software status

B. Only wipe the camera lens

C. Only check paint condition

D. Only verify the SD card exists

C41. [T-06] A swollen lithium battery should generally be:

A. Used only for short flights

B. Removed from service and handled under battery safety procedures

C. Charged at a higher rate

D. Stored in a hot vehicle

C42. [T-06] Firmware/software changes should be controlled because they can affect:

A. Aircraft behavior, geofencing, Remote ID, payload function, and compatibility

B. Only the color of icons

C. Only the flight log font

D. Nothing operational

C43. [T-06] When should an aircraft be removed from service?

A. When a discrepancy may affect airworthiness or safe operation

B. Only after 500 flights

C. Only if the camera breaks

D. Never, if it can hover

C44. [T-06] Maintenance logs should document:

A. Inspections, repairs, batteries, firmware, discrepancies, and return-to-service decisions

B. Only purchase price

C. Only pilot preferences

D. Only flight videos

C45. [T-06] Why is Remote ID status checked before flight?

A. Because the aircraft must comply with applicable identification requirements

B. Because it proves the pilot is certified

C. Because it replaces the mission briefing

D. Because it improves image resolution

C46. [T-06] A damaged propeller should be:

A. Used if the mission is urgent

B. Replaced before flight

C. Balanced by trimming another propeller in the field

D. Ignored if damage is small

C47. [T-06] Battery fire planning should include:

A. Fire extinguisher/appropriate suppression method, isolation, emergency notification, and safe handling

B. Pouring water on every battery regardless of type

C. Putting the battery in a pocket

D. Covering it with paper

C48. [T-07] A mission request should identify:

A. Mission purpose, requester, location, deliverables, data needs, timing, and constraints

B. Only the desired drone model

C. Only the weather

D. Only the pilot name

C49. [T-07] The best launch/recovery area is:

A. Clear, controlled, away from people/vehicles/obstacles, with emergency options

B. The closest sidewalk

C. A roadway shoulder during traffic

D. Anywhere the drone fits

C50. [T-07] Mission briefings should include:

A. Roles, hazards, airspace, weather, emergency actions, callouts, and data plan

B. Only the camera angle

C. Only the mission budget

D. Only the battery brand

C51. [T-07] Standard callouts help because they:

A. Create shared awareness and reduce ambiguity

B. Make the video sound professional

C. Replace checklists

D. Allow the RPIC to stop scanning

C52. [T-07] If a public contact distracts the RPIC during takeoff, the best response is:

A. Pause or hold the operation and assign a crew member to handle public contact

B. Continue while talking

C. Let the public stand near the aircraft

D. Hand over the controller to the person asking questions

C53. [T-07] What should be done between flights if conditions changed?

A. Conduct a shortened or full re-brief covering changes

B. Do nothing if the first flight succeeded

C. Only swap batteries

D. Update the video title

C54. [T-08] A practical flight evaluation should test:

A. Normal flight, precision control, situational awareness, emergency response, and postflight documentation

B. Only fastest speed

C. Only camera zoom

D. Only takeoff

C55. [T-08] Loss of orientation is best addressed by:

A. Using trained recovery procedures, stopping movement, regaining orientation, or using safe automation as briefed

B. Flying faster

C. Turning off the controller

D. Descending immediately without checking location

C56. [T-08] NIST-based testing is used to:

A. Objectively measure practical proficiency and system capability

B. Replace FAA rules

C. Guarantee legal authority for BVLOS

D. Certify the agency automatically

C57. [T-08] Automation should be monitored because:

A. Waypoints, RTH, geofencing, and sensors can fail or behave unexpectedly

B. Automation always removes risk

C. The VO is responsible for automation

D. Manual mode is illegal

C58. [T-08] A go-around during landing should be used when:

A. A hazard appears or the landing is unstable

B. Battery reaches 95 percent

C. The camera is pointed down

D. The VO asks a non-safety question

C59. [T-08] What is a critical failure during a practical evaluation?

A. Evaluator intervention needed to prevent unsafe flight

B. Forgetting the mission title

C. Using the wrong clipboard

D. Taking too long to pack the case

C60. [T-09] In a flyaway, the RPIC should first:

A. Announce the emergency and attempt approved recovery/lost-link procedures

B. Leave the site immediately

C. Delete the flight log

D. Tell the VO not to watch the aircraft

C61. [T-09] If a UAS may enter controlled airspace during a flyaway, the crew should:

A. Notify the appropriate aviation contact/ATC as briefed and agency policy requires

B. Ignore it if the battery is low

C. Post online

D. Wait until the aircraft lands

C62. [T-09] After a crash with possible injury, the first priority is:

A. Protect life and call emergency services when needed

B. Retrieve the memory card first

C. Restart the aircraft

D. Continue the mission

C63. [T-09] Why preserve the aircraft, battery, controller, logs, and data after a mishap?

A. They may be needed for investigation, reporting, insurance, and corrective action

B. They are no longer useful

C. They should be discarded immediately

D. They are only needed for social media

C64. [T-09] A lost-link procedure should be set and briefed:

A. Before flight

B. After the first warning appears

C. Only after a flyaway

D. Only for night flights

C65. [T-09] Which is an appropriate response to an airspace intrusion?

A. Suspend, hold, land, or deconflict as briefed to protect crewed aircraft

B. Fly toward the intruder

C. Ignore if the intruder is far away

D. Ask the payload operator to decide alone

C66. [T-10] Sterile cockpit means:

A. Only mission-essential communications during critical phases or flight operations

B. No one may speak all day

C. Only the RPIC may speak about any topic

D. The public cannot ask questions

C67. [T-10] An authority gradient occurs when:

A. A crew member hesitates to speak up because of rank or pressure

B. The drone climbs too fast

C. The map is tilted

D. The battery warms up

C68. [T-10] A VO sees a hazard the RPIC has not noticed. The VO should:

A. Clearly and assertively call out the hazard

B. Wait until landing

C. Tell another observer only

D. Assume the RPIC saw it

C69. [T-10] Fatigue risk should be managed by:

A. Rest, duty limits, rotation, supervision, and no-go decisions

B. Caffeine only

C. Ignoring duty time during emergencies

D. Flying faster

C70. [T-10] A member of the public asks why the agency is flying. The best response is:

A. Give a simple approved explanation and route detailed records/media questions through agency process

B. Argue with the person

C. Show all live video

D. Provide legal advice

C71. [T-10] Threat and error management requires the crew to:

A. Identify threats, trap errors, and manage undesired aircraft states

B. Avoid all checklists

C. Use only automated flight

D. Delegate safety to the vendor

C72. [T-11] Thermal imagery can be misleading because:

A. Reflective surfaces, water, smoke, glass, insulation, distance, and heat saturation can affect interpretation

B. Thermal cameras always show exact temperature

C. Thermal cameras eliminate privacy concerns

D. Thermal cameras replace search planning

C73. [T-11] A payload operator should not:

A. Operate sensors in a way that collects unnecessary private activity unrelated to the mission

B. Coordinate with the RPIC

C. Use the approved data plan

D. Confirm focus and exposure

C74. [T-11] Mapping accuracy depends on:

A. Flight plan, altitude, overlap, lighting, camera settings, GNSS/RTK, processing, and ground control when used

B. Only battery color

C. Only pilot preference

D. Only weather app selection

C75. [T-11] Data transfer should include:

A. File naming, metadata preservation, storage location, access controls, retention category, and chain of custody when needed

B. Posting to personal cloud storage

C. Texting files to anyone involved

D. Deleting originals before intake

C76. [T-11] Inspection standoff protects:

A. Aircraft, structures, utilities, people, and mission data quality

B. Only the camera lens

C. Only the payload operator

D. Nothing important

C77. [T-12] In mutual aid, the RPIC still retains:

A. Flight safety authority

B. No responsibilities

C. Public records authority for all agencies

D. The receiving agency budget

C78. [T-12] Self-dispatching a UAS to an incident is:

A. Generally prohibited unless specifically authorized

B. Always acceptable if the pilot is certified

C. Required for every pilot

D. Allowed if the drone is small

C79. [T-12] A multiagency airspace plan should include:

A. Sectors, altitude blocks, launch/recovery zones, emergency landing areas, routes, communications, and land-now triggers

B. Only the drone colors

C. Only the name of the incident

D. Only a social media plan

C80. [T-12] Crewed aircraft in incident airspace require UAS crews to:

A. Coordinate, yield, land/hold as directed, and maintain clear communications

B. Continue unless directly overhead

C. Fly above them

D. Ignore them if under 400 feet

C81. [T-12] Data custody should be resolved during mutual aid because:

A. Multiple agencies may have different records, evidence, retention, privacy, and release duties

B. Data is automatically public domain

C. Only the pilot owns it

D. No data is collected in mutual aid

### Core UAS Written Question Bank - Answer Key

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
| C1: B | C2: B | C3: C | C4: A | C5: C |
| C6: A | C7: B | C8: B | C9: B | C10: C |
| C11: C | C12: B | C13: B | C14: B | C15: A |
| C16: B | C17: B | C18: B | C19: A | C20: B |
| C21: A | C22: A | C23: A | C24: A | C25: B |
| C26: A | C27: A | C28: B | C29: A | C30: A |
| C31: A | C32: A | C33: A | C34: A | C35: B |
| C36: A | C37: A | C38: A | C39: B | C40: A |
| C41: B | C42: A | C43: A | C44: A | C45: A |
| C46: B | C47: A | C48: A | C49: A | C50: A |
| C51: A | C52: A | C53: A | C54: A | C55: A |
| C56: A | C57: A | C58: A | C59: A | C60: A |
| C61: A | C62: A | C63: A | C64: A | C65: A |
| C66: A | C67: A | C68: A | C69: A | C70: A |
| C71: A | C72: A | C73: A | C74: A | C75: A |
| C76: A | C77: A | C78: A | C79: A | C80: A |
| C81: A |  |  |  |  |

# Part III. Core Practical Evaluation Program

The core practical should be administered by an approved evaluator who is qualified on the aircraft family and role being tested. The practical must include a real or simulated mission briefing, aircraft inspection, normal flight, precision task, emergency inject, postflight records, and data handling. Agencies should use NIST-based test methods for objective proficiency measurement where practical, but the agency must set its own local pass thresholds and should not imply NIST certification unless a valid external credential is issued.

## Core Practical Critical-Failure Items

Operating without required FAA authority, agency approval, airspace authorization, aircraft registration, Remote ID compliance, or RPIC qualification.

Evaluator intervention required to avoid collision, airspace violation, injury, property damage, loss of control, or violation of operating limits.

Failure to yield to crewed aircraft or failure to execute land-now/deconfliction procedure.

Launching with known unsafe aircraft, battery, payload, firmware/software, lost-link, or maintenance discrepancy.

Failure to maintain required visual line of sight or situational awareness for the operating authority being used.

Unauthorized collection, disclosure, or mishandling of protected, private, evidentiary, medical, or sensitive data.

Refusal to follow stop-training, stop-flight, or safety direction.

## Core RPIC Practical Test Elements

Evaluator scoring: Pass, Remediate, or Fail. A critical-failure item results in failure of the practical event unless the evaluator determines that the item was simulated and safely corrected under supervision. Agencies may add numeric scoring, NIST-based score sheets, and platform-specific requirements.

| Element | Required Performance | Passing Standard | Critical Failure Examples |
| --- | --- | --- | --- |
| 1. Administrative readiness | Trainee presents FAA certificate/recency if RPIC, agency qualification status, approved mission request, aircraft registration, Remote ID status, insurance/contract info if applicable, and relevant permissions. | All required documents present or missing items identified before flight. Trainee can explain operating authority and limitations. | Missing FAA/agency qualification, unsupported operating authority, or proceeding without required authorization. |
| 2. Mission planning and PASP | Trainee completes mission objective, site map, airspace review, weather review, land access, crew roles, communications, emergency plan, data plan, and postflight record plan. | PASP is complete, practical, and tailored to the mission. | Failure to identify controlled airspace, nearby airport/heliport, TFR, public exposure, or major hazard. |
| 3. Risk assessment | Trainee completes risk scoring or narrative risk review and identifies mitigations and residual risk. | Mitigations reduce risk to agency threshold or mission is delayed/cancelled. | Launch with unmitigated elevated/high risk without required approval. |
| 4. Site setup | Trainee selects launch/recovery zone, emergency landing areas, VO position, sterile area, signage/cones, public contact process, and battery/fire controls. | Site setup protects nonparticipants and supports emergency options. | Unsafe launch near people/vehicles/obstacles or no emergency landing consideration. |
| 5. Preflight inspection | Trainee inspects airframe, propellers, batteries, payload, controller, Remote ID, SD/storage, firmware/software status, compass/GNSS, RTH/lost link settings, and logs. | Checklist completed aloud or documented. Discrepancies resolved before flight. | Aircraft launched with unsafe discrepancy, damaged propeller, unsafe battery, or unverified RTH/lost-link setting. |
| 6. Crew briefing and CRM | Trainee briefs roles, callouts, sterile cockpit, battery calls, airspace scan, emergency procedures, public contact, data plan, and go/no-go criteria. | Crew can repeat roles and emergency actions. | No briefing, unclear roles, or failure to accept VO safety callouts. |
| 7. Normal flight control | Trainee conducts safe takeoff, hover, climb, descent, yaw, lateral movement, orientation recovery, approach, landing, and go-around. | Smooth control, stable aircraft, maintains VLOS, altitude limits, standoff, and situational awareness. | Loss of control, contact with obstacle/person, violation of altitude/airspace, or evaluator intervention. |
| 8. NIST-based proficiency or equivalent | Trainee completes agency-selected precision tasks such as position, traverse, orbit, inspect, recon, target identification, or sensor task. | Meets agency score/time/safety threshold with no unsafe deviation. | Collision, unsafe deviation, loss of situational awareness, or failure to complete required task. |
| 9. Payload/sensor operation | Trainee uses camera/thermal/zoom/mapping payload to collect mission data while minimizing unnecessary collection. | Mission data collected to standard and handled according to data plan. | Unauthorized collection, unsafe fixation on camera image, or failure to preserve required data. |
| 10. Emergency procedures | Evaluator injects one abnormal event: crewed aircraft, low battery, lost link, compass/GNSS, public intrusion, flyaway, or crash/injury scenario. | Trainee announces emergency, uses checklist, communicates, protects people/airspace, and documents event. | Fails to yield to crewed aircraft, unsafe response, or failure to notify/report required event. |
| 11. Postflight | Trainee powers down, inspects aircraft, logs flight, battery, maintenance, data intake, discrepancies, and debrief/AAR. | Records complete and discrepancies handled. | Missing flight/maintenance records or continued use after unsafe discrepancy. |
| 12. Professional conduct | Trainee handles public questions, supervisor pressure, and crew concerns appropriately. | Calm, professional, policy-based response. | Unsafe compliance with pressure to launch, inappropriate disclosure, or escalation with public. |

## Core Oral Board Scenario Prompts

You are asked to fly a routine inspection in controlled airspace, but LAANC is unavailable. Explain your decision path and what documentation is needed before flight.

A senior manager asks you to launch before the full crew briefing because the event is about to start. Explain how you respond.

During a bridge inspection, a news helicopter begins orbiting nearby. Explain your immediate actions and notifications.

A member of the public asks whether they are being recorded and demands a copy of the video. Explain what you say at the scene and what agency process applies.

After landing, you discover a cracked propeller that may have occurred during the flight. Explain aircraft status, records, and return-to-service process.

Your payload operator collected imagery outside the mission area. Explain data minimization, retention, and reporting steps.

You are supporting another agency and both agencies assume the other is in operational control. Explain how you resolve the issue before launch.

A flyaway is trending toward controlled airspace. Explain emergency steps, notifications, and post-incident preservation.

# Addendum A. DFR and Remote Operations Training Module

This addendum is optional and should be adopted only by an agency that has approved DFR or remote operations policy, applicable FAA authorization, dispatch/RTIC/RTCC procedures, live-video/data controls, remote launch/dock procedures, and an agency-approved training/evaluation program. This module does not create authority to conduct DFR or BVLOS operations.

| Module | Duration | Training Content | Evaluation |
| --- | --- | --- | --- |
| DFR-01 DFR Program Governance and Authorization | 4 hours | DFR purpose, approved call types, prohibited uses, operating authority, RPIC/remote operator role, airspace monitor role, dispatch/RTIC/RTCC role, supervisor role, and legal/privacy review. | Written exam and oral board using the agency authorization and DFR policy. |
| DFR-02 Dispatch, Call Screening, and Launch Decision | 4 hours | Eligible and non-eligible call types, launch/no-launch decision, mission purpose, recording trigger, termination criteria, responder safety, public transparency, and dispatch logging. | Dispatch simulation with calls that launch and calls that do not launch. |
| DFR-03 Remote/Dock Preflight and Launch Workflow | 6 hours | Dock health, aircraft health, weather sensors, camera view, telemetry, C2 link, Remote ID, battery, launch area verification, obstruction map, RTH altitude, lost-link profile, emergency landing points, and abort criteria. | Remote/dock practical and system-health oral board. |
| DFR-04 RTIC/RTCC, CAD, Live Video, and Data Controls | 4 hours | CAD integration, mission logging, live-feed access, video sharing limits, role-based permissions, recording, data retention, public records, evidence, medical/privacy issues, and public information routing. | Video/data scenario evaluation. |
| DFR-05 Airspace Monitoring and Crewed-Aircraft Deconfliction | 4 hours | Airports, heliports, hospital pads, TFR/NOTAM, altitude/route limits, crewed-aircraft priority, land-now triggers, airspace-monitor responsibilities, and field-responder communications. | Map-based airspace drill and crewed-aircraft intrusion inject. |
| DFR-06 DFR Emergency Procedures and Mission Termination | 4 hours | Lost link, loss of video, C2 degradation, geofence, low battery, dock failure, remote recovery, flyaway, emergency landing, unauthorized UAS/aircraft conflict, public complaint, and post-event reporting. | Emergency simulator or live-safe scenario. |
| DFR-07 Supervised DFR Shifts and Final Scenario Evaluation | Agency-defined minimum, suggested 5-10 shifts or simulations | Supervised operations with evaluator notes, call-screening decisions, prelaunch checks, communications, data controls, and debrief. | Final DFR scenario with no critical safety, privacy, data, or FAA authorization error. |

## Addendum A Written Question Bank - DFR and Remote Operations

Use this as an instructor-controlled question bank. Agencies should remove the answer key before issuing student exams, randomize question order, and add agency-specific legal, airspace, platform, and policy questions before adoption.

D1. [DFR-01] What makes DFR different from a routine field-deployed UAS mission?

A. It often uses prepositioned aircraft, rapid dispatch, remote operations, live video, and defined call-screening workflows

B. It has no aviation rules

C. It is always recreational

D. It cannot collect data

D2. [DFR-01] Before a DFR program begins, personnel must be trained on:

A. The actual FAA authorization, agency policy, call types, privacy, data sharing, and emergency procedures

B. Only the camera controls

C. Only the docking station brand

D. Only the press release

D3. [DFR-01] A DFR launch decision is both:

A. A public-safety decision and an aviation decision

B. A media decision and budget decision only

C. A vendor decision only

D. A social media decision

D4. [DFR-01] Who may view a live DFR video feed?

A. Only persons authorized by agency policy and the mission purpose

B. Anyone in the agency

C. Any nearby responder's family member

D. The general public by default

D5. [DFR-01] A DFR operator receives a call type that is not approved for DFR launch. The correct action is:

A. Do not launch unless a supervisor/legal/authorized pathway approves under policy

B. Launch and document later

C. Launch without recording

D. Ask the vendor to decide

D6. [DFR-02] A remote/dock preflight should verify:

A. Aircraft health, dock status, weather, Remote ID, airspace, authorization, camera view, C2 link, lost-link settings, and emergency landing options

B. Only that the aircraft is charged

C. Only that the dock door opens

D. Only that dispatch is online

D7. [DFR-02] If the dock weather sensor conflicts with observable weather or reports unsafe wind, the operator should:

A. Pause/cancel launch until the hazard is resolved

B. Launch because dispatch requested it

C. Turn off the weather sensor

D. Switch to sport mode

D8. [DFR-02] Remote launch should be cancelled if:

A. Camera, telemetry, C2 link, dock, airspace, weather, or crew status is not acceptable

B. The incident is interesting

C. A supervisor wants video

D. Battery is fully charged

D9. [DFR-02] Remote operations require special attention to latency because:

A. Delayed video or telemetry can affect aircraft control and hazard recognition

B. Latency improves safety

C. Latency only affects public records

D. Latency eliminates the need for a VO

D10. [DFR-02] The return-to-home altitude for a dock operation should be:

A. Set according to obstruction analysis, authorization, and local procedure before flight

B. Always 400 feet

C. Always 50 feet

D. Changed randomly during flight

D11. [DFR-03] CAD/RTIC/RTCC integration training should include:

A. Call screening, launch authorization, mission logging, video access, and data retention

B. Only radio call signs

C. Only battery charging

D. Only aircraft color coding

D12. [DFR-03] If a field responder asks to use DFR video for an unrelated purpose, the operator should:

A. Follow agency policy and deny or route the request through approval channels

B. Share it immediately

C. Record a separate clip on a personal phone

D. Post it online

D13. [DFR-03] A DFR mission log should document:

A. Call type, launch decision, operator/RPIC, aircraft, time, location, authority, data disposition, and any abnormal events

B. Only the video filename

C. Only the dispatcher name

D. Only the battery serial number

D14. [DFR-03] Community transparency training should prepare personnel to explain:

A. What DFR is used for, what it is not used for, how privacy is protected, and how records requests are handled

B. Technical source code

C. Personal opinions about calls

D. Names of involved persons

D15. [DFR-03] Live video should be stopped or restricted when:

A. The mission purpose ends or continued viewing is not authorized or necessary

B. The aircraft is still flying any mission

C. The public asks a question

D. The map opens

D16. [DFR-04] A DFR airspace monitor should be trained to identify:

A. Crewed aircraft, TFRs, controlled airspace, hospital pads, airports, launch routes, and land-now triggers

B. Only the drone icon

C. Only the pilot's username

D. Only nearby roads

D17. [DFR-04] If a crewed aircraft conflict occurs during DFR flight, the operator should:

A. Execute the approved deconfliction or land-now procedure

B. Continue to get video

C. Descend only if convenient

D. Wait for dispatch to call back

D18. [DFR-04] DFR altitude and route limits come from:

A. FAA authorization, agency policy, airspace plan, obstruction analysis, and mission need

B. Operator preference only

C. The highest available setting

D. The camera zoom range

D19. [DFR-04] In a DFR mission, the field responder should not be expected to:

A. Serve as the only aviation safety control unless policy and training specifically assign that role

B. Receive useful situational awareness

C. Communicate safety concerns

D. Request the aircraft land if unsafe

D20. [DFR-04] A DFR mission should terminate when:

A. Authorization, weather, aircraft health, C2 link, airspace, staffing, or mission purpose no longer supports flight

B. The aircraft reaches 90 percent battery

C. The camera sees the incident

D. A person waves

D21. [DFR-05] Which event requires immediate DFR emergency action?

A. Lost link, loss of video needed for safe operation, crewed-aircraft conflict, geofence breach, low battery, or flyaway

B. A minor spelling error in the log

C. A quiet radio channel

D. A full SD card after landing

D22. [DFR-05] A loss-of-video event in remote operations is serious because:

A. Video may be needed for navigation, situational awareness, and mission safety under the authorization

B. Video is only for public relations

C. It proves the aircraft crashed

D. It never matters if telemetry remains

D23. [DFR-05] If a remote operator cannot confirm safe launch conditions, the operator must:

A. Not launch

B. Launch and climb higher

C. Ask the incident commander to accept the risk verbally

D. Disable alarms

D24. [DFR-05] DFR emergency training should include:

A. Lost link, loss of video, C2 degradation, low battery, flyaway, geofence, dock failure, and crewed-aircraft intrusion

B. Only crash photography

C. Only battery swaps

D. Only press notification

D25. [DFR-05] After an abnormal DFR event, the crew should:

A. Document the event, preserve logs/data, notify required officials, and determine return-to-service status

B. Delete all logs

C. Continue normal missions without review

D. Let the vendor handle all agency records

D26. [DFR-06] DFR practical evaluation should include calls that:

A. Should launch and should not launch

B. All require launch

C. Only involve sunny weather

D. Avoid privacy questions

D27. [DFR-06] A DFR scenario evaluation should test whether the trainee can:

A. Justify launch, limit video, communicate with dispatch/responders, manage airspace, and terminate appropriately

B. Fly as fast as possible

C. Ignore call notes

D. Use personal judgment without policy

D28. [DFR-06] A supervised DFR shift should document:

A. Call screening decisions, flight performance, communications, data handling, abnormal events, and evaluator comments

B. Only hours on duty

C. Only number of views

D. Only battery cycles

D29. [DFR-06] A critical failure in DFR evaluation includes:

A. Unauthorized launch, unauthorized video disclosure, violation of FAA authorization, unsafe airspace response, or failure to terminate when required

B. Taking notes slowly

C. Using a checklist

D. Asking a supervisor for clarification

D30. [DFR-06] If DFR video captures uninvolved persons after the mission purpose is complete, the operator should:

A. Stop or redirect collection and handle data under policy

B. Continue recording because the drone is already airborne

C. Save the clip for training automatically

D. Share it with all responders

### Addendum A Written Question Bank - DFR and Remote Operations - Answer Key

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
| D1: A | D2: A | D3: A | D4: A | D5: A |
| D6: A | D7: A | D8: A | D9: A | D10: A |
| D11: A | D12: A | D13: A | D14: A | D15: A |
| D16: A | D17: A | D18: A | D19: A | D20: A |
| D21: A | D22: A | D23: A | D24: A | D25: A |
| D26: A | D27: A | D28: A | D29: A | D30: A |

## Addendum A Practical Test Elements - DFR and Remote Operations

Evaluator scoring: Pass, Remediate, or Fail. A critical-failure item results in failure of the practical event unless the evaluator determines that the item was simulated and safely corrected under supervision. Agencies may add numeric scoring, NIST-based score sheets, and platform-specific requirements.

| Element | Required Performance | Passing Standard | Critical Failure Examples |
| --- | --- | --- | --- |
| DFR-P1. Call screening | Trainee reviews sample CAD/dispatch call types and selects launch/no-launch under agency policy. | Launch decisions match policy and mission purpose. | Unauthorized DFR launch or failure to launch only when required/authorized. |
| DFR-P2. Authorization and prelaunch check | Trainee verifies FAA authorization, weather, airspace, TFR/NOTAM, aircraft health, Remote ID, dock/camera, C2 link, telemetry, staffing, and privacy/data settings. | All required DFR launch criteria verified before takeoff. | Launch with failed system check, unsafe weather, missing authorization, or unstaffed required role. |
| DFR-P3. Dock/remote launch | Trainee demonstrates remote dock workflow, confirms clear launch area, initiates launch, monitors climb, and follows route/altitude limits. | Safe launch and transition to mission route. | Launch despite obstruction/personnel in dock area or loss of required situational awareness. |
| DFR-P4. Dispatch/RTIC/field communications | Trainee communicates with dispatch/RTIC/field responders using approved phraseology and logs key events. | Timely, clear, policy-compliant communications. | Unapproved channels, confusing handoff, or failure to relay safety-critical information. |
| DFR-P5. Live video and data controls | Trainee limits feed access, starts/stops recording according to policy, avoids unnecessary collection, and documents data custody. | Video use tied to mission purpose and access controls. | Unauthorized sharing, recording beyond mission purpose, or improper release. |
| DFR-P6. Crewed-aircraft conflict | Evaluator injects aircraft/helipad/medevac conflict. | Trainee executes hold/land/deconfliction under policy. | Failure to yield or delayed response creating aviation hazard. |
| DFR-P7. Loss of video/C2/lost link | Evaluator injects loss of video or degraded C2. | Trainee follows contingency, communicates, and terminates if required. | Continuing without required safety information or ignoring system failure. |
| DFR-P8. Mission termination | Trainee ends mission when purpose is complete, conditions fail, or termination criteria are met. | Timely return/landing and closure log. | Continues flight or viewing after authority/need ends. |
| DFR-P9. Post-incident documentation | Trainee completes DFR mission log, abnormal event report if needed, data disposition, maintenance status, and AAR. | Records complete and usable for audit. | Missing mission log, missing anomaly report, or failure to remove aircraft from service when needed. |

# Addendum B. BVLOS, COA, Waiver, and Advanced Remote Operations Module

This addendum is optional and must be tailored to the agency's actual FAA authorization. It should not be used as a stand-alone authorization to fly BVLOS. All examples, routes, altitudes, airspace monitoring procedures, visual observer configurations, detect-and-avoid procedures, reporting requirements, and emergency actions must be replaced with the agency's approved conditions before training personnel for operational use.

| Module | Duration | Training Content | Evaluation |
| --- | --- | --- | --- |
| BV-01 Authorization-Specific Ground School | 4 hours | Review the actual waiver/COA/BVLOS provision: who may act as RPIC, area, altitude, aircraft, crew, airspace, weather, DAA or shielding, reporting, and termination criteria. | Authorization oral board and written test. |
| BV-02 Route/Area Design, Shielding, and Risk Controls | 4 hours | Route planning, shielding objects if authorized, lateral and vertical boundaries, obstruction map, emergency landing areas, C2 coverage, ground risk, privacy-sensitive areas, and route documentation. | Route-planning practical. |
| BV-03 Detect-and-Avoid and Airspace Surveillance | 4 hours | DAA source limitations, non-cooperative aircraft, false alerts, alert thresholds, airspace monitor duties, crewed-aircraft priority, communications, and land-now procedures. | Airspace-monitoring simulation. |
| BV-04 C2 Link, Telemetry, Video, and Automation Management | 4 hours | C2 link monitoring, telemetry, video latency, route automation, lost-link settings, geofence, RTH, emergency descent/land, and automation failure modes. | Simulator or controlled scenario. |
| BV-05 BVLOS Emergency and Contingency Procedures | 4 hours | Lost link, loss of surveillance, loss of video, geofence breach, aircraft conflict, weather deterioration, emergency landing, flyaway, notifications, and post-incident preservation. | Emergency inject evaluation. |
| BV-06 BVLOS Records, Reporting, and Quality Assurance | 2 hours | Authorization-required records, anomaly reporting, lost-link reporting, flight logs, DAA logs, maintenance/return-to-service, data custody, AAR, and recurrent qualification. | Records/reporting exercise. |

## Addendum B Written Question Bank - BVLOS, COA, Waiver, and Advanced Remote Operations

Use this as an instructor-controlled question bank. Agencies should remove the answer key before issuing student exams, randomize question order, and add agency-specific legal, airspace, platform, and policy questions before adoption.

B1. [BV-01] Routine BVLOS operations may be conducted only when:

A. The agency has a waiver, COA provision, or other FAA authorization allowing the operation

B. The pilot is comfortable

C. The aircraft has a zoom camera

D. The flight is below 400 feet in all cases

B2. [BV-01] BVLOS training must be authorization-specific because:

A. Conditions, crew roles, airspace limits, weather minimums, DAA methods, and reporting requirements differ

B. All BVLOS approvals are identical

C. The vendor manual replaces FAA conditions

D. Only dispatch needs training

B3. [BV-01] The operator must know who is authorized to act as RPIC because:

A. The authorization may limit who can command, monitor, or terminate BVLOS flight

B. Anyone can act as RPIC remotely

C. The VO always becomes RPIC

D. The software assigns RPIC automatically

B4. [BV-01] Which item should be tested in a BVLOS authorization oral board?

A. Altitude limits, area limits, crew roles, communication requirements, contingency procedures, and reporting obligations

B. Aircraft color

C. Media plan

D. Vendor logo

B5. [BV-01] If the planned BVLOS route is outside the authorized area, the correct action is:

A. Do not fly until the route is revised or proper authorization is obtained

B. Fly if below rooftops

C. Fly only with recording off

D. Ask the public to watch

B6. [BV-02] A BVLOS route plan should identify:

A. Authorized route/area, obstacles, shielding objects if used, emergency landing zones, C2 coverage, airspace, and crewed-aircraft hazards

B. Only the destination

C. Only the camera angle

D. Only the dispatch priority

B7. [BV-02] Shielded operations training should define:

A. Which objects create the shield, lateral/vertical boundaries, altitude limits, and failure conditions

B. Any tree anywhere

C. The pilot's preference

D. A blanket approval for the city

B8. [BV-02] If the agency relies on a detect-and-avoid system, trainees must know:

A. Capabilities, limitations, alerts, false positives, false negatives, non-cooperative aircraft, and response actions

B. Only how to silence alerts

C. Only the subscription cost

D. That DAA eliminates all risk

B9. [BV-02] An emergency landing point in BVLOS planning should be:

A. Known, documented, safe, within authorization, and considered before launch

B. Selected after the battery fails

C. Any roof

D. The original incident scene

B10. [BV-02] C2 coverage matters because:

A. Loss of command/control can create a flyaway or lost-link event requiring contingency actions

B. It improves video color

C. It replaces weather checks

D. It eliminates need for logs

B11. [BV-03] Non-cooperative aircraft are aircraft that:

A. May not be visible to electronic surveillance or cooperative detection systems

B. Always broadcast Remote ID

C. Are always above 10,000 feet

D. Do not exist near cities

B12. [BV-03] BVLOS crews should respond to a crewed-aircraft alert by:

A. Following the authorization and agency deconfliction/land-now procedure

B. Continuing unless the aircraft is visible on camera

C. Climbing to improve signal

D. Disabling the alert

B13. [BV-03] Why is ADS-B information not a complete detect-and-avoid solution by itself?

A. Not all aircraft broadcast or are required to broadcast in all areas, and data may have limitations

B. It always detects birds

C. It replaces visual observers in all cases

D. It controls the drone automatically

B14. [BV-03] Airspace surveillance training should include:

A. What sources are used, what they miss, who monitors them, and what actions are required

B. Only map colors

C. Only the app login

D. Only the vendor contract

B15. [BV-03] If the DAA/airspace-monitoring system fails during BVLOS, the operator should:

A. Execute the approved contingency or terminate as required

B. Continue using camera only unless convenient

C. Ignore the failure if the mission is urgent

D. Ask dispatch to watch social media

B16. [BV-04] BVLOS contingency procedures should include:

A. Lost link, C2 degradation, loss of surveillance, loss of video, GPS/compass anomaly, geofence breach, and emergency landing

B. Only low battery

C. Only a phone call

D. Only a postflight note

B17. [BV-04] A BVLOS lost-link profile should be:

A. Configured, briefed, tested as appropriate, and consistent with authorization, route, and obstacles

B. Left at factory default without review

C. Changed in flight randomly

D. Ignored if the battery is full

B18. [BV-04] What should a BVLOS crew do when telemetry becomes unreliable?

A. Treat it as an abnormal condition and follow contingency procedures

B. Continue normally

C. Increase altitude without analysis

D. End the log

B19. [BV-04] A geofence alert in BVLOS operations should:

A. Trigger immediate evaluation and response under the authorization and emergency procedures

B. Be ignored as a nuisance

C. Be overridden automatically

D. Be handled after landing only

B20. [BV-04] Emergency descent or landing during BVLOS should prioritize:

A. Protection of people, crewed aircraft, property, and compliance with the authorization

B. Camera coverage

C. Fastest possible descent anywhere

D. Keeping the mission going

B21. [BV-05] Remote operations handoff procedures should specify:

A. When control changes, who is RPIC, who monitors airspace, and how communications are confirmed

B. Only usernames

C. Only aircraft serial number

D. Only video bitrate

B22. [BV-05] A BVLOS mission should not continue if:

A. Required crew, systems, surveillance, C2 link, weather, airspace, or authorization conditions are not met

B. The camera is zoomed out

C. The video is ordinary

D. The flight is uneventful

B23. [BV-05] BVLOS records should document:

A. Authorization used, route/area, crew, aircraft, systems, airspace monitoring, anomalies, lost links, and reporting items

B. Only a screenshot

C. Only call notes

D. Only the SD card size

B24. [BV-05] Who should be trained on BVLOS termination criteria?

A. RPIC, remote operator, airspace monitor, dispatch/RTIC staff as assigned, supervisors, and launch/dock personnel as applicable

B. Only the vendor

C. Only public information staff

D. Only finance

B25. [BV-05] If a BVLOS mission becomes VLOS during part of the flight, the crew should:

A. Continue to comply with the authorization and agency procedures; VLOS does not erase other conditions

B. Ignore the authorization

C. Stop logging

D. Allow an untrained observer to command the aircraft

B26. [BV-06] A BVLOS practical should test:

A. Authorization compliance, route planning, launch, system monitoring, abnormal procedures, crew communications, and postflight reporting

B. Only fast flight

C. Only still photography

D. Only hand launching

B27. [BV-06] A critical BVLOS evaluation failure includes:

A. Operating outside authorization, missing crewed-aircraft conflict, ignoring C2/surveillance failure, or unauthorized route deviation

B. Using a checklist

C. Canceling due to weather

D. Asking for clarification

B28. [BV-06] BVLOS training should include supervised simulations because:

A. Some failures are unsafe or impractical to create live, but must still be evaluated

B. Simulations replace all live proficiency

C. They are easier than written tests

D. They avoid the need for policy

B29. [BV-06] Postflight BVLOS debriefing should include:

A. Authorization compliance, anomalies, lost links, crew communications, airspace conflicts, data/reporting, and corrective actions

B. Only whether video was useful

C. Only equipment packing

D. Only weather

B30. [BV-06] Before adding new BVLOS route, dock, aircraft, or DAA system, the agency should:

A. Update training, test procedures, risk assessment, authorization review, and evaluator standards

B. Let operators learn in the field

C. Skip training if vendor support is available

D. Only update the website

### Addendum B Written Question Bank - BVLOS, COA, Waiver, and Advanced Remote Operations - Answer Key

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
| B1: A | B2: A | B3: A | B4: A | B5: A |
| B6: A | B7: A | B8: A | B9: A | B10: A |
| B11: A | B12: A | B13: A | B14: A | B15: A |
| B16: A | B17: A | B18: A | B19: A | B20: A |
| B21: A | B22: A | B23: A | B24: A | B25: A |
| B26: A | B27: A | B28: A | B29: A | B30: A |

## Addendum B Practical Test Elements - BVLOS and Advanced Remote Operations

Evaluator scoring: Pass, Remediate, or Fail. A critical-failure item results in failure of the practical event unless the evaluator determines that the item was simulated and safely corrected under supervision. Agencies may add numeric scoring, NIST-based score sheets, and platform-specific requirements.

| Element | Required Performance | Passing Standard | Critical Failure Examples |
| --- | --- | --- | --- |
| BV-P1. Authorization oral board | Trainee explains the specific waiver/COA/BVLOS authorization, area, altitude, crew roles, surveillance/DAA method, weather minimums, and reporting requirements. | Accurate explanation of authorization and limitations. | Cannot identify authority, limits, required crew, or termination criteria. |
| BV-P2. Route and risk plan | Trainee creates route/area plan with obstacles, shielding if used, emergency landing points, C2 coverage, airspace, and contingency zones. | Route is inside authorization and risk controls are documented. | Route outside authorization or no contingency plan. |
| BV-P3. Airspace monitoring | Trainee demonstrates monitoring method and response to cooperative and non-cooperative aircraft risk. | Aircraft conflict recognized and acted upon. | Fails to respond to crewed aircraft or misunderstands DAA limitations. |
| BV-P4. Remote/BVLOS launch | Trainee conducts launch while monitoring telemetry, video, C2 link, aircraft health, and crew communications. | Launch and climb comply with procedure and authorization. | Launch without required systems or required crew. |
| BV-P5. C2/surveillance failure | Evaluator injects C2 degradation, surveillance failure, or loss of telemetry. | Trainee executes contingency and terminates if required. | Continues without required mitigation. |
| BV-P6. Lost-link/geofence/emergency landing | Trainee demonstrates response to lost link/geofence alert/emergency landing scenario. | Correct checklist use, communications, and documentation. | Incorrect lost-link profile, unsafe route, or no notification. |
| BV-P7. Postflight compliance report | Trainee completes flight record, anomalies, lost-link data, authorization-required reporting, data handling, and AAR. | Records meet authorization and agency requirements. | Failure to report required event or missing logs. |

# Appendix 1. Practical Evaluation Score Sheet

Use one score sheet for each practical event. Attach NIST-based score sheets, scenario notes, platform checklists, and remediation plans when applicable.

| Field | Entry |
| --- | --- |
| Trainee | [Name] |
| Role / Endorsement Tested | [RPIC / VO / Payload / DFR / BVLOS / Other] |
| Evaluator | [Name] |
| Date / Location | [Date / Site] |
| Aircraft / Payload | [Make / Model / Serial / Payload] |
| Operating Authority | [Part 107 / COA / Waiver / Training / Other] |
| Weather / Airspace | [Summary] |
| Overall Result | [Pass / Remediate / Fail] |
| Limitations | [If any] |
| Next Required Action | [Qualification / Remediation / Retest / Recurrent] |

| Element | Pass | Remediate | Fail | Evaluator Notes |
| --- | --- | --- | --- | --- |
| 1. Administrative readiness | [ ] | [ ] | [ ] |  |
| 2. Mission planning and PASP | [ ] | [ ] | [ ] |  |
| 3. Risk assessment | [ ] | [ ] | [ ] |  |
| 4. Site setup | [ ] | [ ] | [ ] |  |
| 5. Preflight inspection | [ ] | [ ] | [ ] |  |
| 6. Crew briefing and CRM | [ ] | [ ] | [ ] |  |
| 7. Normal flight control | [ ] | [ ] | [ ] |  |
| 8. NIST-based proficiency or equivalent | [ ] | [ ] | [ ] |  |
| 9. Payload/sensor operation | [ ] | [ ] | [ ] |  |
| 10. Emergency procedures | [ ] | [ ] | [ ] |  |
| 11. Postflight | [ ] | [ ] | [ ] |  |
| 12. Professional conduct | [ ] | [ ] | [ ] |  |

# Appendix 2. Scenario Evaluation Template

| Scenario Item | Agency Entry |
| --- | --- |
| Scenario title | [Insert] |
| Training objective | [Insert] |
| Participants and roles | [Insert] |
| Operating authority assumed | [Insert] |
| Initial conditions | [Weather, airspace, site, aircraft, call type] |
| Expected actions | [Insert] |
| Inject 1 | [Insert] |
| Inject 2 | [Insert] |
| Inject 3 | [Insert] |
| Critical failures | [Insert] |
| Records to complete | [Flight log, data form, incident form, AAR] |
| AAR questions | [What went well? What changed? What needs correction?] |

# Addendum C: Remote Pilot Certification Standards, Carding, and Task-Based Evaluation

This addendum integrates the May 2026 Certification Standards for Remote Pilot Operations into the expanded curriculum and testing addenda. It is written for generic Oregon public agency use and can be customized for RDPO, regional mutual-aid, fire, public works, transportation, DFR, ELOS/BVLOS, and agency-specific carding systems.

This module converts the certification standards into trainable lesson content, written test questions, oral board prompts, and practical evaluation elements. It should be used after the core UAS curriculum and before final agency qualification or mission endorsement.

## C-1 Learning Objectives

Explain why the FAA Remote Pilot Certificate is the baseline but not the full agency qualification for interagency or specialized public agency operations.

Identify the special emphasis areas that must be evaluated throughout training and checkrides.

Use the four-phase training and evaluation method for demonstration, training, evaluation, and debrief.

Apply the rating system of DM, 4, 3, 2, 1 during training and S/U during final evaluation.

Use the Master Task List to build practical evaluation plans for basic, aircraft-specific, mission-specific, DFR, ELOS/BVLOS, and non-standard operations.

Maintain pilot cards, current-year designations, endorsements, evaluator approvals, and training records in an auditable format.

## C-2 Lesson Content

### C-2. Certification Standard Concept

Agency certification is a documented finding that the pilot possesses the knowledge, skill, ability, and judgment required for the aircraft and mission under the agency's operational control. It adds local policy, aviation safety, risk management, crew coordination, interagency airspace, and mission-specific proficiency to FAA Part 107 certification.

### C-2. Special Emphasis Areas

Every lesson, scenario, and practical test shall reinforce cooperation, professionalism, aviation safety culture, risk assessment, aeronautical decision making, crew resource management, checklist usage, and stop-flight authority.

### C-2. Checklists and Division of Attention

Pilots shall use appropriate aircraft and mission checklists. Evaluators should consider whether checklist use is safe at the moment, whether the pilot maintains visual scan and situational awareness, and whether deferred checklist review is appropriate in a single-pilot or high-workload event.

### C-2. Training Methods

Demonstration flights introduce the correct standard. Training flights allow the pilot to practice with feedback. Evaluation flights are pass/fail or standards-check events. Simulator, mock-up, computer-based instruction, or classroom methods may be used when appropriate to the objective.

### C-2. Evaluation Phases

Phase 1 is introduction and records review. Phase 2 is knowledge competency. Phase 3 is the flight exercise with briefing, preflight, flight, and postflight tasks. Phase 4 is debrief and documentation.

### C-2. Evaluation Criteria

Training ratings use DM, 4, 3, 2, and 1. Final evaluations use only Satisfactory or Unsatisfactory. Any final task that is equivalent to a 1, or any critical safety, legal, or authorization failure, is unsatisfactory.

### C-2. Basic Qualities

Evaluators should describe performance using cognitive qualities such as retention, aptitude, planning, judgment, and alertness; physical qualities such as control touch, coordination, reaction time, and crew coordination; and emotional qualities such as motivation, attitude, confidence, composure, and safety.

### C-2. Records and Carding

The agency training file shall show the pilot's qualifications, endorsements, designations, limitations, current-year training, flight time, checkrides, evaluator signatures, remediation, and expiration dates.

## C-3 Written Test Bank: Remote Pilot Certification Standards

| # | Question | Choices | Answer |
| --- | --- | --- | --- |
| C1 | The FAA Remote Pilot Certificate should be treated as which of the following for agency UAS operations? | A. The only qualification needed for all agency missions. B. The federal baseline that must be supplemented by agency training, task proficiency, and mission endorsements. C. A substitute for aircraft-specific training. D. A substitute for agency policy review. | B |
| C2 | Which special emphasis area must be evaluated even when not listed under a specific task? | A. Public relations only. B. Risk management, ADM, CRM, checklist usage, professionalism, and aviation safety attitude. C. Payload image quality only. D. Maximum speed performance. | B |
| C3 | What is the purpose of a Master Task List? | A. To replace the operations manual. B. To identify task standards that can be assembled into training, standards-check, or final evaluation plans. C. To eliminate evaluator judgment. D. To approve FAA waivers. | B |
| C4 | During a demonstration flight, what is the expected outcome? | A. The pilot must be fully proficient. B. The evaluator or trainee introduces the correct task standard and answers questions. C. The task must be scored S/U. D. The pilot is automatically carded. | B |
| C5 | During a training flight, how should the evaluator document performance? | A. No documentation is needed until final evaluation. B. Use the training criteria, provide feedback, and note deviations or basic qualities. C. Only record aircraft serial number. D. Only record weather. | B |
| C6 | A final evaluation should be scored as: | A. DM only. B. 4, 3, 2, or 1 only. C. Satisfactory or Unsatisfactory. D. Pass only. | C |
| C7 | A rating of 1 during training indicates: | A. The pilot performed near optimum standard. B. The pilot lacks sufficient knowledge, skill, or ability and safety or outcome may be in doubt. C. The task was not evaluated. D. The task was demonstrated only. | B |
| C8 | What must occur before a pilot is assigned to unsupervised agency flight duties? | A. Completion of all applicable training and evaluation phases with required documentation. B. Purchase of a personal drone. C. Completion of one informal flight with no records. D. A verbal approval only. | A |
| C9 | Which is a proper role of a UAS Pilot Evaluator? | A. Approve missions without regard to policy. B. Conduct training or evaluation using approved task standards and document performance. C. Replace the RPIC during all missions. D. Remove all need for recurrent training. | B |
| C10 | Which is a proper role of a Final Evaluator? | A. Provide quality assurance of pilot standards and coordinate or approve final carding decisions. B. Fly every mission. C. Eliminate the UAS Program Manager role. D. Waive FAA rules. | A |
| C11 | The correct response to an unsatisfactory final evaluation is: | A. Issue the pilot card anyway. B. Deny the card or endorsement, document deficiencies, and retest at least the unsatisfactory tasks if a retest is authorized. C. Delete the record. D. Count simulator time as an automatic pass. | B |
| C12 | A pilot evaluator should adjust evaluation expectations when: | A. Conditions deviate from the ideal, such as high wind, turbulence, or poor visibility. B. The pilot wants a shorter test. C. The mission is not important. D. The aircraft is new. | A |
| C13 | Which items belong in a pilot training and qualification record? | A. FAA certificate, current-year training, flight time, aircraft qualifications, mission endorsements, evaluator records, and limitations. B. Only the pilot's phone number. C. Only the aircraft color. D. Only the mission photographs. | A |
| C14 | A VO under Part 107 or a COA must be trained or briefed to: | A. Replace the RPIC. B. Communicate hazards and help maintain separation from aircraft, obstacles, terrain, weather, and ground hazards. C. Authorize FAA waivers. D. Act as a pilot without qualification. | B |
| C15 | A task plan for DFR, ELOS/BVLOS, or non-standard operations should: | A. Use the same task-based methodology and add authorization-specific tasks. B. Avoid documentation. C. Skip emergency procedures. D. Assume all operations are allowed. | A |

## C-4 Practical Evaluation Elements: Basic Remote Pilot Task Plan

| Task | Element | Evaluator Standard | Critical Failure / Remediation Trigger |
| --- | --- | --- | --- |
| BR 01 | Mission Aviation Safety Plan | Plan identifies mission manager, objectives, dates, location, maps, hazards, aircraft, pilot, participants, flight following, emergency/SAR procedures, PPE/survival equipment, and risk assessment. | Missing authorization, missing hazards, no emergency plan, or unapproved MASP/PASP. |
| BR 10 | UAS Mission Plan | Selects appropriate aircraft, payload, data storage, transportation, ground support, replacement parts, power, and supplies. | Aircraft or payload unsuitable for mission, no data plan, or no ground support for mission duration. |
| BR 15 | Crew/Mission Brief | Briefs Big 4: approved operations plan, airspace authorization, carded pilot, carded aircraft; covers objectives, hazards, communications, emergency procedures, FRAT, assignments, and CRM. | Crew does not understand assignments, airspace, emergencies, or stop-flight triggers. |
| BR 20 | System Ground Inspection | Reviews logbooks and maintenance records; verifies aircraft data card/registration; inspects aircraft, GCS, props, antennas, batteries, payload, landing gear, controls, screen, and support systems. | Unairworthy aircraft accepted or maintenance discrepancy missed. |
| BR 25 | Radio Communication Procedures | Programs and operates radios; uses correct call sign, phraseology, airspace/incident scripts, and two-way radio failure mitigation. | Cannot reach required contact, uses wrong call sign, or fails to mitigate communications failure. |
| BR 30 | System Preflight | Verifies launch/recovery zone, support equipment, storage media, battery minimums, calibration, warnings, RTH/lost-link settings, and preflight/before takeoff checklists. | Launches with unresolved warning, wrong RTH/lost-link setting, insufficient battery, or unsafe LRZ. |
| BR 35 | Takeoff | Completes takeoff and after-takeoff checks, coordinates with crew, confirms airspace and hazards, and remains within authorization. | Takeoff creates hazard or violates authorization. |
| BR 40 | Manual Control | Maintains altitude, heading, position, speed, boundaries, operating limits, smooth inputs, and CRM. | Loss of control, boundary/airspace violation, or evaluator intervention. |
| BR 45 | Automatic Flight Mode | Plans, monitors, and manages automated flight while maintaining ability to intervene; verifies route, altitude, geofence, mission limits, and emergency controls. | Automation not monitored, route unsafe, or pilot unable to take over. |
| BR 50 | Optical Sensor Payload | Confirms sensor status, operates visual/thermal/zoom settings as required, records malfunctions, and collects only mission-required data. | Payload interferes with flight safety or data collected outside mission purpose. |
| BR 55 | Battery Management | Verifies charge, plans endurance for wind/distance/on-station time, monitors consumption, and executes alternate plan before minimums are reached. | Battery emergency caused by poor planning or delayed recovery. |
| BR 60 | Basic Maneuvers | Completes circle, vertical diamond, vertical square, figure eight, and mission-specific precision tasks with smooth accurate inputs. | Contact, uncontrolled drift, loss of situational awareness, or repeated unsafe inputs. |
| BR 65 | Night Flight | Uses lighting, night scan, VO/crew coordination, obstacle planning, altitude discipline, and emergency procedures suitable for night operations. | Loss of orientation, inadequate lighting, or failure to manage night hazards. |
| BR 70 | Emergencies | Responds to lost link, flyaway, low battery, GNSS/compass issue, crewed aircraft conflict, crash, fire, and public intrusion using checklist and communications. | Incorrect emergency action or failure to prioritize safety of people and crewed aircraft. |
| BR 75 | Landing | Completes before-landing/landing checks, clears area, controls descent, and lands safely in planned or alternate site. | Unsafe descent, landing zone conflict, contact, or loss of control. |
| BR 80 | After Landing Checks | Powers down, secures aircraft, inspects for damage, records discrepancies, manages batteries and data, and determines return-to-service status. | Damaged aircraft returned to service or data/battery mishandled. |
| BR 85 | After Action Review | Restates plan, describes what happened, identifies causes, discusses CRM and training, identifies lessons learned, and assigns follow-up items. | No AAR after safety issue or no documentation of corrective actions. |

## C-5 Practical Checkride Scenario

The evaluator should present a realistic agency mission that includes a site survey, airspace/authorization verification, mission planning, ground inspection, preflight, takeoff, manual and automated flight segments, payload use, battery management, emergency inject, landing, postflight, and AAR. The evaluator may use role reversal by acting as an ineffective crew member to test CRM, assertiveness, and stop-flight authority.

Minimum duration: enough time to observe every required task, not less than one complete flight cycle unless conducted as multiple short flights.

Required injects: one communications challenge, one payload or automation issue, one airspace or crewed-aircraft scenario, and one emergency or abnormal event.

Passing standard: all required tasks are satisfactory, no critical safety/legal/authorization failure, no evaluator intervention required to prevent unsafe outcome.

Retest standard: at minimum, all unsatisfactory tasks must be reevaluated. Additional tasks may be retested if the evaluator believes the deficiency affects broader performance.

## C-6 Pilot Evaluator / Final Evaluator Written Test Bank

| # | Question | Expected Answer | Format |
| --- | --- | --- | --- |
| PE1 | What is the purpose of PE 165 Develop Evaluation Plan? | To ensure the evaluator selects appropriate tasks, confirms policy requirements, prepares materials, and builds an event-specific evaluation plan. | Open response |
| PE2 | Why must a training records review occur before a flight evaluation? | To verify prerequisites, qualifications, currency, limitations, and whether the pilot may be evaluated for the aircraft or mission. | Open response |
| PE3 | What is the difference between teaching a task and evaluating a task? | Teaching introduces and develops performance with feedback. Evaluation measures performance against the established standard and documents the result. | Open response |
| PE4 | What must the evaluator do when a pilot fails a task because of a safety-critical error? | Stop or control the event as needed, document the error, debrief, identify remediation, and prevent carding or endorsement until retest is satisfactory. | Open response |
| PE5 | What does a Final Evaluator add to the program? | Quality assurance, standardization, carding/endorsement oversight, record review, evaluator coordination, and final qualification authority. | Open response |

## C-7 Pilot Evaluator Practical Elements

| Task | Practical Element |
| --- | --- |
| PE 165 | Develop an evaluation plan for a routine inspection mission and a separate DFR/BVLOS mission. Include tasks, knowledge topics, required records, aircraft/simulator/classroom mode, and safety controls. |
| PE 170 | Teach a short lesson on airspace authorization, lost-link, or battery management using approved materials and adjust instruction based on student questions. |
| PE 175 | Administer an oral or written knowledge check, explain criteria, score consistently, debrief, and record results. |
| PE 180 | Demonstrate and coach at least two remote pilot tasks using the DM/4/3/2/1 rating system and basic qualities terminology. |
| PE 185 | Evaluate remote pilot tasks, compare observed performance to standards, identify deviations, assign S/U where appropriate, and complete remarks. |
| PE 190 | Administer a final checkride with scenario injects that test risk management, operational decision making, ADM, CRM, and unexpected events. |

## C-8 Addendum C Answer Key

Written test answers: C1-B, C2-B, C3-B, C4-B, C5-B, C6-C, C7-B, C8-A, C9-B, C10-A, C11-B, C12-A, C13-A, C14-B, C15-A.

# Addendum D: DFR, ELOS/BVLOS, Remote Operations, and Non-Standard Operations Evaluation Plans

This addendum uses the same task-based method for DFR, ELOS/BVLOS, remote launch, and non-standard operating procedures. The source certification standards identify ELOS/BVLOS and NSOP as under development, so this addendum provides a generic Oregon public agency framework that must be finalized against the agency's actual FAA authorization and local policy before use.

## D-1 DFR / Remote Operations Task Plan

| Task # | Task Title | Training Mode | Evaluation Mode |
| --- | --- | --- | --- |
| DFR 01 | Verify FAA authorization and agency dispatch criteria | Classroom / Simulator | Classroom / Simulator |
| DFR 05 | Conduct remote/dock system health check | Simulator / Aircraft | Simulator / Aircraft |
| DFR 10 | Complete DFR launch decision under time pressure | Simulator | Simulator |
| DFR 15 | Coordinate with dispatch, RTIC/RTCC, field units, and airspace monitor | Simulator | Simulator / Aircraft |
| DFR 20 | Manage live video, recording, data minimization, and feed sharing | Simulator | Simulator |
| DFR 25 | Operate remote aircraft within geofence, route, altitude, and authorization limits | Simulator / Aircraft | Simulator / Aircraft |
| DFR 30 | Respond to crewed aircraft, lost link, loss of video, geofence, and battery abnormal events | Simulator / Aircraft | Simulator / Aircraft |
| DFR 35 | Terminate, land, or hand off mission and complete logs | Simulator / Aircraft | Simulator / Aircraft |

## D-2 ELOS/BVLOS Task Plan

| Task # | Task Title | Training Mode | Evaluation Mode |
| --- | --- | --- | --- |
| BVLOS 01 | Review waiver/COA/authorization limits, area, altitude, visibility, detect-and-avoid, shielding, and reporting conditions | Classroom | Classroom |
| BVLOS 05 | Prepare route, lost-link route, emergency landing sites, contingency volumes, and recovery plan | Classroom / Simulator | Classroom / Simulator |
| BVLOS 10 | Confirm airspace monitoring or VO configuration and crewed-aircraft priority | Classroom / Simulator | Classroom / Simulator |
| BVLOS 15 | Execute remote or extended operation within command-and-control, telemetry, and geofence limits | Simulator / Aircraft | Simulator / Aircraft |
| BVLOS 20 | Respond to detect-and-avoid alert, non-cooperative aircraft, lost C2, lost video, GNSS anomaly, and land-now instruction | Simulator / Aircraft | Simulator / Aircraft |
| BVLOS 25 | Complete authorization-specific logs, incident reports, and postflight review | Classroom | Classroom |

## D-3 Non-Standard Operating Procedure Task Plan

| Task # | Task Title |
| --- | --- |
| NSOP 01 | Define the non-standard procedure, aircraft, payload, environment, risk, and legal authority. |
| NSOP 05 | Obtain UAS Program Manager approval and safety/legal review when needed. |
| NSOP 10 | Develop procedure-specific checklist, training plan, emergency plan, and evaluation standard. |
| NSOP 15 | Conduct demonstration and training event under controlled conditions. |
| NSOP 20 | Conduct final evaluation with evaluator and document endorsement, limitations, and recurrent requirements. |

## D-4 DFR/BVLOS Written Test Questions

| # | Question | Expected Answer | Format |
| --- | --- | --- | --- |
| D1 | A DFR call is eligible under dispatch policy, but the dock weather sensor shows winds beyond aircraft limits. What is the correct action? | Do not launch. Document the reason and notify dispatch/RTIC/RTCC or supervisor as required. | Open response |
| D2 | A BVLOS authorization requires operations below a specific altitude and within a defined area. A field unit asks for a higher view outside the area. What should the pilot do? | Decline or modify the request to remain within authorization. Do not exceed the authorized area or altitude without additional FAA authority. | Open response |
| D3 | During a DFR mission, a medevac helicopter is reported inbound. What is the required priority? | Crewed aircraft has priority. Land, hold, or move as required by the deconfliction plan and notify the appropriate contacts. | Open response |
| D4 | What must be tested during a remote/dock practical before independent DFR duty? | Remote preflight, dock health check, launch/no-launch decision, telemetry, Remote ID, video, lost-link, airspace check, communications, emergency landing, data handling, and logging. | Open response |
| D5 | Why should ELOS/BVLOS training be authorization-specific? | Because each waiver, COA, or authorization has different airspace, altitude, detect-and-avoid, crew, reporting, weather, and termination conditions. | Open response |

## D-5 DFR/BVLOS Practical Critical-Failure Items

Launch without verifying FAA authorization, weather, aircraft status, Remote ID, and airspace.

Operate outside waiver/COA/geofence/altitude/route limits.

Fail to land, hold, or deconflict when crewed aircraft enters or is expected to enter the area.

Continue flight after lost C2, lost video, aircraft warning, or dock system failure when policy requires termination.

Share live video or recorded data with an unauthorized person or for an unauthorized purpose.

Fail to document mission, incident, abnormal event, or postflight status required by the authorization or agency policy.

Source Integration Note

This document was updated to incorporate the May 2026 Certification Standards for Remote Pilot Operations model, including its task-based carding concept, special emphasis areas, evaluator roles, four-phase evaluation methodology, Master Task List, Basic Remote Pilot task plan, Type 3/Type 4 aircraft task plan, pilot evaluator task plan, and current-year training and qualification record concepts.

Part IV. Supplemental Public-Safety DFR Test Bank and Scenario Practicals

Additional comprehensive knowledge bank and practical standards drawn from the DFR/Public Safety UAS training, testing, and qualification program.

Use this supplemental bank when building larger exams, public-safety endorsement exams, refresher tests, oral boards, or remedial training. It overlaps with the core bank intentionally so the Training Officer can randomize exams and select role-specific questions.

# 10. Knowledge Examination Program

The knowledge examination program uses question banks mapped to training objectives. The agency may administer a comprehensive initial exam or a core exam plus mission endorsement exams. The recommended initial RPIC or DFR operator exam is 50 questions drawn from the bank, with at least ten legal/authority questions, ten safety/risk/airspace questions, ten operations/emergency questions, five data/privacy questions, and the remainder selected from the candidate's role. Mission endorsement exams should include 20 to 30 questions specific to fire/EMS/SAR, law enforcement, DFR/BVLOS, or airspace coordination.

A trainee passes a standard written exam with a minimum score of 80 percent if all critical questions are answered correctly. Critical questions should cover RPIC authority, crewed aircraft priority, airspace authorization, no-launch/no-fly rules, law-enforcement legal authority, data handling, lost link, battery fire, and emergency reporting. A trainee who misses a critical question must receive remediation and re-test on the missed objective before qualification.

The question bank below is intended for agency customization. It includes more questions than are needed for a single exam so that the Training Officer can randomize tests, build refresher exams, and create role-specific endorsement exams.

# 11. Knowledge Test Question Bank

Question 1. What is the primary purpose of this training program?

A. To replace FAA certification for public employees.

B. To teach and verify agency-specific UAS competence in addition to FAA and policy requirements.

C. To authorize recreational drone flights by employees.

D. To eliminate the need for written mission planning.

Question 2. Who has final authority over the safe conduct of a small UAS flight?

A. The mission requester.

B. The public information officer.

C. The Remote Pilot in Command.

D. The person receiving the live video feed.

Question 3. Before a multiagency UAS mission begins, the participating agencies must identify:

A. The newest aircraft in the fleet.

B. Which entity has operational control and who may task the aircraft.

C. The most experienced firefighter on scene.

D. The media release date.

Question 4. A government mission is automatically exempt from FAA UAS rules.

A. True.

B. False.

C. True only if the agency owns the aircraft.

D. True only during daylight.

Question 5. Which operating authority is the usual baseline for routine small UAS public agency missions under 55 pounds when no COA or waiver is used?

A. 14 CFR Part 107.

B. 49 CFR Part 830.

C. The agency social media policy.

D. A park reservation permit.

Question 6. A public aircraft COA may be used only when:

A. The agency wants to avoid training.

B. A qualifying governmental operation and FAA authorization support that pathway.

C. A contractor is unavailable.

D. The aircraft has a camera.

Question 7. Which statement best describes an SGI or emergency authorization request?

A. It is a convenience tool for routine missions.

B. It is an expedited FAA pathway for urgent public-interest operations that cannot reasonably wait for normal processing.

C. It replaces incident command.

D. It removes the need to document the mission.

Question 8. Remote ID compliance should be checked:

A. Only at purchase.

B. Only if a member of the public asks.

C. Before flight when the aircraft is required to comply.

D. Only after a crash.

Question 9. A Visual Observer's primary role is to:

A. Approve law-enforcement legal authority.

B. Assist the RPIC with airspace, obstacle, ground hazard, and mission awareness.

C. Edit video for public release.

D. Repair batteries during flight.

Question 10. The Check 3 GPS field habit means:

A. Gear, Plan, Skills.

B. GPS, Propellers, Software.

C. Ground, Police, Supervisor.

D. Geofence, Payload, Storage.

Question 11. Which of the following is a no-launch condition?

A. The mission requester is impatient.

B. Crewed aircraft deconfliction cannot be assured.

C. The aircraft has a fresh memory card.

D. The VO is wearing a safety vest.

Question 12. A PASP should include:

A. Only the pilot's name.

B. Mission objective, crew, aircraft, airspace, hazards, mitigations, data plan, and emergency procedures.

C. Only weather after the flight.

D. The social media caption.

Question 13. The purpose of a site survey is to:

A. Find the best scenic view.

B. Identify launch/recovery areas, obstacles, public access, emergency landing zones, and hazards.

C. Avoid documenting the mission.

D. Replace airspace review.

Question 14. Which human factor is a common UAS risk?

A. Task fixation and pressure to complete the mission.

B. Too many labeled batteries.

C. Writing a debrief.

D. Checking the weather twice.

Question 15. The sterile cockpit concept means:

A. No talking at all during the mission.

B. Only necessary communications during high workload phases such as launch, landing, emergency, or obstacle work.

C. Only the supervisor may speak.

D. The aircraft must be disinfected.

Question 16. When a crewed aircraft is observed near the operating area, the UAS crew should:

A. Continue unless the mission is almost complete.

B. Yield right-of-way and descend, hold, or land as needed to maintain separation.

C. Climb above the crewed aircraft.

D. Ignore it if the VO saw it first.

Question 17. If the RPIC loses command link and the aircraft does not behave as expected, the RPIC should:

A. Wait until the public calls.

B. Follow the lost-link plan, notify appropriate contacts when needed, and document the event.

C. Delete the flight logs.

D. Start a new mission request.

Question 18. A battery that is swollen, overheated, damaged, or questionable should be:

A. Used only for training.

B. Used at low altitude.

C. Removed from service until evaluated under agency procedure.

D. Charged faster.

Question 19. Firmware updates should be:

A. Installed immediately before every mission without review.

B. Reviewed, documented, and tested when they may affect flight behavior or critical systems.

C. Avoided forever.

D. Done only by the mission requester.

Question 20. UAS data collected for a public agency mission may be:

A. Automatically private personal property.

B. A public record, evidence, incident record, or protected/sensitive record depending on context.

C. Deleted by any pilot after landing.

D. Shared freely on personal devices.

Question 21. The best practice for data collection is to:

A. Collect as much as possible in case it becomes useful.

B. Collect only what is reasonably necessary for the approved mission.

C. Record every nearby home for transparency.

D. Avoid documenting the data objective.

Question 22. Public records requests for UAS data should be routed to:

A. The agency public records process or records officer.

B. The individual pilot's personal email.

C. The aircraft manufacturer.

D. Any person who asks first.

Question 23. Which event may require urgent internal and external reporting?

A. A completed routine landing.

B. Flyaway, collision, injury, battery fire, or qualifying FAA/NTSB event.

C. A training reminder.

D. A charged battery.

Question 24. After a crash or hard landing, the aircraft should:

A. Return to service immediately if it looks fine.

B. Be inspected and released under maintenance/return-to-service procedures before operational use.

C. Be hidden until the next review.

D. Be used only at night.

Question 25. What is the purpose of an after-action review?

A. To assign blame automatically.

B. To identify lessons, corrective actions, and improvements.

C. To replace training records.

D. To avoid reporting hazards.

Question 26. A DFR launch should be based on:

A. Curiosity about a call.

B. Approved call type, lawful authority, airspace, launch-site readiness, mission value, and safety.

C. The availability of a live stream.

D. The popularity of the neighborhood.

Question 27. A DFR remote operator should decline or stop launch when:

A. The launch site cannot be confirmed clear.

B. The incident is nearby.

C. The battery is full.

D. The weather display is working.

Question 28. Under a COA or waiver, the operator must:

A. Follow only the parts that are convenient.

B. Follow the specific conditions and limitations of the authorization.

C. Ignore normal safety practices.

D. Operate at any altitude within the city.

Question 29. BVLOS authorization means:

A. The aircraft may fly anywhere without deconfliction.

B. The operation must follow the FAA-approved detect-and-avoid, communications, airspace, and contingency procedures.

C. Visual observers are prohibited.

D. Remote ID is never required.

Question 30. Which is the correct priority in a DFR live video workflow?

A. Maximize viewers.

B. Limit access to mission need and agency policy.

C. Share the stream on personal devices.

D. Keep no record of viewers.

Question 31. For fire/EMS operations, UAS crews must avoid:

A. Supporting size-up.

B. Interfering with suppression, medevac, responder safety, or patient privacy.

C. Thermal assessment when authorized.

D. Post-incident documentation when assigned.

Question 32. At a structure fire, a thermal camera may:

A. Always show exact interior conditions.

B. Provide useful surface temperature information but has limitations and can be misleading.

C. Replace firefighter reports.

D. Eliminate the need for command.

Question 33. During a search and rescue mission, a UAS target sighting should be reported with:

A. A vague description only.

B. Location, time, sensor view, confidence, and any coordinate or landmark reference.

C. A public social media post.

D. No documentation until the mission ends.

Question 34. Wildfire UAS operations should not launch unless:

A. The UAS crew can see smoke.

B. The operation is assigned, authorized, and deconflicted with fire aviation and incident procedures.

C. The pilot has flown recreationally near forests.

D. The launch area is outside city limits.

Question 35. Unauthorized UAS activity near an incident should be:

A. Pursued by agency UAS without authorization.

B. Reported through incident, dispatch, aviation, or supervisory channels with observable details.

C. Ignored.

D. Disabled by any responder.

Question 36. A law-enforcement UAS mission should begin with:

A. A general desire to see what is happening.

B. A documented lawful pathway, mission scope, and data handling plan.

C. A live stream link.

D. A verbal rumor.

Question 37. A non-law-enforcement agency supporting law enforcement should:

A. Make independent legal decisions in the field.

B. Operate under counsel-reviewed procedures or documented emergency authority.

C. Use law-enforcement data however it chooses.

D. Avoid documenting the requesting agency.

Question 38. Training data collected during law-enforcement UAS training should be:

A. Handled according to policy and not used as case evidence unless lawfully authorized.

B. Automatically used to establish probable cause.

C. Posted publicly.

D. Mixed with evidence.

Question 39. Evidence video should be transferred to:

A. An approved evidence or records system with custody documentation.

B. A personal cloud folder.

C. A text message thread.

D. The first available USB drive with no log.

Question 40. A tactical UAS mission should be coordinated with:

A. The tactical commander and relevant incident or aviation coordination roles.

B. Only the person who bought the aircraft.

C. No one if the pilot is skilled.

D. The public before launch.

Question 41. General surveillance of homes, people, or protected activity unrelated to an approved mission is:

A. Prohibited unless specifically authorized by law and policy.

B. Always allowed in public airspace.

C. Required for DFR.

D. Allowed if the flight is short.

Question 42. Mutual-aid UAS response should occur only after:

A. A pilot hears about the event on the radio.

B. Authorized request/tasking through mutual aid, incident command, dispatch, emergency management, or agreement.

C. The pilot charges batteries.

D. A news crew arrives.

Question 43. In multiagency airspace, a single airspace coordination point helps:

A. Create confusion.

B. Separate aircraft, assign altitudes/sectors, and maintain the airspace picture.

C. Avoid records.

D. Replace the RPIC.

Question 44. Crewed aircraft priority means:

A. UAS may continue if below 100 feet.

B. UAS must yield and land/hold/avoid as necessary.

C. The larger agency decides.

D. Only helicopters count.

Question 45. Altitude blocks and sectors are used to:

A. Make video more cinematic.

B. Reduce conflict among multiple UAS and preserve clear operating areas.

C. Avoid communication.

D. Increase battery use.

Question 46. The UAS Airspace Coordinator should track:

A. Only agency-owned drones.

B. UAS identifiers, locations, altitudes, mission areas, crewed aircraft hazards, lost-link plans, and landing status.

C. Only law-enforcement flights.

D. Only weather after the incident.

Question 47. If deconfliction cannot be assured during a multiagency response:

A. Continue and rely on luck.

B. Suspend, hold, or land UAS operations until coordination is restored.

C. Climb higher.

D. Launch a second aircraft to look for traffic.

Question 48. The best description of CRM/TEM is:

A. A records software product.

B. Crew communication and threat/error management to prevent small issues from becoming unsafe conditions.

C. A battery charging mode.

D. A method for deleting data.

Question 49. An evaluator should fail a practical test immediately when the trainee:

A. Asks a clarifying question.

B. Creates an unsafe condition, disregards a safety instruction, or violates a critical rule.

C. Takes extra time to brief.

D. Uses a conservative landing approach.

Question 50. NIST-style lanes are useful because they:

A. Certify a pilot nationally.

B. Provide repeatable objective tasks to measure aircraft and operator proficiency.

C. Replace FAA rules.

D. Eliminate judgment testing.

Question 51. NIST does not certify public safety UAS pilots; agencies use NIST-style methods to set their own thresholds.

A. True.

B. False.

C. True only for law enforcement.

D. False if the pilot passes a written test.

Question 52. A practical evaluation should test:

A. Only smooth video.

B. Planning, legal authority, site setup, crew coordination, aircraft control, emergency response, data handling, and judgment.

C. Only takeoff and landing.

D. Only speed.

Question 53. The purpose of supervised operational flights is to:

A. Let trainees skip the checkride.

B. Bridge training and real missions under a mentor before unsupervised deployment.

C. Avoid training records.

D. Use up batteries.

Question 54. A trainee who fails a critical item should:

A. Be passed if the total score is high.

B. Receive documented remediation and re-test before qualification.

C. Delete the failed test.

D. Teach the next class.

Question 55. Which is a proper role for a payload operator?

A. Operate sensors under mission scope and RPIC direction.

B. Take over legal approval.

C. Ignore data limits.

D. Distract the VO with unrelated tasks.

Question 56. A VO traffic call should be:

A. Long and uncertain.

B. Clear, concise, directional, and acknowledged by the RPIC.

C. Made only after the flight.

D. Sent by email.

Question 57. An emergency landing zone should be identified:

A. Only after low battery.

B. Before launch when feasible.

C. Only for night operations.

D. Only for law enforcement.

Question 58. Return-to-home altitude should be set:

A. Randomly.

B. With regard to obstacles, authorization limits, lost-link path, and mission area.

C. As high as possible.

D. At zero.

Question 59. If a member of the public enters the launch/recovery area, the crew should:

A. Ignore the person.

B. Pause launch or abort/modify landing as needed and clear the area safely.

C. Yell while the RPIC continues landing.

D. Move the aircraft toward the person.

Question 60. Which data practice is not acceptable?

A. Using approved agency storage.

B. Uploading agency mission video to an unapproved personal cloud account.

C. Logging data transfer.

D. Restricting access by role.

Question 61. The Training Officer should maintain records of:

A. Only certificates.

B. Certificates, tests, practicals, flight currency, endorsements, remediation, and expiration dates.

C. Only failed tests.

D. Only aircraft serial numbers.

Question 62. An agency may adopt stricter standards than FAA minimums for its own personnel.

A. True.

B. False.

C. True only for contractors.

D. False if Part 107 applies.

Question 63. A qualified remote pilot who is fatigued or impaired should:

A. Self-ground and notify the appropriate supervisor or RPIC.

B. Fly only low altitude.

C. Use a VO as a substitute.

D. Fly if the mission is urgent.

Question 64. Which source is best for local law-enforcement UAS authority?

A. A general internet forum.

B. Agency counsel and applicable statutes, policy, warrants, court orders, and prosecutor guidance.

C. Aircraft marketing material.

D. The camera manual.

Question 65. For crash reconstruction, a law-enforcement UAS crew should:

A. Collect without case documentation.

B. Follow approved legal, evidence, scene safety, and data procedures.

C. Delete raw images after the map is built.

D. Publish the orthomosaic immediately.

Question 66. A fire/EMS public-information image should be released:

A. By any pilot who captured it.

B. Through the agency public information and records process after privacy and safety review.

C. Only by the manufacturer.

D. Automatically with all raw footage.

Question 67. During hazmat support, the UAS crew should consider:

A. Contamination, standoff distance, wind, sensor limits, responder safety, and decontamination plan.

B. Only video quality.

C. Only social media value.

D. Only battery color.

Question 68. In SAR operations, thermal sensors are affected by:

A. Vegetation, terrain, water, sun loading, weather, distance, angle, and target behavior.

B. Nothing.

C. Only pilot certification.

D. Only Remote ID.

Question 69. During a medevac operation, UAS should:

A. Remain clear and deconflicted or land as directed; crewed medical aircraft have priority.

B. Orbit above the landing zone.

C. Continue because the drone is small.

D. Climb to film the helicopter.

Question 70. What should be included in a DFR postflight entry?

A. Launch time, incident/call identifier, purpose, authority, aircraft, crew, flight time, data disposition, issues, and any deviations.

B. Only whether the video was interesting.

C. No record if nothing happened.

D. Only battery percentage.

Question 71. A public agency should use privately owned UAS for agency business only when:

A. Any employee volunteers.

B. Specifically authorized in writing and registration, insurance, maintenance, data, and risk requirements are satisfied.

C. The agency aircraft is charging.

D. The aircraft is less than 250 grams.

Question 72. Operations over people or moving vehicles require:

A. No special consideration if brief.

B. Compliance with Part 107 categories, waiver, COA, or other authorization and agency approval.

C. Only a VO.

D. Only a warning sign.

Question 73. Controlled airspace operations require:

A. No authorization below 400 feet.

B. LAANC, manual authorization, COA coverage, or another FAA-recognized authorization as applicable.

C. Only local police approval.

D. Only a phone call to a friend.

Question 74. A TFR is:

A. A training flight record.

B. A Temporary Flight Restriction that may limit or prohibit UAS operations unless authorized.

C. A thermal frequency range.

D. A tactical fire report.

Question 75. Mission creep occurs when:

A. The aircraft returns home.

B. The crew expands collection beyond the approved mission purpose without authority.

C. The VO scans carefully.

D. A battery cools.

Question 76. The correct response to mission creep is to:

A. Continue because the aircraft is already airborne.

B. Stop or seek proper approval before expanding scope.

C. Delete the extra data silently.

D. Ask a bystander.

Question 77. The agency's annual review should examine:

A. Only successful flights.

B. Flight activity, training, currency, incidents, near misses, complaints, maintenance, data, legal updates, and corrective actions.

C. Only battery cycle counts.

D. Only social media metrics.

Question 78. A tabletop exercise is useful because it:

A. Allows policy and decision-making practice without flight risk.

B. Replaces all live flight training.

C. Eliminates documentation.

D. Is only for chiefs.

Question 79. A functional exercise should include:

A. Players performing roles in a simulated operational environment with evaluation and after-action review.

B. Only a lecture.

C. Only equipment inventory.

D. Only lunch.

Question 80. The HSEEP improvement plan concept means:

A. After-action findings should be assigned to corrective actions with owners and deadlines.

B. Exercises are for entertainment.

C. No records are needed.

D. Only failures are documented.

Question 81. An instructor may adjust the training scenario during a practical when:

A. Needed to preserve safety or evaluate an objective fairly.

B. The trainee is doing too well.

C. The evaluator wants a surprise unrelated to training.

D. The agency wants to avoid scoring.

Question 82. What is a critical failure in a practical?

A. A minor spelling error in a log.

B. An unsafe act, regulatory violation, critical policy violation, loss of aircraft control, or failure to yield to crewed aircraft.

C. A conservative go/no-go decision.

D. A request for clarification.

Question 83. If the trainee makes a safe no-go decision during evaluation, the evaluator should:

A. Fail the trainee automatically.

B. Credit the decision and continue with an alternate scenario or oral evaluation as appropriate.

C. Force launch.

D. Delete the practical.

Question 84. A law-enforcement DFR flight that begins as a public safety response and becomes investigative should:

A. Continue without change.

B. Pause for supervisor/legal pathway review when required and document the transition.

C. Delete earlier data.

D. Transfer control to any viewer.

Question 85. A UAS crew supporting another agency should not release data directly unless:

A. The release is authorized by the applicable records, evidence, incident command, and interagency procedures.

B. A requester is friendly.

C. The file is small.

D. The aircraft has landed.

Question 86. Airspace deconfliction during mutual aid should be documented:

A. Only if there is a crash.

B. In mission logs, PASP, incident action plan, airspace log, or equivalent record.

C. Only on a sticky note.

D. Never.

Question 87. A good lost-link plan should include:

A. Expected aircraft behavior, RTH altitude/path, contingency landing, notification, and recovery procedures.

B. Only battery percentage.

C. A decision to wait.

D. No settings review.

Question 88. A DFR aircraft operating from a dock should not land when:

A. The dock camera shows intrusion or the landing area cannot be verified clear.

B. The battery is above reserve.

C. The RPIC sees the landing pad on video.

D. The mission is over.

Question 89. When DFR live video shows a medical patient, the crew should:

A. Limit access and handle the data as sensitive public safety information.

B. Share it broadly for awareness.

C. Take screenshots on personal phones.

D. Use it for training without review.

Question 90. Which training record is most defensible?

A. A date and the word 'done'.

B. Objectives taught, instructor, trainee, test score, practical elements, errors, remediation, qualification decision, and expiration.

C. Only attendance.

D. Only the pilot's signature.

Question 91. Currency requirements are intended to:

A. Punish pilots.

B. Ensure recent practice and continued readiness for assigned aircraft and missions.

C. Replace recurrent knowledge.

D. Avoid audits.

Question 92. A person may serve as RPIC under direct supervision for a training event only when:

A. The certificated RPIC can immediately take direct control and the operation is otherwise lawful and approved.

B. The trainee asks nicely.

C. The aircraft is small.

D. There is no VO.

Question 93. An agency's public safety annex training should be adopted by:

A. Any agency, even with no public safety role.

B. Agencies that actually conduct public safety, emergency response, fire, SAR, disaster, or incident support missions.

C. Only vendors.

D. Only recreational clubs.

Question 94. An agency's law-enforcement annex training should be adopted by:

A. Only a law-enforcement agency or agency providing law-enforcement support under counsel-reviewed procedures.

B. Any public works agency for routine inspections.

C. Any person with a camera.

D. All trainees regardless of mission.

Question 95. What is the correct response to a supervisor pressuring the RPIC to fly in unsafe conditions?

A. Fly because the supervisor outranks the pilot.

B. Explain the risk and decline or delay; RPIC safety authority controls the flight decision.

C. Transfer the flight to the VO.

D. Launch quickly before conditions worsen.

Question 96. What should an RPIC do if weather deteriorates toward aircraft or authorization limits?

A. Continue until limits are exceeded.

B. Return, land, or modify mission before limits are exceeded.

C. Ask the aircraft to auto-decide.

D. Ignore weather if GPS is good.

Question 97. What does a mission endorsement prove?

A. That the person passed a role-specific training and evaluation for a higher-risk or specialized mission.

B. That the person never needs refresher training.

C. That the agency accepts all legal risk.

D. That the person can train others automatically.

Question 98. Why should training include both oral scenarios and practical flight?

A. Because written tests alone do not verify judgment and performance under workload.

B. Because oral questions replace flight logs.

C. Because practicals are optional entertainment.

D. Because scenarios make FAA rules unnecessary.

# 12. Knowledge Test Answer Key

The Training Officer should control the answer key as an examination-security record. Instructors may use the explanation field during remediation, but trainees should not receive the keyed bank before testing unless the agency intentionally uses it as a study guide.

| 1: B | 2: C | 3: B | 4: B | 5: A |
| --- | --- | --- | --- | --- |
| 6: B | 7: B | 8: C | 9: B | 10: A |
| 11: B | 12: B | 13: B | 14: A | 15: B |
| 16: B | 17: B | 18: C | 19: B | 20: B |
| 21: B | 22: A | 23: B | 24: B | 25: B |
| 26: B | 27: A | 28: B | 29: B | 30: B |
| 31: B | 32: B | 33: B | 34: B | 35: B |
| 36: B | 37: B | 38: A | 39: A | 40: A |
| 41: A | 42: B | 43: B | 44: B | 45: B |
| 46: B | 47: B | 48: B | 49: B | 50: B |
| 51: A | 52: B | 53: B | 54: B | 55: A |
| 56: B | 57: B | 58: B | 59: B | 60: B |
| 61: B | 62: A | 63: A | 64: B | 65: B |
| 66: B | 67: A | 68: A | 69: A | 70: A |
| 71: B | 72: B | 73: B | 74: B | 75: B |
| 76: B | 77: B | 78: A | 79: A | 80: A |
| 81: A | 82: B | 83: B | 84: B | 85: A |
| 86: B | 87: A | 88: A | 89: A | 90: B |
| 91: B | 92: A | 93: B | 94: A | 95: B |
| 96: B | 97: A | 98: A |  |  |

# 13. Practical Evaluation Program

The practical evaluation program verifies that trainees can perform the work, not merely describe it. A written exam tests knowledge. A practical test confirms that the trainee can plan, brief, fly, coordinate, respond to abnormal conditions, handle data, and document the mission while under realistic workload. Practical testing should be conducted in a controlled environment first and in supervised operational conditions only after the trainee has shown basic proficiency.

The evaluator should use scenario-based tasks, NIST-style repeatable lanes, oral questioning, and observed crew performance. The evaluator may stop the test at any time for safety. The evaluator should not create hidden hazards that would be unfair or unsafe, but may inject realistic changes such as a crewed aircraft call, public intrusion, data classification issue, lost-link warning, dispatch update, or weather change.

## 13. Practical Test Passing Rules

A trainee passes only if all critical items are satisfactory and the score meets or exceeds the agency pass threshold. Critical items include RPIC authority, airspace compliance, crewed aircraft right-of-way, safe aircraft control, preflight condition for safe operation, launch/recovery area safety, emergency procedure execution, data/legal compliance, and truthful documentation. A trainee who fails a critical item fails the practical even if the numerical score is otherwise passing.

| Evaluation Type | Recommended Pass Standard | Critical Failure Rule |
| --- | --- | --- |
| Core RPIC practical | Minimum 85 of 100 points. | Any critical unsafe/legal failure is a fail. |
| Visual Observer practical | Satisfactory on all required calls and scanning tasks. | Failure to call traffic, obstacle, ground intrusion, or lost aircraft is a fail if safety is affected. |
| Payload/Data practical | Minimum 85 percent and correct data pathway. | Unauthorized data release or mission-scope violation is a fail. |
| DFR remote operations practical | Minimum 90 percent due to remote launch risk. | Launch without confirmed site/airspace/legal readiness is a fail. |
| Fire/EMS/SAR practical | Minimum 85 percent and safe incident integration. | Interference with crewed aircraft, responders, patient privacy, or ICS authority is a fail. |
| Law enforcement practical | Minimum 90 percent and correct legal pathway. | Unsupported legal authority, evidence mishandling, or prohibited surveillance is a fail. |
| Airspace coordinator practical | Satisfactory common airspace picture and conflict resolution. | Failure to suspend operations when deconfliction is lost is a fail. |

# 14. Practical Test Elements and Scoring Rubrics

## 14.1 Core RPIC Practical Checkride

The core RPIC practical should be completed on each aircraft family the pilot will operate. The evaluator may use a live aircraft, a simulator for selected abnormal events, and oral scenarios. The flight portion should include normal manual control, positioning, precision movement, target orientation, emergency response, and postflight documentation. NIST-style lanes may be used to measure repeatable accuracy, orientation, target inspection, and maneuver discipline.

| Element | Points | Performance Standard |
| --- | --- | --- |
| Mission legality and authorization | 10 | Identifies operating authority, pilot status, aircraft registration, Remote ID, airspace, NOTAM/TFR, local constraints, and any waiver/COA conditions. |
| PASP, site survey, and risk assessment | 15 | Completes mission plan, identifies hazards, applies mitigations, sets launch/recovery and emergency landing zones, and makes a defensible go/no-go decision. |
| Crew briefing and CRM/TEM | 10 | Assigns roles, briefs communications, mission limits, lost link, crewed aircraft response, public intrusion, data plan, and stop-flight authority. |
| Preflight inspection and configuration | 15 | Checks aircraft, propellers, motors, payload, controller, batteries, Remote ID, GNSS/compass, RTH altitude/path, geofence, memory, and documents condition for safe operation. |
| Launch, aircraft control, and mission task | 20 | Performs stable launch, maintains boundaries, conducts designated maneuvers, responds to VO calls, and operates smoothly without unsafe drift or task fixation. |
| NIST-style proficiency task | 10 | Completes selected lane, box, orbit, alignment, target-identification, or inspection task within agency tolerances. |
| Emergency/abnormal procedure | 10 | Responds correctly to one evaluator-injected abnormal condition such as crewed aircraft, low battery, lost video, public intrusion, or lost-link warning. |
| Landing, postflight, logs, and data | 10 | Lands safely, inspects aircraft, documents flight/battery/maintenance/data, debriefs lessons, and reports hazards or deviations. |

Core RPIC practical evaluator checklist:

[ ] 1. Confirm the trainee is medically and mentally fit for flight and has required documents.

[ ] 2. Confirm the evaluation aircraft is in training status and has no unresolved maintenance discrepancy.

[ ] 3. Confirm the evaluator has briefed safety stop words and emergency authority.

[ ] 4. Confirm weather, airspace, launch area, and bystander controls are suitable.

[ ] 5. Confirm the trainee completes Check Gear, Check Plan, and Check Skills before launch.

[ ] 6. Confirm a safe launch, hover check, flight control check, mission maneuver, and recovery.

[ ] 7. Inject one abnormal condition that can be handled safely.

[ ] 8. Stop the test immediately for unsafe behavior, uncontrolled flight, unbriefed overflight of people, crewed aircraft conflict, disregard of evaluator instruction, or legal/policy violation.

[ ] 9. Complete scoring, oral review, and training record before qualification decision.

## 14.2 Visual Observer Practical

The Visual Observer practical verifies that the VO can assist with airspace and ground safety rather than passively watching the screen. The trainee must demonstrate scanning, concise calls, acknowledgement discipline, obstacle and ground hazard observation, public intrusion management, and emergency support.

| Task | Satisfactory Performance |
| --- | --- |
| Positioning | Chooses or accepts a VO position that supports visual scan, communication, and mission boundaries. |
| Traffic calls | Uses plain calls such as 'Aircraft, north, low, moving east, recommend hold or land' and obtains RPIC acknowledgement. |
| Obstacle calls | Identifies wires, towers, trees, terrain, vehicles, people, animals, and launch/recovery intrusions. |
| Lost aircraft or visual uncertainty | Immediately tells RPIC when visual contact or orientation is uncertain. |
| Emergency support | Assists with lost link, landing zone clearing, public safety perimeter, and airspace notification as assigned. |
| Distraction control | Avoids unrelated screen viewing, phone use, or conversation during critical phases. |

## 14.3 Payload, Sensor, and Data Practical

The payload practical verifies that the operator can collect mission-authorized data while protecting privacy, evidence, public records, and cybersecurity. The evaluator should require the trainee to select settings, capture a target or inspection image, avoid unnecessary collection, transfer data to approved storage, and complete a data record.

| Task | Satisfactory Performance |
| --- | --- |
| Mission scope | States what data is authorized and what data should be avoided. |
| Sensor setup | Configures visual/thermal/zoom/mapping sensor for mission objective and records settings when needed. |
| Collection discipline | Keeps camera angle, zoom, and recording limited to mission purpose where feasible. |
| Data labeling | Uses mission number, date/time, aircraft, operator, location, and retention category. |
| Transfer and storage | Transfers data to approved system and avoids personal or unapproved cloud storage. |
| Evidence or sensitive data | Applies chain-of-custody, access restriction, and supervisor/records notification when applicable. |

## 14.4 DFR Remote Operations Practical

The DFR practical should be conducted only after the trainee is already a qualified RPIC or is being evaluated under the direct supervision of a qualified DFR RPIC who can immediately intervene. The scenario should begin with a CAD or dispatch call, require a launch/no-launch decision, include remote launch-site verification, require communication with dispatch or RTIC/RTCC, and conclude with postflight data handling.

| Element | Pass Standard |
| --- | --- |
| Call triage | Accepts only approved call type and identifies lawful mission purpose. |
| Launch authority | Confirms who may approve launch and records the incident/call identifier. |
| Remote site verification | Uses camera/sensor/weather/dock status to confirm launch and landing area are clear. |
| Airspace and authorization | Confirms airspace, TFR/NOTAM, COA/waiver, altitude, lost-link path, and crewed aircraft hazards. |
| Remote flight | Maintains authorized limits, preserves situational awareness, and uses live video appropriately. |
| Responder coordination | Provides useful information without distracting command or interfering with responder operations. |
| Abnormal event | Handles injected issue such as loss of dock camera, crewed aircraft, public intrusion, changed call type, or low battery. |
| Termination and postflight | Lands safely, secures aircraft/dock, logs flight, classifies data, and reports issues. |

## 14.5 Fire/EMS/SAR Practical

The fire/EMS/SAR practical should be scenario-based. The trainee should receive an incident assignment, identify the incident command contact, brief airspace and responder safety, complete the task, and deliver useful information to the assigned function. The evaluator should include at least one deconfliction or privacy challenge.

| Scenario Element | Expected Performance |
| --- | --- |
| Structure fire overwatch | Positions aircraft to support command, avoids interfering with suppression, reports roof/exposure/thermal observations with sensor limitations. |
| EMS/medevac interface | Identifies landing zone or air-medical activity and lands/holds as needed to preserve crewed aircraft priority. |
| SAR target detection | Executes search pattern, reports target/clue location and confidence, avoids overclaiming sensor evidence. |
| Hazmat/dangerous scene | Maintains standoff, considers wind and contamination, protects responder safety, and avoids unnecessary exposure. |
| Wildland fire support | Does not launch until assigned and deconflicted through appropriate fire aviation/incident channels. |

## 14.6 Law Enforcement Practical

The law-enforcement practical should be reviewed and approved by agency counsel or command staff before operational use. The trainee should be evaluated on lawful pathway selection, mission-scope discipline, tactical coordination, evidence handling, and public interaction. The evaluator should include an inject where the requested use exceeds lawful or policy authority.

| Scenario Element | Expected Performance |
| --- | --- |
| Legal pathway | Identifies warrant, exigency, consent, search and rescue, emergency assistance, scene reconstruction, training, or other authorized basis. |
| Scope control | Limits flight area, sensor use, time, recording, and data access to the authorized purpose. |
| Tactical coordination | Coordinates with incident/tactical commander and does not compromise responder safety. |
| Evidence handling | Preserves original media, metadata, custody log, and case linkage when evidentiary. |
| DFR transition | Recognizes when a public-safety DFR response becomes investigative and pauses for supervisor/legal review if required. |
| Public complaint | Handles the complaint without distracting RPIC, compromising evidence, or arguing during active flight. |

## 14.7 Airspace Coordinator Practical

The airspace coordinator practical should be a tabletop or functional exercise with multiple UAS, crewed aircraft injects, incident command tasking, and at least one lost-link or unauthorized UAS report. The trainee should maintain a common airspace picture and issue clear coordination instructions.

| Task | Satisfactory Performance |
| --- | --- |
| Airspace inventory | Identifies current airspace class, authorizations, TFRs, NOTAMs, airports/heliports, incident aircraft, and UAS resources. |
| Resource tracking | Tracks aircraft identifier, RPIC, agency, mission, altitude, sector, launch site, lost-link plan, and landing status. |
| Deconfliction plan | Assigns sectors, altitude blocks, routes, launch/recovery zones, and communication plan. |
| Conflict response | Suspends or modifies UAS operations when crewed aircraft, medevac, fire aviation, or loss of communications makes deconfliction uncertain. |
| Documentation | Maintains airspace log and contributes to after-action review. |

# 15. Scenario-Based Practical Exercises

Scenario-based practicals should be conducted as tabletop, simulation, live flight, or full-scale exercises depending on risk and resources. The Training Officer should select scenarios that match the agency's adopted mission categories. Exercises should be evaluated with written exercise evaluation guides and converted into after-action improvements using the agency's corrective action process.

## Scenario 1. Routine public works inspection

The trainee receives a request to inspect storm damage to a pump station near a road and a private residence. The trainee must identify public purpose, site access, airspace, launch area, privacy controls, data retention, and postflight records.

## Scenario 2. DFR law-enforcement call

Dispatch reports a priority disturbance call. The DFR operator must determine if the call type is approved, verify launch site, check airspace, launch or decline, coordinate with RTIC/RTCC, avoid mission creep, and classify data.

## Scenario 3. Structure fire overwatch

Incident command requests roof and exposure assessment. The UAS crew must integrate with ICS, avoid apparatus and hose lines, identify medevac risk, use thermal appropriately, and report observations without overclaiming.

## Scenario 4. Search and rescue

A missing person search segment is assigned in mixed terrain. The crew must set search boundaries, brief ground teams, conduct visual/thermal search, mark a clue, and transfer coordinates and imagery properly.

## Scenario 5. Multiagency mutual aid

Three agencies respond with UAS to a flood evacuation. The trainee must establish operational control, airspace coordinator role, altitude blocks, data sharing pathway, public information controls, and demobilization.

## Scenario 6. Wildland fire aviation conflict

A UAS crew arrives to assist with perimeter mapping while fire aircraft are operating. The trainee must ground the UAS until assigned and deconflicted, coordinate through incident aviation channels, and respond to an unauthorized drone sighting.

## Scenario 7. Law-enforcement warrant and evidence

A command post requests UAS support under a warrant. The trainee must read the scope, operate within limits, preserve original media, complete chain of custody, and handle an unrelated incidental capture.

## Scenario 8. DFR emergency procedure

A remote aircraft loses video while responding to a call and a helicopter is reported near the area. The trainee must execute lost-video/crewed-aircraft procedures, communicate with dispatch, land or hold as appropriate, and document the event.

Part V. Supplemental Training Forms and Records

Additional training matrices, qualification checklists, scenario evaluation forms, recurrent training forms, AAR fields, and tracker fields.

# 18. Appendices and Forms

The appendices may be used as paper forms, electronic forms, LMS records, spreadsheet trackers, or fields in a records-management system. Forms should remain simple enough for field use and complete enough to prove training, qualification, currency, and corrective action.

## Appendix A: Training Program Customization Matrix

| Section | Decision Needed | Agency Selection / Notes |
| --- | --- | --- |
| Cover | Agency name, version, effective date, training custodian. | [Insert] |
| 1. Authority | Part 107, public-aircraft COA, ECOA/SGI, DFR waiver, contractor, or combination. | [Insert] |
| 3 Roles | Role holders and alternates for UAS Program Manager, Training Officer, Chief Pilot, instructors, evaluators, records, IT/security, counsel. | [Insert] |
| 4 Testing Methods | Which NIST levels, written tests, oral boards, and scenarios are required for each role. | [Insert] |
| 5 Role Tracks | Which roles and endorsements the agency will use. | [Insert] |
| 6 Curriculum | Local modules, external training credit, and required state/local content. | [Insert] |
| 7 Passing Standards | Written exam scores, NIST thresholds, scenario passing standards, critical-fail items. | [Insert] |
| 9 DFR/BVLOS | FAA authorization, DFR software, dispatch criteria, RTIC/RTCC workflow, dock locations, airspace monitoring plan. | [Insert] |
| 10 Fire/EMS/SAR | Adopt, delete, or mark not adopted. | [Insert] |
| 11 Law Enforcement | Adopt, delete, or mark not adopted; insert counsel-approved legal module. | [Insert] |
| 12 Mutual Aid | Mutual-aid agreements, credential recognition, airspace contacts, radio/phone channels, data-sharing protocols. | [Insert] |
| 13 Currency | Agency-specific recurrent and currency intervals. | [Insert] |
| 16 Records | Record storage, retention, access, privacy, and audit process. | [Insert] |

## Appendix B: Role Training Matrix

| Role | Required Modules | Initial Evaluation | Recurrent Requirement |
| --- | --- | --- | --- |
| Program Manager | T-01 to T-06, T-11, T-15, T-16 | Program management, legal, records, risk, FAA authorization, training oversight. | Annual policy/legal update and program review. |
| RPIC | T-01 to T-10 and assigned endorsements | FAA certificate, policy exam, platform checkout, NIST, checkride (flight test), emergency drill. | FAA recurrent, 90-day currency, annual NIST/checkride (flight test). |
| VO | T-01, T-02 awareness, T-04, T-05, T-06, T-09, T-10 | VO practical, communications drill, lost-link drill. | Annual refresher and drill. |
| Payload Operator | T-01, T-04, T-05, T-07, T-09, T-10 and payload-specific module | Payload practical, data/privacy exercise. | Annual refresh and mission-specific scenario. |
| DFR Teleoperator | T-01 to T-12 and authorization-specific module | DFR exam, simulator/live shift, emergency drill. | DFR shift currency and annual DFR scenario. |
| Fire/EMS Specialist | T-01 to T-11, T-13, T-15 | Fire/EMS scenario, thermal exercise, ICS/NIMS records. | Annual fire/EMS/SAR scenario. |
| Law Enforcement Operator | T-01 to T-12, T-14, counsel-approved legal module | Legal exam, evidence/data exercise, DFR/tactical scenario. | Annual legal/evidence update and scenario. |
| Airspace Coordinator | T-01, T-02, T-05, T-06, T-09, T-11, T-15 | Airspace tabletop, deconfliction drill. | Annual multiagency drill. |
| Instructor/Evaluator | T-16 plus current role qualification | Teach-back, evaluator calibration, observed evaluation. | Annual calibration. |

## Appendix C: Written Exam Blueprint

| Exam Category | Recommended Weight | Notes |
| --- | --- | --- |
| FAA rules, operating authority, and airspace | 25% | Include Part 107, COA/waiver, Remote ID, airspace authorization, night/OOP/moving-vehicle rules, accident reporting, and emergency authorization awareness. |
| Agency policy, roles, and operational control | 15% | Include mission approval, prohibited uses, RPIC authority, stop-flight authority, mutual aid, and contractor/partner roles. |
| Safety management, risk, and emergency procedures | 20% | Include risk matrix, PASP, preflight, Check 3 GPS, lost link, flyaway, battery fire, crewed aircraft, crash, and reporting. |
| Privacy, records, data, and cybersecurity | 15% | Include data minimization, retention, public records, third-party systems, live video, evidence, medical/privacy restrictions, and public release. |
| Role-specific mission content | 20% | Include DFR, fire/EMS, law enforcement, mapping/inspection, mutual aid, airspace coordination, or other assigned missions. |
| Public interaction and professionalism | 5% | Include transparency, complaints, media, public contact, and civil liberties. |

### Appendix D: Initial RPIC Qualification Checklist

| # | Done | Checklist Item | Evidence / Initials |
| --- | --- | --- | --- |
| 1 | [ ] | Candidate approved to enter RPIC training by UAS Program Manager or designee. | [Initials / Date] |
| 2 | [ ] | FAA Remote Pilot Certificate verified or training conducted under permissible supervision pending certificate. | [Initials / Date] |
| 3 | [ ] | Agency policy, legal, privacy, data, and public records modules completed. | [Initials / Date] |
| 4 | [ ] | Aircraft systems, preflight, batteries, Remote ID, and maintenance module completed. | [Initials / Date] |
| 5 | [ ] | Airspace, weather, LAANC, NOTAM/TFR, and crewed-aircraft priority module completed. | [Initials / Date] |
| 6 | [ ] | SMS, risk matrix, PASP, Check 3 GPS, and emergency procedure modules completed. | [Initials / Date] |
| 7 | [ ] | Minimum supervised flight practice completed on assigned aircraft family. | [Initials / Date] |
| 8 | [ ] | NIST-based proficiency event completed at agency-required level and threshold. | [Initials / Date] |
| 9 | [ ] | Platform checkout passed for each assigned aircraft family. | [Initials / Date] |
| 10 | [ ] | Written exam passed with required score and no critical miss. | [Initials / Date] |
| 11 | [ ] | Oral board or scenario discussion completed. | [Initials / Date] |
| 12 | [ ] | Final practical checkride (flight test) passed and limitations documented. | [Initials / Date] |
| 13 | [ ] | Credential entered into training tracker with expiration and currency requirements. | [Initials / Date] |

### Appendix E: Visual Observer Qualification Checklist

| # | Done | Checklist Item | Evidence / Initials |
| --- | --- | --- | --- |
| 1 | [ ] | VO candidate completed agency orientation and role responsibilities module. | [Initials / Date] |
| 2 | [ ] | Candidate understands airspace scanning, traffic calls, obstacle awareness, and ground hazard monitoring. | [Initials / Date] |
| 3 | [ ] | Candidate understands crew communications, sterile cockpit, emergency callouts, and stop-flight authority. | [Initials / Date] |
| 4 | [ ] | Candidate understands lost-link, flyaway, emergency landing, crewed aircraft, and public-intrusion procedures. | [Initials / Date] |
| 5 | [ ] | Candidate demonstrated VO performance in a simulated or live training flight. | [Initials / Date] |
| 6 | [ ] | Candidate completed privacy/data and public interaction awareness training. | [Initials / Date] |
| 7 | [ ] | Evaluator approved candidate for VO duties with any limitations documented. | [Initials / Date] |

### Appendix F: Payload / Sensor Operator Qualification Checklist

| # | Done | Checklist Item | Evidence / Initials |
| --- | --- | --- | --- |
| 1 | [ ] | Candidate completed agency privacy, records, data, cybersecurity, and public release training. | [Initials / Date] |
| 2 | [ ] | Candidate completed payload-specific controls, camera settings, zoom, focus, exposure, thermal palette, recording, and metadata training. | [Initials / Date] |
| 3 | [ ] | Candidate can operate payload under RPIC direction without distracting the RPIC during critical phases of flight. | [Initials / Date] |
| 4 | [ ] | Candidate can identify sensor limitations, misinterpretation risks, and mission-appropriate data minimization controls. | [Initials / Date] |
| 5 | [ ] | Candidate completed data-transfer and retention workflow exercise. | [Initials / Date] |
| 6 | [ ] | Candidate completed mission-specific scenario for thermal, mapping, fire, DFR, evidence, or inspection use as applicable. | [Initials / Date] |

### Appendix G: Platform Checkout Checklist

| # | Done | Checklist Item | Evidence / Initials |
| --- | --- | --- | --- |
| 1 | [ ] | Aircraft, controller, payload, batteries, Remote ID, and software/firmware configuration documented. | [Initials / Date] |
| 2 | [ ] | Candidate can inspect airframe, propellers, motors, landing gear, payload mount, sensors, gimbal, antennas, lights, and controller. | [Initials / Date] |
| 3 | [ ] | Candidate can verify registration, Remote ID, home point, RTH altitude, lost-link behavior, geofence, obstacle sensing, and battery warnings. | [Initials / Date] |
| 4 | [ ] | Candidate can launch, hover, maneuver, position, inspect, orbit, return, land, and secure aircraft safely. | [Initials / Date] |
| 5 | [ ] | Candidate can demonstrate emergency procedures for low battery, lost link, GNSS/compass issue, loss of video, crewed aircraft, and public intrusion. | [Initials / Date] |
| 6 | [ ] | Candidate can complete postflight inspection, flight log, battery log, maintenance note, and data handoff. | [Initials / Date] |

### Appendix H: NIST-Based Proficiency Event Local Setup Checklist

| # | Done | Checklist Item | Evidence / Initials |
| --- | --- | --- | --- |
| 1 | [ ] | Test level, objective, aircraft, payload, evaluator/proctor, and local passing threshold are selected before the event. | [Initials / Date] |
| 2 | [ ] | Test apparatus, bucket/target spacing, scoring sheets, flight boundaries, launch/recovery area, and emergency landing area are set up according to the selected method. | [Initials / Date] |
| 3 | [ ] | FAA operating authority, site permission, airspace, weather, Remote ID, and safety plan are verified. | [Initials / Date] |
| 4 | [ ] | Evaluator briefs trainee on purpose, scoring, fault criteria, termination criteria, and safety rules. | [Initials / Date] |
| 5 | [ ] | Evaluator records aircraft make/model, payload, firmware/software version, camera settings when relevant, lighting/weather, and any faults. | [Initials / Date] |
| 6 | [ ] | Scoresheet is completed immediately after the event and stored in the trainee record. | [Initials / Date] |
| 7 | [ ] | Certificate wording avoids implying NIST certification unless the credential is issued under an authorized certification mechanism. | [Initials / Date] |

### Appendix I: DFR Scenario Evaluation Checklist

| # | Done | Checklist Item | Evidence / Initials |
| --- | --- | --- | --- |
| 1 | [ ] | Trainee identifies call type, mission purpose, launch authority, and whether DFR response is permitted. | [Initials / Date] |
| 2 | [ ] | Trainee verifies FAA authorization, weather, airspace, dock status, Remote ID, and crew availability before launch. | [Initials / Date] |
| 3 | [ ] | Trainee communicates correctly with dispatch, RTIC/RTCC, RPIC, VO/airspace monitor, field units, and supervisor. | [Initials / Date] |
| 4 | [ ] | Trainee operates within altitude, route, geofence, recording, live-feed, and data-sharing limits. | [Initials / Date] |
| 5 | [ ] | Trainee responds correctly to a crewed-aircraft, public privacy, low battery, lost-link, and mission-termination inject. | [Initials / Date] |
| 6 | [ ] | Trainee completes log, data handling, and after-action documentation. | [Initials / Date] |

### Appendix J: Fire/EMS/SAR Scenario Evaluation Checklist

| # | Done | Checklist Item | Evidence / Initials |
| --- | --- | --- | --- |
| 1 | [ ] | Trainee checks in with incident command or assigned supervisor and confirms UAS assignment. | [Initials / Date] |
| 2 | [ ] | Trainee identifies incident hazards, airspace hazards, air medical considerations, and crewed-aircraft priority. | [Initials / Date] |
| 3 | [ ] | Trainee uses thermal/visual payload appropriately and explains limitations of observed data. | [Initials / Date] |
| 4 | [ ] | Trainee avoids interference with responders, apparatus, evacuees, patients, and suppression/rescue operations. | [Initials / Date] |
| 5 | [ ] | Trainee delivers information through approved command or EOC pathway. | [Initials / Date] |
| 6 | [ ] | Trainee completes data and record handling according to incident and agency policy. | [Initials / Date] |

### Appendix K: Law-Enforcement Scenario Evaluation Checklist

| # | Done | Checklist Item | Evidence / Initials |
| --- | --- | --- | --- |
| 1 | [ ] | Trainee identifies legal pathway and required approval before flight or explains why flight is not permitted. | [Initials / Date] |
| 2 | [ ] | Trainee limits flight area, sensor use, recording, altitude, time, and data collection to the authorized purpose. | [Initials / Date] |
| 3 | [ ] | Trainee protects evidence, metadata, chain of custody, public records, and legal hold requirements. | [Initials / Date] |
| 4 | [ ] | Trainee avoids general surveillance, protected-activity monitoring, fishing expeditions, and unauthorized data use. | [Initials / Date] |
| 5 | [ ] | Trainee communicates with command, field units, prosecutor/counsel/evidence custodian as required by policy. | [Initials / Date] |
| 6 | [ ] | Trainee completes mission record and data segregation for training, incidental, evidentiary, and public-safety data. | [Initials / Date] |

### Appendix L: Annual Recurrent Training Checklist

| # | Done | Checklist Item | Evidence / Initials |
| --- | --- | --- | --- |
| 1 | [ ] | FAA recurrent training or aeronautical knowledge recency verified for RPICs. | [Initials / Date] |
| 2 | [ ] | Agency policy, legal, privacy, records, cybersecurity, and prohibited-use update completed. | [Initials / Date] |
| 3 | [ ] | Safety bulletin and lessons learned from prior year reviewed. | [Initials / Date] |
| 4 | [ ] | Emergency procedures drill completed. | [Initials / Date] |
| 5 | [ ] | Platform currency verified or supervised proficiency flight completed. | [Initials / Date] |
| 6 | [ ] | NIST-based proficiency event or practical evaluation completed as required by role. | [Initials / Date] |
| 7 | [ ] | Mission-specific endorsements reviewed and revalidated or expired. | [Initials / Date] |
| 8 | [ ] | Training record updated and credential expiration date revised. | [Initials / Date] |

### Appendix M: Training After-Action Review Checklist

| # | Done | Checklist Item | Evidence / Initials |
| --- | --- | --- | --- |
| 1 | [ ] | Training objective and scenario were clearly stated. | [Initials / Date] |
| 2 | [ ] | Safety plan, risk assessment, airspace, and emergency procedures were adequate. | [Initials / Date] |
| 3 | [ ] | Trainees demonstrated required knowledge, skills, and judgment. | [Initials / Date] |
| 4 | [ ] | Evaluation criteria were understood and applied consistently. | [Initials / Date] |
| 5 | [ ] | Equipment, software, aircraft, batteries, and training site performed as expected. | [Initials / Date] |
| 6 | [ ] | Any safety, privacy, public interaction, or data issues were documented. | [Initials / Date] |
| 7 | [ ] | Improvement items were assigned to responsible persons with due dates. | [Initials / Date] |
| 8 | [ ] | Curriculum, checklists, or policies requiring revision were identified. | [Initials / Date] |

Part VI. Sources Consulted and Local Review Checklist

This section consolidates source and review notes from all source documents into one adoption checklist.

# Sources Consulted and Model Basis

| Source / Reference Category | How It Was Used |
| --- | --- |
| Uploaded policy, operations, training, curriculum, testing, practical, and RPCS-integrated documents | Primary source material for this unified manual. Duplicate provisions were consolidated, while specialized test-bank, practical-evaluation, DFR, BVLOS, and carding content was retained as administrative material. |
| FAA Part 107, Remote Pilot certification and recurrent training, public aircraft COA, SGI/ECOA, waivers, Remote ID, airspace authorization, and public-safety UAS guidance | Operating authority, RPIC duties, recurrent knowledge, registration, Remote ID, COA/waiver compliance, emergency authorization, and reporting concepts. |
| NIST aerial drone standard test methods for response robots and public-safety UAS proficiency | Objective practical proficiency, platform checkout, sensor/payload testing, annual proficiency, and evaluator scoring methods. |
| FEMA / EMI ICS and NIMS concepts | Incident command, unified command, resource ordering, operational period, demobilization, mutual aid, and exercise improvement planning. |
| DRONERESPONDERS and LEDA public-safety UAS resources and DFR working group concepts | Responsible-use principles, public-safety benchmarking, DFR implementation, airspace deconfliction, waiver navigation, and program-management concepts. |
| NFPA 2400 public-safety UAS concepts | Program management, personnel qualification, operational procedures, safety, maintenance, and job-performance expectations for public-safety UAS programs. |
| NWCG wildfire and incident aviation concepts | Wildland and WUI UAS operations, Fire Traffic Area awareness, aviation deconfliction, TFR/SGI coordination, and incident aviation integration. |
| Oregon public body UAS law and policy placeholders, including ORS/OAR concepts from the source documents | Optional Oregon addendum material for public body registration, annual reporting, data policy, public records, and law-enforcement limitations. Must be updated and reviewed by counsel before adoption. |
| DHS/NUSTL, NJDOT, ODF, ODOT, DPSST, and other model materials summarized in the source documents | Program documentation, operational control, SMS, PASP, emergency response, maintenance, forms, field procedures, training, and public-agency UAS governance concepts. |

# Local Review Checklist Before Adoption

| No. | Review Item | Responsible Office | Status / Notes |
| --- | --- | --- | --- |
| 1 | Complete all bracketed agency fields, role assignments, version dates, and review cycles. | Program Manager / Training Officer | [INSERT] |
| 2 | Verify FAA operating pathways for each mission type, including Part 107, COA, waiver, SGI/ECOA, DFR, BVLOS, shielded operations, operations over people, operations over moving vehicles, controlled airspace, and contractor operations. | Program Manager / Chief Pilot / Counsel | [INSERT] |
| 3 | Insert current state, local, tribal, public records, privacy, retention, evidence, medical privacy, procurement, grant, insurance, labor, and constitutional requirements. | Counsel / Records / Risk / HR | [INSERT] |
| 4 | Approve mission categories and delete or mark unused annexes as not adopted. | Agency Head / Program Manager | [INSERT] |
| 5 | Insert actual aircraft, payload, battery, dock, software, RTIC/RTCC, CAD, DFR, live-video, cybersecurity, and data-storage workflows. | Chief Pilot / IT/Security / Operations | [INSERT] |
| 6 | Set written exam passing scores, critical-fail rules, NIST thresholds, practical scoring standards, currency intervals, and supervised shift requirements. | Training Officer / Chief Pilot | [INSERT] |
| 7 | Separate the student version from the administrative version by removing answer keys and evaluator-only injects. | Training Officer | [INSERT] |
| 8 | Publish the public-facing policy, complaint process, records request instructions, authorized mission categories, and transparency information as permitted by law. | PIO / Records / Counsel | [INSERT] |
| 9 | Adopt a schedule for annual program review, training audit, instructor calibration, records audit, and corrective-action tracking. | Program Manager / Training Review Group | [INSERT] |