SP00310  (Special Provisions for the 2021 Book) 	(Bidding on or after: 12-01-20

Last updated: 08-31-20)

# SECTION 00310 - REMOVAL OF STRUCTURES AND OBSTRUCTIONS

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

(Note: Only include the "Asphalt Pavement Saw Cutting" pay item in the schedule of items when the estimated quantity is greater than 500 feet.)

(Note: Include Pay Item for “Removal of Concrete Barrier” when removal of concrete barrier work is included on the Project. Use either Lump Sum or Length unit of measurement. If Section 00820 “Remove and Reinstall Concrete Barrier” is also included in the project, do not include that quantity of barrier removal in this Section.)

Comply with Section 00310 of the Standard Specifications modified as follows:

00310.90  Payment - Add the following to the end of this subsection:

No separate or additional payment will be made for removal or disposal Work included in Section 00330 according to 00310.02.

(Use the following subsection .92 on Projects that have more than 8 curb ramps that are not included in Section 00330, otherwise (8 curb ramps or less not in Section 00330) curb ramp removal work will be included in the lump sum Removal of Structures and Obstructions. Include Pay Item for Asphalt Pavement Saw Cutting to accompany this item. Include 00759.90 to remove saw cutting and removal from item (k).)

00310.92  Separate Item Basis - Add the following Pay Item to the Pay Item list:

(i)	Removal of Curb Ramp, Walk, Curb, Surfacing	Square Yard

Add the following to the end of this subsection:

Item (i) includes all removal Work associated with curb ramp replacement, including but not limited to PCC Saw Cutting. Item (i) does not include Asphalt Pavement Saw Cutting.