SP00503  (Special Provisions for the 2021 Book) 	(Bidding on or after: 02-01-22

Last updated: 10-28-21

This Section requires SP00220.45)

# SECTION 00503 - BRIDGE DECK COLD PLANE PAVEMENT REMOVAL

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Comply with Section 00503 of the Standard Specifications modified as follows:

00503.01  Terminology - For the purposes of this Section, the terms “end panel” and “end panels” shall respectively refer to and shall be read to mean “approach slab” and “approach slabs”.

00503.20  Equipment for Grinding on Bridge Decks - Add the following to the paragraph that begins "To remove Pavement from bridge decks…":

Limit the gross operational weight of machines to comply with the load limitations of 00220.45. Limit machines to a forward speed of 2.5 feet per minute. Operate at a drum speed of at least 120 RPM.