SP00520  (Special Provisions for the 2021 Book) 	(Bidding on or after: 04-01-23

Last updated: 12-21-22)

# SECTION 00520 - DRIVEN PILES

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

(Use the following lead-in paragraph when none of the following subsections are included in the project.)

Comply with Section 00520 of the Standard Specifications.

(Use the following lead-in paragraph when any of the following subsections are included in the project.)

Comply with Section 00520 of the Standard Specifications modified as follows:

(Use the following subsection .11 on projects that require steel piles. Insert bridge structure number and bent number under "Location". Insert the number of piles in each bent under "Number". Insert the estimated length of the individual piles in each bent under "Length". Insert the type and dimensions of the pile under "Type and Size". If the piles will be coated, insert the top and bottom elevations of the coating in the "Coating Top Elevation" and "Coating Bottom Elevation" columns, otherwise insert "n/a" Add rows to the table as necessary. Obtain information from the Geotechnical Designer.)

00520.11  Engineer's Estimated Length List - Add the following to the end of this subsection:

The Engineer's estimated lengths of steel piling are:

| Location | Number | Length (feet) | Type and Size | Coating Top Elevation1 | Coating Bottom Elevation1 |
| --- | --- | --- | --- | --- | --- |

1  Protective coating system and color requirements according to 00594.10.

(Use the following subsection .12 on projects that require prestressed concrete piles or timber piles. Delete "(timber piling)" or "(concrete piling)" if not on project. Insert bridge number and bent number under "Location". Insert the number of pile in each bent under "No.". Insert the order length of the individual piles in each bent under "Length". Insert the type and dimensions of the pile under "Type and Size". For timber piling, include butt or tip size and whether they are friction or bearing pile under "Type and Size". Obtain information from the Geotechnical Designer.)

00520.12  Pile Order List - Add the following to the end of this subsection:

The pile order list for precast prestressed (concrete piling) (timber piling) is:

Location	No.	Length (Feet)	Type and Size

(Use the following subsection .13 on projects that require test piles. Insert bridge structure number and bent number under "Location". Insert the number of piles in each bent under "No.". Insert the estimated length of the individual piles in each bent under "Length". Insert the type and dimensions of the pile under "Type and Size". Obtain information from the Geotechnical Designer.)

00520.13  Test Piles - Add the following to the end of this subsection:

The required test piles are:

Location	No.	Length (Feet)	Type and Size

(Use the following subsection .20(c)(4) when required by the Geotechnical Designer.)

00520.20(c)(4)  Followers - Add the following to the end of this subsection:

Followers are permitted.

(Use the following subsection .20(d)(3) when WEAP input values are required. Include further explanations if necessary. More than one table may be required to assess pile driving stresses through upper hard or dense soils layers or other conditions. Consult with the Geotechnical Designer regarding the appropriate input(s) to use. Typically supply the highest Rn. value and the worst-case driving conditions that the contractor should use in the WEAP analysis to determine hammer requirements. Supply WEAP input data for a complete "drivability analysis" if appropriate. Obtain information from the Geotechnical Designer.)

00520.20(d)(3)  Wave Equation Method - Add the following paragraph and table(s) to the end of this subsection:

The input values for the wave equation analyses are:

| Bent | Pile Type | Pile Length * (Feet) | Quake (Inches) |  | Damping (sec./ft.) |  | % skin (ITYS) | Rn (kips) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Skin | Toe | Skin | Toe |  |  |

*	These pile lengths are based on the top of the pile being at the finished cutoff elevation. All additional pile length above the cutoff elevation, that may be required to accommodate the Contractor’s pile installation method or site conditions, shall be added to the lengths listed above and appropriate changes made to the skin friction distribution input listed below.

(Use one of the following two options. Delete the one that does not apply.)

[ Option 1 - Use this option when either triangular or rectangular distribution is required. Delete the one (triangular or rectangular) that does not apply. ]

Use (triangular)(rectangular) skin friction distribution.

[ Option 2 - Use this option when providing relative skin friction values in table form. Insert Relative Skin Friction Distribution values in the table. Fill in the bent number. Add or delete rows and columns as appropriate. ]

Use the relative skin friction distribution values listed below in the WEAP analysis:

| Bent _____ |  | Bent _____ |  | Bent _____ |  |
| --- | --- | --- | --- | --- | --- |
| Depth (Feet) | Relative Distribution | Depth (Feet) | Relative Distribution | Depth (Feet) | Relative Distribution |

(Use the following subsection .41(d) when augering or wet-rotary drilling is allowed by the Geotechnical Designer.)

00520.41(d)  Preboring - Add the following sentence to the end of this subsection:

Use augering, wet-rotary drilling or other approved methods of preboring as directed.

(Use the following subsection .41(e) when jetting is allowed by the Geotechnical Designer.)

00520.41(e)  Jetting - Add the following sentence to the end of this subsection:

Jetting is permitted.

(Use the following subsection. 42(d) when a minimum "set period" of longer than 24 hours is required or if piles are required to set before redriving. Check with the Geotechnical Designer.)

00520.42(d)  Set Period and Redriving -

(Use the following two paragraphs when a minimum “set period” of longer than 24 hours is required. Fill in the blank. Use days or hours as appropriate and delete whichever doesn’t apply.)

Replace the sentence that begins “The “set period” shall be…” with the following sentence:

The “set period” shall be a minimum of _____ (Days)(hours) unless otherwise approved by the Engineer.

(Use the following two paragraphs when piles are required to set before redriving.)

Add the following sentence to the end of this subsection:

Piles are required to set before redriving.

(Use the following subsection .43(c) on projects with steel pipe piles that do not require reinforced tips. Delete the end treatment that does not apply and remove the parentheses. Obtain information from the Geotechnical Designer. If reinforced tips are required, do not use this subsection; instead use subsection .43(d) below.)

00520.43(c)  End Treatment - Add the following sentence to the end of this subsection:

Drive steel pipe piles (open)(closed)-ended with tip treatment as shown.

(Use the following subsection .43(d) on projects with steel pipe piles that require reinforced pile tips. Delete "inside" or "outside" as appropriate and remove the parentheses. Obtain information from the Geotechnical Designer.)

00520.43(d)  Reinforced Pile Tips - Add the following sentence to the end of this subsection:

For steel pipe piling, provide (inside)(outside) fit, open end cutting shoes meeting the requirements of 02520.10(e).

(Use the following subsection .43(g)(1) when splices are required.)

00520.43(g)(1)  Splices - Replace this subsection, except for the subsection number and title, with the following:

Splice joints for pipe piles shall conform to Joint B-U4a or B-U4a-GF (Single-Bevel Groove Weld) in D1.1. Weld back-up rings with a full penetration groove weld. Pipe pile splices that include a steel plate for Soil plug formation shall conform to Joint TC-U4a or TC-U4a-GF.

Splice joints for H-piles shall conform to Joint B-U3b or B-U3-GF (Double V-Groove Weld) in D1.1 for both the web and flange sections. Joint B-U4a or B-U4a-GF may be substituted on the flange weld. Provide access holes at the ends of the web according to D1.1.

(Use the following subsection .43(g)(2) when submittals are required.)

00520.43(g)(2)  Submittals - Replace the bullet that begins “A Welding Procedure Specification (WPS) for all pile…” with the following bullet:

A Welding Procedure Specification (WPS) for all pile welds, conforming to the limitations of D1.1. Both ASTM A36 and ASTM A252 Grade 1 and 2 may be treated as prequalified base metals under Group 1. ASTM A252 Grade 3 will not be considered a prequalified base metal unless the steel has a Carbon Equivalent (CE) of 0.30 percent or less. Develop a Procedure Qualification Record (PQR) for all welding using Grade 3 steel or present proof that the chemistry of the steel meets the CE requirements.

Replace the bullet that begins “An inspection report stating that the welding under the Contract …” with the following bullet:

An inspection report stating that the welding under the Contract was performed according to D1.1. The report shall include a review of the WPS, a review of welder qualifications and a report on visual inspection of the welds on the Project Site. The inspection shall be signed by a Certified Welding Inspector (CWI) holding QC1 certification as defined in D1.1.

(Add the following lead-in paragraph and subsection .80(g) when protective coatings are required on steel piles.)

Add the following subsection:

00520.80(g)  Steel Pile Protective Coatings - The estimated quantities of steel pile protective coatings are:

Length of Coated

Coating System	Pile (feet)