SP00550  (Special Provisions for the 2021 Book) 	(Bidding on or after: 03-01-23

Last updated: 12-06-22

This Section requires SP00530,

SP02001 and SP02560.

Requires SP02510 if epoxy coated

reinforcement is required)

# SECTION 00550 - PRECAST PRESTRESSED CONCRETE MEMBERS

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Comply with Section 00550 of the Standard Specifications modified as follows:

(Use the following subsection .06 when precast members with UHPC connection are specified and Camber correction is required.)

Add the following subsection:

00550.06  Submittals - Submit unstamped plan of differential Camber correction for the adjacent precast concrete members for UHPC joint connections at least 21 Calendar Days before precast member fabrication according to 00150.35. Include step-by-step procedure and identify Equipment, Materials, and any items to be cast into the precast concrete members in the submittal.

(Use the following subsection .50 when precast members with UHPC connection are specified and Camber correction is required.)

00550.50  Tie Rods - Delete this subsection

(Use the following subsection .51 when precast members with UHPC connection are specified and Camber correction is required.)

00550.51  Keyway Grouting for Slabs, Box Beams, and Integral Deck Members - Delete this subsection

(Use the following subsection .52 when precast members with UHPC connection are specified and Camber correction is required.)

00550.52  Hot Applied Joint Sealant for Integral Deck Members with AC Wearing Surface - Delete this subsection

00550.53  Differential Camber Correction - Replace this subsection with the following subsection:

00550.53 - Differential Camber Correction for Adjacent Precast Prestressed Concrete Members with No ACP Wearing Surface or with No Cast-in-place Concrete Deck -

Correct differential Camber between adjacent slabs, box beams or integral deck bulb tee girders in a span (measured in place at the site) by a method approved by the Engineer if the variance between adjacent members or stages is 1/4 inch or more at the top edge corners at midspan.

Equalize the Camber differences by either patching with an epoxy or non-epoxy grout or other approved method, at no additional cost to the Agency. Before patching, clean the area by sandblasting. Water cure the patch for the period of time indicated by the manufacturer. If patching is used, slope it away from the joint on a 1V:6H Slope of flatter.

00550.80  Measurement -

(Use one of the following options)

[Option 1 - Use the following when Option 2 is not used.]

[ Begin Option 1 ]

Add the following paragraph to the end of this subsection:

Stirrup extension reinforcement will be measured according to 00530.80. Estimated quantities of reinforcement for the lump sum method will be listed in 00530.80(a).

[ End Option 1 ]

[Option 2 - when precast members with UHPC connection are specified or Camber correction is required]

[ Begin Option 2 ]

Replace this subsection, except for the subsection number and title, with the following:

The quantities of Work performed under this Section will be measured according to the following:

(a)  Length Basis - Precast prestressed members will be measured on the length basis, and will be the sum of the horizontal lengths shown for each type and size of member.  Field measurement of each member length will not be made. The quantities will be determined by calculating the theoretical horizontal length shown, added together for a total for each type and size.

(b)  Lump Sum - No measurement of quantities will be made for Camber Correction.

Stirrup extension reinforcement will be measured according to 00530.80. Estimated quantities of reinforcement for the lump sum method will be listed in 00530.80(a).

[ End Option 2 ]

00550.90  Payment - Add the following paragraph to the end of this subsection:

Stirrup extension reinforcement, as shown, will be paid for according to 00530.90.

(Use the following paragraph and Pay Item when precast members with UHPC connection are specified or Camber correction is required.)

Add the following Pay Item to the Pay Item list:

(d) Camber Correction ………………………..…	Lump Sum