SP00581  (Special Provisions for the 2021 Book) 	(Bidding on or after: 06-01-23

Last updated: 02-22-23

This Section requires SP02530

And SP02560.)

# SECTION 00581 - BRIDGE DRAINAGE SYSTEMS

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

(Use the following lead-in paragraph when none of the following subsections are included in the project.)

Comply with Section 00581 of the Standard Specifications.

(Use the following lead-in paragraph when any of the following subsections are included in the project.)

Comply with Section 00581 of the Standard Specifications modified as follows:

(Use the following subsection .00 for rehabilitation work.)

00581.00  Scope - Add the following paragraph to the end of this subsection:

This Work consists of adjusting manholes and inlets on Bridges as shown, specified or directed.

(Use the following subsection .10 for rehabilitation work.)

00581.10  Materials - Replace the paragraph that begins "Furnish deck drains, hangers, clamps, …" with the following paragraph:

Furnish deck drains, hangers, clamps, replacement inlet grates and other Incidentals meeting the requirements of AASHTO M183 (ASTM A36), Section 02530 and Section 02560. Galvanize after fabrication according to AASHTO M111 (ASTM A123).

(Use the following subsection .41 for rehabilitation work.)

Add the following subsection:

00581.41  Modifying Existing Manholes and Inlets - Modify existing manholes and inlets on Bridges as shown and according to the following:

(a)  Modifying Manholes - Modify manholes as follows:

Remove existing manhole covers and reuse when shown.

Remove existing concrete deck around the manhole frame according to Section 00504.

Clean and prepare the existing steel manhole frame in place.

Field weld steel riser rings as shown. Perform all field welding according to AWS D1.1.

Adjust riser rings to be flush with the proposed Roadway profile and maintain a uniform contact with the existing manhole frame. Complete this portion of the Work prior to the installation of the overlay.

Clean the new surface by brushing and moistening with water at the time of placing the overlay.

(b)  Modifying Inlets - Modify inlets as follows:

Remove existing inlet grates and reuse when shown.

Remove existing concrete deck around inlet frame according to Section 00504.

Remove portions of the existing inlet frames as shown.

Clean and prepare the existing steel inlet frame in place.

Adjust or install replacement inlet grate as shown.

Adjust the inlet grate to be flush with the proposed Roadway profile and maintain uniform contact with the existing inlet frame. Complete this portion of the Work prior to the installation of the overlay.

Clean the new surface by brushing and moistening with water at the time of placing the overlay.

(Use the following subsection .80 for rehabilitation works.)

00581.80  Measurement - Add the following paragraph to the end of this subsection:

The quantities of modifying existing manholes and inlets on Bridges will be measured on the unit basis of each manhole or inlet replaced.

(Use the following subsection .90 for rehabilitation works.)

00581.90  Payment - Replace this subsection, except for the subsection number and title, with the following:

The accepted quantities of Work performed under this Section will be paid for at the Contract unit price, per unit of measurement, for the following items:

(Delete pay item(s) from the list that are not included in the Schedule of Items, but do not change the alpha characters next to the pay items.)

Pay Item	Unit of Measurement

(a)	Bridge Drains	Each

(b)	Modifying Existing Bridge Drains	Each

Item (b) includes modifying existing drainage manholes and inlets on Bridges.

Payment will be payment in full for furnishing and placing all Materials, and for furnishing all

Equipment, labor, and Incidentals necessary to complete the Work as specified.

(Delete the following two paragraphs if Item (b) Modifying Existing Bridge Drains is deleted.)

No separate or additional payment will be made for drain pipe connected to bridge drains or for water used in testing the drain systems.

No separate or additional payment will be made for field measurement, removal and disposal of existing grates.