SP00743  (Special Provisions for the 2021 Book) 	(Bidding on or after: 08-01-23

Last updated: 05-04-23)

# SECTION 00743 - POROUS ASPHALT CONCRETE (PAC)

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Comply with Section 00743 of the Standard Specifications modified as follows:

00743.45(a)(2)  Wearing Course - Replace this subsection, except for the subsection number and title, with the following:

Construct longitudinal joints six inches from  either skip lines or fog lines, or as shown or directed. On Median lanes and on Shoulder areas construct longitudinal joints only at lane lines or at points of change in the transverse slopes, as shown or as directed. Otherwise, do not construct longitudinal joints within the width of a Traffic Lane unless shown or directed.

00743.45(b)  Drop-Offs - Replace the bullet that begins “Provide warning signs and markings…” with the following bullet:

Provide warning signs and markings according to Sections 00221, 00222, 00224 and 00225 where abrupt or sloped edge drop-offs greater than 1 inch in height occur.