SP00850  (Special Provisions for the 2021 Book) 	(Bidding on or after: 12-01-20

Last updated: 06-15-20)

(When this Section is used on a project and when it has a completion date of September 15 or later, contact the Scheduler to determine if a separate completion date for striping is required. If a separate completion date is required, include an interim completion date in 00180.50(h).)

# SECTION 00850 - COMMON PROVISIONS FOR PAVEMENT MARKINGS

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

(Use the following lead-in paragraph when none of the following subsections are included in the project.)

Comply with Section 00850 of the Standard Specifications.

(Use the following lead-in paragraph when any of the following subsections are included in the project.)

Comply with Section 00850 of the Standard Specifications modified as follows:

(Use the following subsection .30 only with approval from the ODOT Pavement Marking Manager and when the project meets ALL of the following requirements:

Only surface applied markings (no groove installed markings or tape), AND

Less than 2000’ of 4” longitudinal markings, AND

Fewer than 7 legends, AND

Less than 400 square feet of bars.)

00850.30  Manufacturer’s Representative - Replace this subsection, except for the subsection number and title, with the following:

For Sections referencing 00850.30, the services of a manufacturer’s representative are not required. Place pavement markings only when the pavement is ready for the pavement marking material according to the manufacturer’s installation instructions.

(Use the following subsection .47(c) only when Section 00868 – "Colored Lane Markings" is included in the Special Provisions)

00850.47(c)  Retroreflectivity - Replace the sentence that begins "Except for paint applications…" with the following sentence:

Except for paint and colored lane marking applications, evaluate longitudinal and transverse marking retroreflectivity according to ODOT TM 777.