SP01120  (Special Provisions for the 2021 Book) 	(Bidding on or after: 10-01-23

Last updated: 06-28-23)

# SECTION 01120 - IRRIGATION SYSTEMS

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Comply with Section 01120 of the Standard Specifications modified as follows:

01120.40(b) Electrical Service – Replace this subsection, except for the subsection number and title, with the following:

Install electrical service according to the National Electrical Code and all State and local laws. Power sources will be as shown or as directed. Be responsible for coordination and installation of electrical service. Furnish and install meter bases at the power source conforming to the requirements of the power supplier. Give the power supplier's representative notice before making any installation. Provide a separate, dedicated circuit for the controller.