SP00221  (Special Provisions for the 2021 Book) 	(Bidding on or after: 08-01-23

Last updated: 05-04-23

This Section requires SP00222,

SP00223 and SP00226 when

mobile barrier is required)

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

(NOTE: All Federal-aid projects, including local government projects, that are advertised and awarded by ODOT require "Method 'A' Unit Basis" measurement [see Standard Specifications 00221.80].)

# SECTION 00221 - COMMON PROVISIONS FOR WORK ZONE TRAFFIC CONTROL

Comply with Section 00221 of the Standard Specifications modified as follows:

00221.03  Traffic Safety and Operations - Replace the bullet that begins “When paving operations create…” with the following bullet:

When paving operations create an abrupt or sloped edge drop off greater than 1 inch, protect traffic by installing signing according to the "2 Lane, 2 Way Roadway Overlay Area" detail shown on the Standard Drawings. Protect longitudinal and transverse Pavement joints by placing and maintaining an asphalt concrete wedge according to 00221.07(c)(1).

00221.06  (c)  Tourist-Oriented Directional and Business Logo Signs - Replace this subsection, except for the subsection number and title, with the following:

Submit one of the following for approval, at least 5 Calendar Days before the preconstruction conference:

(1)  No Signs - If there are no tourist-oriented directional (TOD) or business logo signs on the Project, a written notification that no TOD or business logo signs exist within the Project limits or

(2)  Signs - Submit one copy of a sketch map of the Project showing all existing TOD and business logo signs and a written narrative describing how these signs will be kept in service and protected throughout all the construction stages. If modifications are necessary, submit updated information to the Engineer for approval at least 21 Calendar Days before the change is needed.

00221.07(c)(1)  Paving - Replace this subsection, except subsection number and title, with the following:

When the longitudinal joint is greater than 1 inch in height, install additional TCD according to 00221.03. Complete the placing of ACP and construction of paving joints according to 00735.48, 00735.49, 00743.45, 00744.44, 00744.45, 00745.47, and 00745.48, as applicable.

00221.90(b)  Temporary Protection and Direction of Traffic - Delete the bullet that begins “Moving temporary barrier to and from Contractor’s stockpile areas”.

Replace the bullet that begins "When the Schedule of Items does not include …" with the following bullet:

Preparing and signing the daily “Traffic Control Inspection Report”, when a TCS is not included in the Schedule of Items or when a TCS is not onsite for a work shift.

(Use the paragraph and bullets when Mobile Barrier is required.)

Add the following bullets to the end of the bullet list:

Mobilization of the mobile barrier to and from the project.

Moving the mobile barrier from one location of actual use to another or to and from the Contractor’s storage site.

(This page is only used to provide a list of standard drawings to the specification writer for listing on the plan title sheet. Remove this page before advance and final.)

NUMBER OF TRAFFIC CONTROL PLAN SHEETS: _____

(Add or delete Standard Drawings, as applicable.)

To be accompanied by Standard Drawings:

RD410	Guardrail Parts (Thrie Beam)

RD420	Energy Absorbing Terminal

RD425	Non Energy-Absorbing Terminal 3' or 4' Flare

RD500	Precast Concrete Barrier Pin and Loop Assembly

RD510	Concrete Barrier Terminal

RD530	Guardrail Transition to Concrete Barrier

RD535	Concrete Barrier (Modified) Around Median Obstacle

RD545	Precast Tall (42") Concrete Barrier

RD560	Cast-in-Place Tall Barrier Transition to Standard Concrete Barrier

BR203	Transition Concrete Bridge Rail to Guardrail

TM204	Flag Board Mounting Details

TM211	Signing Details US & Interstate Route Shields

TM212	Signing Details Oregon Route Signs

TM570	Traffic Delineators

TM575	Traffic Delineator Installation for Freeways

TM576	Traffic Delineator Installation for Non-Freeways

TM670	Wood Post Sign Supports

TM671	3 Second Gust Wind Speed Map

TM677	Sign Mounts

TM681, TM687, TM688	Perforated Steel Square Tube (PSST)Sign Support Installation and Foundation

TM800	Tables, Abrupt Edge and PCMS Details

TM810	Temporary Pavement Markers

TM820	Temporary Barricades

TM821, TM822	Temporary Sign Supports

TM830	Temporary Concrete Barrier & Rumble Strip Details

TM831, TM832, TM833	Temporary Impact Attenuators

TM840	Closure Details

TM841	Intersection Work Zone Details

TM842	Signalized Intersection Details

TM843	Multi-Lane Signalized Intersection Details

TM844	Temporary Pedestrian Access Routing

TM850	2 Lane, 2-Way Roadways

TM851, TM852, TM853	Non-Freeway Multi-Lane Sections

TM860, TM861, TM862	Freeway Sections

TM870	Bridge Construction

TM871	Blasting Zones

TM880	Freeway or Divided Highway Speed Reduction (Paving

Operations)