SP00334  (Special Provisions for the 2021 Book) 	(Bidding on or after: 12-01-20

Last updated: 08-19-20)

(Use this Section on highway widening projects only when a small amount of grading is required.)

# SECTION 00334 - PREPARATION OF SHOULDERS

(Follow all instructions and make all edits with “Track Changes” turned on. This Section is not published in the Oregon Standard. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the project, unless the item(s) that are included in the subsection, paragraph, sentence, or bullet are not required on the Project and then they should be deleted. In general do not re-number or re-letter subsections when item(s) are deleted. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Section 00334, which is not a Standard Specification, is included in this Project by Special Provision.

## Description

00334.00  Scope - This Work consists of blading, leveling, and compacting shoulders as required to bring the shoulder subgrade to the required widths, grade, and condition to receive the pavement as shown.

## Construction

00334.40  General - Sidecast all material resulting from shoulder preparation without obstructing existing ditches.

(Use one of the following options as instructed below.  Delete the option that does not apply.)

[ Option 1 - Use the following measurement and payment headings and subsections .80 and .90 when measurement is on the length basis. ]

[ Begin Option 1 ]

## Measurement

00334.80  Measurement - The quantities of preparation of shoulders will be measured on the length basis.  When preparation is performed on both shoulders, measurement will be measured along the center line of the highway excluding the length of bridges.  Where preparation of shoulders is performed on only one shoulder, the pay length will be one-half the length as measured above.

## Payment

00334.90  Payment - The accepted quantities of shoulder preparation will be paid for at the Contract unit price, per mile, for the item "Preparation of Shoulders".

Payment will be payment in full for furnishing all Equipment, labor, and Incidentals necessary to complete the Work as specified.

No separate or additional payment will be made for watering.

[ End Option 1 ]

[ Option 2 - Use the following measurement and payment headings and subsections .80 and .90 when measurement is lump sum.  Fill in the blanks.  Delete the language in orange parentheses that does not apply and delete all orange parentheses.)

[ Begin Option 2 ]

## Measurement

00334.80  Measurement - No measurement of quantities will be made for Work performed under this Section.

It is estimated that ______ centerline miles of shoulder preparation will be performed on (one shoulder) (both shoulders).

## Payment

00334.90  Payment - The accepted quantities of shoulder preparation will be paid for at the Contract lump sum amount for the item "Preparation of Shoulders".

Payment will be payment in full for furnishing all Equipment, labor, and Incidentals necessary to complete the Work as specified.

No separate or additional payment will be made for watering.

[ End Option 2 ]