SP00350  (Special Provisions for the 2021 Book) 	(Bidding on or after: 02-01-23

Last updated: 11-07-22

This Section requires SP02320)

# SECTION 00350 - GEOSYNTHETIC INSTALLATION

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Comply with Section 00350 of the Standard Specifications modified as follows:

00350.01  Definitions - Replace the sentence that begins “Embankment Geotextile - For installation…“ with the following sentence:

Embankment Geotextile - Embankment geotextile is used as a reinforcement within embankments and as a separation and reinforcement under embankments.

Replace the bullet that begins “Nonwoven Geotextile - A textile…“ with the following bullet:

Nonwoven Geotextile - A textile produced by bonding or interlocking of fibers by mechanical, heat or chemical means.

Replace the sentence that begins “Riprap Geotextile - For installation…“ with the following sentence:

Riprap Geotextile - Riprap geotextile is used as a filter and separator behind or beneath riprap, Buttresses, inlays, shear keys and erosion control applications.

Replace the sentence that begins “Subgrade Geotextile - For installation…“ with the following sentence:

Subgrade Geotextile - Subgrade geotextile is used as a separator and reinforcement on Subgrades and in other material separation applications.

00350.41(f)(5)  Geotextile Placement - Replace the paragraph that begins “Slit wrinkles or folds …“ with the following paragraph:

Slit wrinkles or folds exceeding 1 inch and lay flat. Shingle-lap not more than 6 inches in the direction of the paving. Broom or squeegee to smooth the geotextile and pneumatic roll to maximize geotextile contact with the Pavement surface. Additional hand-placed sealant material may be required at laps as determined.