SP00411  (Special Provisions for the 2021 Book) 	(Bidding on or after: 05-01-21

Last updated: 01-25-21)

SECTION 00411 - PIPE BURSTING AND SLIP LINING

This Section title is replaced with the following:

# SECTION 00411 - PIPE BURSTING

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Comply with Section 00411 of the Standard Specifications modified as follows:

00411.00  Scope - Replace this subsection, except for the subsection number and title, with the following:

This Work consists of furnishing and installing high density polyethylene (HDPE) pipe in gravity sewer pipe by the pipe bursting method.

00411.01  Submittals - Replace the bullet that begins “Installation plan that includes the method…” with the following bullet:

Installation plan that includes the method of installation, sequencing, lubricant, and temporary modification of existing Structures,

Delete the bullet that begins “Method of slip lining…”.

Delete the bullet that begins “Contingency repair plan…”.

Delete the bullet that begins “Host pipe point repair…”.

00411.41  Receiving Pits and Insertion Pits - Replace the paragraph that begins “Locate all pits to suit…” with the following paragraph:

Locate all pits to suit the specified pipe bursting operation. Use existing manholes where practicable. Remove manhole inverts, benches, and channels to permit access for installation Equipment. Enlarge the input and output pipe openings if required to accommodate the maximum outside diameter size of the insertion Equipment. Do not put undue stress on existing Structures.

00411.42(a)  Pipe Bursting - Add the following sentence to the end of the paragraph:

Do not score or damage the liner pipe during the installation process.

00411.42(b)  Slip Lining - Delete this subsection.

00411.80  Measurement - Replace the paragraph that begins “The quantities of pipe bursting…” with the following paragraph:

The quantities of pipe bursting will be measured on the length basis. The length will be measured, with no deduction for Structures or fittings, along the pipe flow line from center to center of manholes, inlets, Structures, special sections, or the ends of pipe, whichever is applicable.

00411.90  Payment - Delete Pay Item (b) from the Pay Item list

Replace the paragraph that begins “In Items (a) and (b)…” with the following paragraph:

In item (a), the nominal size of the new pipe will be inserted in the blank.