SP00590  (Special Provisions for the 2021 Book) 	(Bidding on or after: 06-01-22

Last updated: 03-01-22

This Section requires SP00504 & SP00730)

# SECTION 00590 - POLYMER MEMBRANE

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

(Use this specification only for cast-in-place decks. Obtain information from the Bridge Designer.)

Comply with Section 00590 of the Standard Specifications modified as follows:

00590.10(d)  Tack Coat - Replace this subsection, except for the subsection number and title, with the following:

Furnish Emulsified Asphalt tack coat according to Section 00730.

00590.40(e)  Daily Report Requirements - Replace the bullet that begins “Total gallons of hot asphalt cement…” with the following bullet:

Total gallons of Emulsified Asphalt tack coat applied to each Bridge.

00590.46  ACP Paving - Replace the sentence that begins “Before placing ACP, apply a hot asphalt cement…” with the following sentence:

Before placing ACP, apply an Emulsified Asphalt tack coat at the rates specified in Section 00730 to the entire membrane surface, extending 2 inches up the face of the curb or rail.

00590.90  Payment - Replace the sentence that begins “No separate or additional payment will be made …” with the following sentence:

No separate or additional payment will be made for Emulsified Asphalt tack coat, or for performing Work for the inclement weather plan according to 00590.40(c).

ACP will be paid for according to 00744.90 or 00745.90 as applicable.