SP01069  (Special Provisions for the 2021 Book) 	(Bidding on or after: 10-01-22

Last updated: 06-09-22

Requires SP00440, Requires SP02830

for Metal Handrail and SP02831

for Pedestrian Fence)

# SECTION 01069 - METAL HANDRAIL AND PEDESTRIAN FENCE

(Follow all instructions and make all edits with “Track Changes” turned on. This Section is not published in the Oregon Standard. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the project, unless the item(s) that are included in the subsection, paragraph, sentence, or bullet are not required on the Project and then they should be deleted. In general do not re-number or re-letter subsections when item(s) are deleted. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Section 01069, which is not a Standard Specification, is included in this Project by Special Provision.

## Description

01069.00  Scope - This Work consists of furnishing and installing metal handrails and pedestrian rail units as shown or directed.

## Materials

01069.10  Materials - Furnish Materials meeting the following requirements:

Commercial Grade Concrete	00440

Metal Handrail	02830

Pedestrian Fence	02831

## Construction

01069.40  Metal Handrail and Pedestrian Fence:

(a)  Handrail - Fabricate and install imbedded and bolted down metal handrail as shown.

(b)  Pedestrian Fence - Fabricate and install pedestrian fence units as shown.

01069.41  Welding - Welding, welder qualifications, prequalification of weld details and inspection of welds shall conform to AWS D1.1 or AWS D1.2. Submit all welding procedure specifications 7 Days prior to fabrication to the Engineer for approval.

01069.42  Concrete Footings - Dimensions of footings shall not be less than shown and shall fill the excavated areas. Place the concrete with contact against firm Soil at the sides and bottom and tamp around the posts and brace ends after the posts and braces have been brought to and firmly held in proper position. Strike off, slope or crown and smooth the surface of the concrete at the ground level to shed water. Allow to cure for at least 5 Calendar Days before subjecting the posts to strain.

Excavate for concrete footings to reasonably Neat Lines, but not less than the specified dimensions in Soil, or not less than 18 inches deep in Rock. Prevent disturbance of original ground at the sides and bottom of the excavation.

Dispose of Materials removed under these provisions, including excess excavation, in a satisfactory manner.

01069.43  Bolt Holes:

(a)  Punched Holes - Use a die with a diameter not exceeding the diameter of the punch by more than 1/16 inch. Ream holes that are required to be enlarged to admit the anchor bolts. Make clean cut holes without torn or ragged edges.

(b)  Accuracy of Punched Holes - After punching the holes in the plate, stack the plates with the edges even and insert a cylindrical pin, 1/8 inch smaller in diameter than the nominal size of the punched hole, through the punched holes perpendicular to the face of the plate. No drifting of the rod while passing through each of the punched holes in the stack is allowed. Ensure that the edges of the stack stay in alignment. Non-conforming pieces will be rejected.

## Measurement

01069.80  Measurement - The quantities of Work performed under this Section will be measured according to the following:

Length Basis - Metal handrail will be measured on the length basis, by measuring along the top rail member, from center of end post to center of end post.

Unit Basis - Pedestrian fence will be measured on the unit basis. Pedestrian fence will be counted for each 62 inch long unit.

## Payment

01069.90  Payment - The accepted quantities of Work performed under this Section will be paid for at the Contract unit price, per unit of measurement, for the following items:

Pay Item 	Unit of Measurement

(a)	Metal Handrail, _____ Rails	Foot

(b)	Pedestrian Fence	Each

In item (a), the number of rails will be inserted in the blank.

Payment will be payment in full for furnishing and placing all Materials, and for furnishing all Equipment, labor, and Incidentals necessary to complete the Work as specified.