SP02320  (Special Provisions for the 2021 Book) 	(Bidding on or after: 12-01-22

Last updated: 09-01-22)

# SECTION 02320 - GEOSYNTHETICS

(Follow all instructions and make all edits with “Track Changes” turned on. This Section is not published in the Oregon Standard. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the project, unless the item(s) that are included in the subsection, paragraph, sentence, or bullet are not required on the Project and then they should be deleted. In general do not re-number or re-letter subsections when item(s) are deleted. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Comply with Section 02320 of the Standard Specifications modified as follows:

02320.10(a)(1)  Geotextiles - Add the following bullet to the beginning of the bullet list:

QPL approved for the intended application.

02320.10(b)  Acceptance Requirements - Replace this subsection with the following subsection:

02320.10(b)  Identification - Identify geotextiles by the product name printed directly on the geotextile by the Manufacturer. For all other geosynthetics and when geotextiles are not marked with a product name, identify geosynthetics by the product label attached to the original packaging or the geosynthetic itself by the Manufacturer.

Allow the Engineer to visually verify geosynthetic products before installation. Open packaged geosynthetics before use in the presence of the Engineer to confirm the correct product. Geotextile rolls without the product name printed on the geotextile or the product label affixed to the geotextile or roll core by the Manufacturer will be rejected. Any other geosynthetics that are unwrapped, missing original packaging or previously opened may not be used unless approved by the Engineer.

02320.10(c)(1)  Geotextiles - Replace this subsection, except for the subsection number and title, with the following:

Geotextile products listed in the QPL that are identified as “NTPEP listed” in the remarks column have been approved based on participation in the AASHTO National Transportation Product Evaluation Program (NTPEP) and test data from the program. Manufacturer’s test certification is not required for NTPEP listed geotextiles from the QPL. For other geotextiles, include the following unless directed otherwise:

QPL product category and proposed project application.

Product name printed directly on the geotextile by the Manufacturer. For geotextiles that are not marked with a product name, provide geotextile with product label attached to the geotextile or original packaging by the Manufacturer.

Manufacturer’s name, lot number, roll number, production facility address, and full product information (style, brand, name, etc.).

Chemical composition of filaments and yarns, including polymer(s) used.

Minimum average roll values for each of the specified properties from the same lot of geotextiles as the delivered material.