SP02926  (Special Provisions for the 2021 Book) 	(Bidding on or after: 11-01-21

Last updated: 07-22-21)

# SECTION 02926 - HIGHWAY ILLUMINATION MATERIALS

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Comply with Section 02926 of the Standard Specifications modified as follows:

Add following subsection:

02926.41(f)  Electrical Splice Materials - Furnish electrical splice materials meeting the following requirements:

Split bolt - Made of silicon bronze to securely join the wires both mechanically and electrically.

Heat-shrink tubing - Split-resistant and adhesive-lined tube made of polyolefin complying with UL 224 or UL 486D, temperature range -67 °F to 230 °F, with 600 V rated inner melting wall or liner to provide void-free encapsulated insulation.

Insulating rubber tape - Electrical grade, nondrying, rubber based, elastic type complying with ASTM D4388.

Insulating vinyl plastic tape - Low temperature (0 °F) resistant, vinyl chloride plastic, electrical insulating tape with pressure-sensitive adhesive. Comply with ASTM D3005.