SP00205  (Special Provisions for the 2024 Book)	(Bidding on or after: 12-01-23

Last updated: 05-23-23)

# SECTION 00205 - FIELD LABORATORY, WEIGHHOUSE, ETC.

(Follow all instructions and make all edits with “Track Changes” turned on. This Section is not published in the Oregon Standard. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the Project, unless the item(s) that are included in the subsection, paragraph, sentence, or bullet are not required on the Project and then they should be deleted. In general do not re-number or re-letter subsections when item(s) are deleted. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Section 00205, which is not a Standard Specification, is included in this Project by Special Provision.

## Description

00205.00  Scope - This Work consists of providing facilities for Agency use to perform testing, weighing and other necessary functions during the course of the Project.

## Materials and Equipment

(Use the following subsection .10 when a contractor furnished field laboratory is required.)

00205.10  Contractor-Furnished Field Laboratory - Provide a leveled field laboratory for Agency use in close proximity to the Contractor's plant at least 5 Calendar Days before Aggregate production, paving, or processing Work begins under the Contract, meeting the safety and health requirements of the Oregon Department of Consumer and Business Services, the Oregon Health Division, the State Fire Marshall, and the following minimum requirements:

Length - 16 feet

Width - 8 feet

Ceiling Height - 7 feet

Floor

Insulation - Walls and ceiling - R-11

Doors - At least one, 3 feet wide, all with locks

Windows:

Four

Adequate for good lighting

Capable of being opened for adequate ventilation

One providing a view of the crushing or processing plant

Interior Walls and Ceiling - White

Counter - 20 feet long, 30 inches wide, 36 inches high, with a durable, smooth surface

Sink - One deep double with adequate supply of cold potable, clear, running water

Electrical Power System:

120/240 V, single phase, 60 A service

Wired according to the National Electrical Code

Electrical Service:

Continuous, 24 hours per Day during crushing and Aggregate mixing operations

For 24 hours following termination or interruption of operations

Electrical Outlets - six duplex

Electrical Light Fixtures - Enough to provide good overall lighting

Heating and Air Conditioning - Adequate to provide suitable heating and cooling

Exhaust Ventilation System - Adequate for all activities performed in the laboratory

Toilet - One, portable, for use by Agency employees

The site will be approved by the Engineer before Work is to begin.

Remove the field laboratory when the Project is complete.

(Use the following subsection .11 when an Agency furnished field laboratory is required.)

00205.11  Agency-Furnished Field Laboratory - Provide a level site for an Agency-furnished laboratory trailer at least 5 Calendar Days before Aggregate production, paving, or processing Work begins under the Contract, at a location in close proximity to the Contractor's plant. The site will be approved by the Engineer before Work is to begin.

Employ a commercial hauler to bring the trailer to the Project, relocate at the site if necessary, and to return the trailer to its storage area. Employ a licensed electrician to connect and disconnect the power source. Provide an adequate supply of potable water, electricity, and a portable toilet for use by Agency employees, according to 00205.10.

(Use the following subsection .12 when a Weighouse is required.)

00205.12  Weighhouse - When Materials are weighed on platform-type scales or by other means and the Contractor chooses to have the Agency provide a weigher, provide a weatherproof weighhouse or other approved shelter for the weighperson. The weighhouse shall meet the safety and health requirements of the Oregon Department of Consumer and Business Services, the Oregon Health Division, the State Fire Marshall and the following minimum requirements:

Length - 9 feet

Width - 6 feet

Ceiling Height - 7 feet

Floor

Protect scale recording device and Agency's weigher from weather

Provide space to store scale testing Equipment

Provide adequate shelf space

Provide artificial lighting and good visibility throughout

Provide adequate heat, as required

Provide stool and other facilities for keeping records and performing other duties of the weigher

Doors - At least one, all with locks

Windows:

Capable of being opened for adequate ventilation, unless air conditioning is provided

One facing the scale

One at each end

Adequate size and position to permit view of loading operations and movements of hauling vehicles

Toilet - One, portable, for use by Agency employees

Remove the weighhouse when the Project is complete.

(Use the following subsection .13 when the contractor is required to provide a job inspector trailer for Agency use.]

00205.13  Job Inspector Trailer - At least 15 Calendar Days before beginning On-Site Work, or as otherwise agreed to by the Engineer, provide a leveled office trailer for sole use by the Agency in close proximity to the Project and the Contractor’s trailer, if one is present. Obtain approval of the trailer and the proposed trailer location from the Engineer before delivery and installation. Provide an inspector trailer that meets the safety and health requirements of the Oregon Department of Consumer and Business Services, the Oregon Health Division, the State Fire Marshall, and the following requirements:

Be at least 20 feet in length.

Be at least 8 feet in width.

Have a minimum ceiling height of 7 feet.

At a minimum, be insulated in the walls, ceiling, and floor.

Have at least one exterior door with locks. Provide the Engineer with two sets of keys to the exterior door locks.

Have at least two windows that:

Provide adequate lighting to the interior of the trailer.

Can be opened for ventilation.

Have a light colored ceiling and light colored interior walls.

Have a counter across one end that is a minimum of 30 inches wide and 30 to 42 inches high with a durable smooth surface on top. Alternately, provide a similarly sized table or desk.

Be wired according to the National Electrical Code with 120/240 volt, single phase, 60 amp minimum service.

Have electrical service 24 hours per Day for the duration of the Project. The Engineer may waive the continuous requirement if it is unavailable.

Have high-speed internet service 24 hours per Day for the duration of the Project, at no additional cost to the Agency. The Engineer may waive the high speed requirements if it is unavailable

Have at least four, evenly spaced, 120 volt duplex electrical outlets.

Have adequate electrical light fixtures to provide good overall interior lighting.

Have an adequate heater and air conditioning unit to maintain a temperature of 68 to 72 °F.

Have adequate parking for two Agency vehicles, near the trailer, on an aggregate pad or other surface free of mud and standing water.

Have a walkway between the Agency vehicle parking area and the trailer, free of mud and standing water.

Have steps with a handrail at each exterior door.

Provide and maintain a portable toilet for use by Agency employees.

Similar facilities, such as Agency space in a shared trailer or space in a building close to the Project, may be allowed if approved by the Engineer.

Remove the job inspector trailer when the Project is complete.

(Use the following subsection .14 when the contractor is required to provide a job office trailer for Agency use.]

00205.14   Job Office Trailer - At least 15 Calendar Days before beginning On-Site Work, or as otherwise agreed to by the Engineer, provide a leveled office trailer for sole use by the Agency in close proximity to the Project and the Contractor’s trailer, if one is present. Obtain approval of the trailer and the proposed trailer location from the Engineer before delivery and installation  Provide an office trailer that meets the safety and health requirements of the Oregon Department of Consumer and Business Services, the Oregon Health Division, the State Fire Marshall, and the following requirements:

Be at least 28 feet in length.

Be at least 8 feet in width.

Have a minimum ceiling height of 7 feet.

At a minimum, be insulated in the walls, ceiling, and floor with R 11 insulation.

Have at least one 3 foot wide exterior door with locks. Provide the Engineer with two sets of keys to the exterior door locks.

Have at least two windows, each with an approximate minimum size of 46 by 30 inches, that:

Provide adequate lighting to the interior of the trailer.

Are equipped with blinds or other window coverings, as approved.

Can be opened for ventilation.

Have a light colored ceiling and light colored interior walls.

Have a counter across one end that is an approximate minimum of 30 inches wide and 30 to 42 inches high with a durable smooth surface on top.

Be wired according to the National Electrical Code with 120/240 volt, single phase, 60 amp minimum service.

Have electrical service 24 hours per Day for the duration of the Project. The Engineer may waive the continuous requirement if it is unavailable.

Have at least four, evenly spaced, 120 volt duplex electrical outlets.

Have adequate electrical light fixtures to provide good overall interior lighting.

Have an adequate heater and air conditioning unit to maintain a temperature of 68 to 72 °F.

Have adequate parking for two Agency vehicles, near the trailer, on an aggregate pad or other surface free of mud and standing water.

Have a walkway between the Agency vehicle parking area and the trailer, free of mud and standing water.

Have a minimum of two telephone services, each with local and long distance service at no additional cost to the Agency, a phone number specific to the service, and continuous service at all times during the Project. The Engineer may waive some telephone requirements if they are unavailable.

Have high -speed internet service 24 hours per Day for the duration of the Project, at no additional cost to the Agency. The Engineer may waive the high speed requirements if it is unavailable.

Have steps with a handrail at each exterior door.

Provide and maintain a portable toilet for use by the Agency employees.

Similar facilities, such as Agency space in a shared trailer or space in a building close to the Project, may be allowed if approved by the Engineer.

Remove the job office trailer when the Project is complete.

## Measurement

00205.80  Measurement - The quantities of Work performed under this Section will be measured according to the following:

(Use the following subsection (a) when a Contractor-Furnished field laboratory is required.)

(a)  Contractor-Furnished Field Laboratory - Contractor-furnished field laboratories will be measured on the unit basis.

(Use the following subsection (b) when an Agency-Furnished field laboratory is required.)

(b)  Agency-Furnished Field Laboratory - Agency-furnished field laboratory Work described in 00205.11 will be measured on the unit basis for each Agency-furnished field laboratory required to be located at the Contractor's site of operations.

(Use the following subsection (c) when the contractor is required to provide a job inspector trailer for Agency use.]

(c)  Job Inspector Trailer - Job inspector trailers will be measured on the unit basis.

(Use the following subsection (d) when the contractor is required to provide a job office trailer for Agency use]

(d)  Job Office Trailer - Job office trailers will be measured on the unit basis.

(Use the following subsection (e) when a weighhouse is required.)

(e)  Weighhouse - No measurement will be made for providing weighhouses.

## Payment

00205.90  Payment - The accepted quantities of Work performed under this Section will be paid for at the Contract unit price, per unit of measurement, for the following items:

(Delete Pay Items from the list that are not included in the Schedule of Items, but do not change the alpha characters next to the Pay Items.)

Pay Item	Unit of Measurement

(a)	Contractor-Furnished Field Laboratory	Each

(b)	Agency-Furnished Field Laboratory	Each

(c)	Job Inspector Trailer	Each

(d)	Job Office Trailer	Each

(Use the following paragraph when a Contractor-Furnished field laboratory, job inspector trailer, or job office trailer is required.)

Payment will be payment in full for furnishing facilities for Agency use, and for furnishing all Equipment, labor, and Incidentals necessary to complete the Work as specified.

(Use the following paragraph when an Agency-Furnished field laboratory is required.)

Payment will be payment in full for furnishing all Equipment, labor, and Incidentals necessary to complete the Work as specified.

(Use the following paragraph when a weighhouse is required.)

No separate or additional payment will be made for providing weighhouses. Payment will be included in payment made for the appropriate items under which this item is required.