SP00225  (Special Provisions for the 2024 Book) 	(Bidding on or after: 12-01-23

Last updated: 05-19-23)

# SECTION 00225 - TEMPORARY PAVEMENT MARKINGS

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

(Use the following lead-in paragraph when none of the following subsections are included in the project.)

Comply with Section 00225 of the Standard Specifications.

(Use the following lead-in paragraph when any of the following subsections are included in the project.)

Comply with Section 00225 of the Standard Specifications modified as follows:

(Use the following subsection .40 on Emulsified Asphalt Chip Seal (Section 00710 or 00715) projects, or on Level 1, 2, or 3 ACP Overlay (00744 or 00745) Preservation projects)

00225.40  Temporary Pavement Markers - Replace the paragraph that begins "Unless otherwise shown..." and the three bullets with the following paragraphs and bullets:

(Use one of the following two options as instructed. Delete the option that does not apply.)

[ Option 1 - Use the following paragraphs and bullet lists on Emulsified Asphalt Chip Seal (00710 or 00715) projects, provided the following criteria are met:

Compile Field Data Summary

ADT < 5,000 for roadways with posted speed > 45 mph

ADT < 10,000 for roadways with posted speed ≤ 45 mph

Federally funded projects require FHWA approval ]

Install temporary flexible oiling pavement markers for temporary centerline marking as follows:

Place and maintain one temporary flexible oiling pavement marker on 40-foot spacing in tangent and curve sections except as below.

Place and maintain one temporary flexible oiling pavement marker on 20-foot spacing in curved alignment sections identified by a speed rider displaying less than the posted speed and channelization areas.

Establish alignment for placing the temporary flexible oiling pavement markers as follows:

Control markers at:

200-foot intervals on tangents

50-foot intervals on curves

40-foot intervals on curves with speed rider

Use string line or other appropriate means to maintain proper alignment of the markers. Adjust placement to avoid straddling a longitudinal joint, while maintaining a suitable alignment of markers.

Remove and replace misaligned markers at no additional cost to the Agency.

[ End Option 1 ]

[ Option 2 - Use the following paragraphs and bullet lists on Level 1, 2, or 3 ACP Overlay (00744 or 00745) Preservation projects, provided the following criteria are met:

Perform and/or document enough traffic analysis to confirm traffic volumes meet the following criteria:

ADT < 5,000 for roadways with posted speed > 45 mph

ADT < 10,000 for roadways with posted speed ≤ 45 mph ]

Install temporary flexible overlay pavement markers for temporary centerline marking as follows:

Place and maintain one temporary flexible overlay pavement marker on 40 foot spacing in tangent and curve sections except as below.

Place and maintain one temporary flexible overlay pavement marker on 20 foot spacing in curved alignment sections identified by a speed rider displaying less than the posted speed and channelization areas.

Establish alignment for placing the temporary flexible overlay pavement markers as follows:

Control markers at:

200 foot intervals on tangents

50 foot intervals on curves

40 foot intervals on curves with speed rider

Use string line or other appropriate means to maintain proper alignment of the markers. Adjust placement to avoid straddling a longitudinal joint, while maintaining a suitable alignment of markers.

Remove and replace misaligned markers at no additional cost to the Agency.

[ End Option 2 ]

(Use the following subsection .40(b) on Emulsified Asphalt Chip Seal (00710 or 00715) projects, provided the following criteria are met:

Compile Field Data Summary

ADT < 5,000 for roadways with posted speed > 45 mph

ADT < 10,000 for roadways with posted speed ≤ 45 mph

Federally funded projects require FHWA approval)

00225.40(b)  Flexible Oiling Pavement Markers - Add the following paragraphs to the end of this subsection:

Remove the markers on underlying surfaces before placing the next full Lift of paving that would obliterate the marker.

Use flexible oiling pavement markers with double covers, as directed, when a surface treatment requires two oil applications for a single Pavement Lift. Remove the marker covers, as directed, before reopening the Roadway to traffic.