SP00759  (Special Provisions for the 2024 Book) 	(Bidding on or after: 12-01-23

Last updated: 05-24-23)

# SECTION 00759 - MISCELLANEOUS PORTLAND CEMENT CONCRETE STRUCTURES

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the Project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

(Use the following lead-in paragraph when none of the following subsections are included in the Project.)

Comply with Section 00759 of the Standard Specifications.

(Use the following lead-in paragraph when any of the following subsections are included in the Project.)

Comply with Section 00759 of the Standard Specifications modified as follows:

(Use the following lead-in paragraph and subsection .13 when detectable guide strips are required.)

Add the following subsection:

00759.13  Detectable Guide Strips - Furnish surface applied or liquid applied detectable guide strips. Use only adhesives recommended or supplied by the manufacturer. Furnish slip-resistant detectable guide strips from one of the following list of pre-approved products:

Manufacturer	Material

Vanguard ADA Systems	Liquid Applied GuideStrip / TWSI

18122 SR9 Suite F 	Color: Blue

Snohomish, WA 98296

Phone: (360) 668-5700

(Use the following subsection .50(a) when detectable guide strips are required.)

00759.50(a)  General - Add the following paragraph to the end of this subsection:

Install detectable guide strips as shown. Place according to the manufacturer's recommendation. Place abutting panels within 1/4-inch of each other and install anchors, as specified by manufacturers, along cut edge. Miter adjacent panels at locations where detectable guide strips change direction. Install detectable guide strips on clean and dry concrete surfaces.

(Use the following subsection .51 when detectable guide strips are required.)

00759.51  Curing - Add the following paragraph to the end of this subsection:

Do not apply curing compounds to areas designated for detectable guide strip installation.

(Use the following subsection .90 when detectable guide strips are required.)

00759.90  Payment -

Add the following Pay Item to the Pay Item list:

(o)	Detectable Guide Strips, Blue	Square Foot

Item (o) includes installation of blue detectable guide strips on a concrete or asphalt surface.