SP00905  (Special Provisions for the 2024 Book) 	(Bidding on or after: 12-01-23

Last updated: 05-24-23)

# SECTION 00905 - REMOVAL AND REINSTALLATION OF EXISTING SIGNS

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the Project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

(Use the following lead-in paragraph when none of the following subsections are included in the Project.)

Comply with Section 00905 of the Standard Specifications.

(Use the following lead-in paragraph when any of the following subsections are included in the Project.)

Comply with Section 00905 of the Standard Specifications modified as follows:

(Use the following "Modify and Repair Language" when modifying or repairing extruded aluminum panel signs and sign legends.

[ Begin "Modify and Repair Language" ]

00905.00  Scope - In the paragraph that begins "This Work consists of…" replace the word "both" with the word "more".

Add the following bullet to the end of the bullet list:

Modifying signs, modifying sign legends, and repairing existing signs as shown.

Add the following subsection:

00905.10  Materials - Furnish materials for modifying signs and legends, and for repairing signs, meeting the requirements of Section 02910.

Add the following subsection:

00905.41  Modify Legends - Remove existing removable legends and install new legends as shown and according to 00940.45.  Fill all rivet holes not covered by new legends with new blind rivets.  Do not damage background sheeting and substrates.

Add the following subsection:

00905.42  Modify Signs - Modify signs as shown and according to Section 00940.

00905.90  Payment - Add the following Pay Item to the end of the Pay Item list:

(c)	Modify Existing Signs and Legends	Lump Sum

[ End "Modify and Repair Language" ]