SP01030  (Special Provisions for the 2024 Book) 	(Bidding on or after: 12-01-23

Last updated: 08-08-23

Requires SP01040 when

soil testing or planting area

preparation is required.)

# SECTION 01030 - SEEDING

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the Project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Comply with Section 01030 of the Standard Specifications modified as follows:

(Use the following subsection .13(a) when alternate labeling for native plant seeds is required. Obtain information from the Erosion Control Designer.)

01030.13(a)  Label - Add the following to the end of this subsection:

Provide alternate labeling for native plant seeds as follows:

(Use the following subsection .13(f) to include seed mix formulas. Obtain the information from the Erosion Control Designer. Delete what does not apply. Detail seed mixes according to the following instructions:

Use the format in .13(f) which follows. Show seeding categories as needed: Permanent Seeding, Temporary Seeding, Wildflower Seeding, Plant Seeding, Water Quality Seeding, Wetland Seeding, or Native Plant Seeding. To add more categories, blank columns, or lines, copy from the blank seeding category below.

Fill in the botanical and common names and the PLS seeding rate. For Oregon Certified Seed, show an asterisk (*) at the beginning of the botanical name.

For several mixes within the same category, use (Category Name) Seed Mix No. 1, No. 2, etc. and each mix will be a bid item. When using these numbered mixes, a brief mix description may also be added that doesn’t show in the bid item name. An example is Wetland Seeding, Mix No. 1 (Emergent Areas).)

When more than one seed mix needs to be specified but quantities seem too small to be more than one bid item, this format can be used: Under a single bid item such as Wetland Seeding, detail multiple mixes by showing Seed Mix A, Seed Mix B, etc. Give an approximate quantity for each mix that equals the quantity of the bid item.)

01030.13(f)  Types of Seed Mixes - Add the following to the end of this subsection:

Provide the following seed mix formulas:

__________________ Seeding:

Botanical Name	PLS Specified Rate

(Common Name)	(lb/acre)

_______________

(______________)	____

_______________

(______________)	____

_______________

(______________)	____

_______________

(______________)	____

_______________

(______________)	____

* Oregon Certified Seed

Lawn Seeding:

Name	PLS Specified Rate

(lb/acre)

Fine Fescue**	____

Perennial Ryegrass**	____

Kentucky Bluegrass**	____

Colonial Bentgrass**	____

** Furnish strong growing, site appropriate, disease resistant varieties

(Use the following lead-in paragraph and subsection .14(b)(4) when organic fertilizer is required. Obtain information from the Erosion Control Designer.)

Add the following subsection:

01030.14(b)(4)  Organic Fertilizer - Furnish organic fertilizer that analyzes __ % nitrogen, __ % phosphoric acid, and __ % soluble potash. Furnish fertilizer that has no toxicity to sites where it will be applied.

(Use the following subsection .15 and bullet(s) to specify other types of mulch not listed in the Standard Specifications, or when hydromulch may be used for temporary erosion control seeding. Delete "(s)" or parentheses as applicable, such as composted yard debris. Use the generic name for the mulch and include Specifications, or cite the ODOT QPL if applicable. Obtain information from the Erosion Control Designer.)

01030.15  Mulch - Add the following paragraph(s) and bullets to the end of this subsection:

Use the following paragraph and bullet(s) to specify other types of mulch not listed in the Standard Specifications, such as composted yard debris. Use the generic name for the mulch and include Specifications, or cite the ODOT QPL if applicable. Obtain information from the Erosion Control Designer.)

Furnish mulch for seeding according to the following:

(Use the following paragraph and bullets when hydromulch may be used for temporary erosion control seeding. Obtain the Erosion Control Designer's approval before using.)

Furnish straw mulch for all temporary roadside erosion control seeding, except hydromulch may be used under the following conditions:

Spring planting west of the Cascades between March 1 and May 15.

Slopes are steeper than 1V to 1.5H and longer than 16 feet.

Residential or commercial sites with low erosion potential such as sidewalk, Median, or parking lot planter strips.

Projects that have variable slopes may include straw mulch and hydromulch when approved.

(Use the following subsection .17 when the Federal Aid Highway Program Biological Opinion is applicable to the project and herbicide use will be allowed. Coordinate with the Region Environmental Coordinator and include the weed control pay item.)

01030.17  Pesticides - Add the following to the end of this subsection:

Do not apply herbicides that include any herbicide carrier (i.e. solvent) other than water or vegetable oil. Utilize only the following herbicides for Project Work:

aquatic imazapyr

aquatic glyphosate

aquatic triclopyr-TEA

chlorsulfuron

clopyralid

imazapic

imazapyr

metsulfuron-methyl

picloram

sethoxydim

sulfometuron-methyl

Apply only the herbicide adjuvants described in Table 01030-1 for Project Work.

Table 01030-1

| Adjuvant Type | Trade Name |
| --- | --- |
| Surfactant | Agri-Dex |
| Surfactant | LI 700 |
| Drift Retardant | 41-A |
| Drift Retardant | Vale |

(Use the following subsection .42 to list Specified Weeds and plant species to be removed. Obtain list from the Erosion Control Designer.)

01030.42  Weed Control - Add the following paragraph and bullets after the paragraph that begins "If a pesticide has been approved for…" and before subsection (a):

The Specified Weeds and plant species to be removed include the following:

(Use the following lead-in paragraph and subsection .42(e) and table(s) when the Federal Aid Highway Program Biological Opinion is applicable to the project and herbicide use will be allowed.)

Add the following subsection:

01030.42(e)  Herbicide Application - Obtain written approval from the Engineer prior to application of herbicides in sensitive areas. Apply herbicides at the lowest effective label rates. Mix herbicides a minimum of 150 feet away from natural water bodies. Wash spray tanks a minimum of 300 feet from any natural water body. Utilize a non-hazardous indicator dye during herbicide applications within 100 feet of natural water bodies. Keep spray nozzles as low as possible and utilize the largest droplet size possible in order to minimize drift. Provide documentation of wind speed and direction, air temperature, ground temperature, and date of application to the Engineer within 5 Days of broadcast herbicide applications.

Do not apply herbicides during the following conditions:

When soil is saturated, except for soil-activated herbicides.

Within 48 hours prior to and during a forecasted rain event of 0.5 inches or greater in a 24-hour period.

When wind speeds exceed 10 miles per hour, or are less than 2 miles per hour.

During temperature inversions.

When ground temperatures exceed 80 °F.

Adhere to the adjuvant application limitations in Table 01030-2. Regulated work area (RWA) is defined in 00290.34(a).

Table 01030-2

| Adjuvant Type | Trade Name | Limitations |
| --- | --- | --- |
| Surfactant | Agri-Dex | Do not apply within RWA |
| Surfactant | LI 700 | Do not apply within RWA |
| Drift Retardant | 41-A | Do not apply within RWA |
| Drift Retardant | Vale | Do not apply within 150 feet of natural water bodies |

During herbicide application, adhere to the buffer distance requirements in Table 01030-3.

Table 01030-3

| Herbicide | Application Buffer Width (feet) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Perennial Streams and Wetlands, and Intermittent Streams and Roadside Ditches with flowing or standing water present |  |  | Dry Intermittent Streams, Dry Intermittent Wetlands, Dry Roadside Ditches |  |  |
|  | Broadcast Spraying | Spot Spraying | Hand Selective | Broadcast Spraying | Spot Spraying | Hand Selective |
| Labeled for Aquatic Use |  |  |  |  |  |  |
| aquatic glyphosate | 100 | Waterline | Waterline | 50 | No buffer | No buffer |
| aquatic imazapyr | 100 | 15 | Waterline | 50 | No buffer | No buffer |
| aquatic triclopyr-TEA | Not Allowed | 15 | Waterline | Not Allowed | No buffer | No buffer |
| Low Risk to Aquatic Organism |  |  |  |  |  |  |
| Imazapic | 100 | 15 | RWA boundary | 50 | No buffer | No buffer |
| Clopyralid | 100 | 15 | RWA boundary | 50 | No buffer | No buffer |
| metsulfuron-methyl | 100 | 15 | RWA boundary | 50 | No buffer | No buffer |
| Moderate Risk to Aquatic Organism |  |  |  |  |  |  |
| Imazapic | 100 | 50 | RWA boundary | 50 | 15 | RWA boundary |
| sulfometuron-methyl | 100 | 50 | 5 | 50 | 15 | RWA boundary |
| Chlorsulfuron | 100 | 50 | RWA boundary | 50 | 15 | RWA boundary |
| High Risk to Aquatic Organism |  |  |  |  |  |  |
| Picloram | 100 | 50 | 50 | 100 | 50 | 50 |
| Sethoxydim | 100 | 50 | 50 | 100 | 50 | 50 |

(Use the following lead-in paragraph and subsection .44(c) when organic fertilizer is required. Obtain information from the Erosion Control Designer.)

Add the following subsection:

01030.44(c)  Organic Fertilizer - Apply organic fertilizer at a rate of ___ pounds per acre at the following locations:

(Use the following subsection .60 when the "Plant Seeding, ____" or "Native Plant Seeding, ____" Pay Items are included in the Schedule of Items. Delete "(s)" or the parentheses as applicable.)

01030.60  General - Add the following sentence(s) after the last bullet:

(Use the following paragraph when the "Plant Seeding, ____" Pay Item is included in the Schedule of Items. Obtain information from the Erosion Control Designer and fill in the blank.)

The minimum living plant coverage for woody or other plant seeding is ____ percent of ground surface.

(Use the following paragraph when the "Native Plant Seeding, ____" Pay Item is included in the Schedule of Items. Obtain information from the Erosion Control Designer and fill in the blank.)

The minimum living plant coverage for native plant seeding is ____ percent of ground surface.