SP00589  (Special Provisions for the 2024 Book) 	(Bidding on or after: 05-01-25

Last updated: 01-23-25)

# SECTION 00589 - UTILITY ATTACHMENTS ON STRUCTURES

(Follow all instructions and make all edits with “Track Changes” turned on. This Section is not published in the Oregon Standard. If there are no instructions [purple text] above a subsection, paragraph, sentence, or bullet, then include it in the Project, unless the item(s) that are included in the subsection, paragraph, sentence, or bullet are not required on the Project and then they should be deleted. In general do not re-number or re-letter subsections when item(s) are deleted. Delete all purple text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Section 00589, which is not a Standard Specification, is included in this Project by Special Provision.

## Description

00589.00  Scope - This Work consists of providing for attachment or installation of utilities on new and existing Structures as shown or as directed.

(Use the following two paragraphs when Structures have been designated "Special Structures" by Bridge Engineering Section. List the designated Special Structures by bridge number.)

The following Structures are designated "Special Structures":

Bridge No. _______

Bridge No. _______

Utility attachments to Special Structures shall not alter the appearance of the Structure. Make attachments only inside the girder line, as close as possible to the soffit, or as required to make the installation as inconspicuous as practicable.

## Materials

00589.10  General - Furnish Utility attachment systems using Materials from the QPL and meeting the following requirements:

Structural Steel 	 02530

Forgings, Shafting, Castings, and Nonferrous Materials 	 02540

Fasteners 	 02560

Reflective Sheeting 	 02910.20(a)

Resin Bonded Anchor System 	 00535.10

Furnish brackets constructed of stainless steel or hot-dip galvanized structural steel.

## Construction

00589.40  General - Provide sufficient space around utilities for maintenance activities.

Avoid drilling through reinforcing steel. If reinforcing steel is hit, move the anchor location and patch the hole with an approved patching material from the QPL.

Attach conduits or brackets to concrete Structures with resin bonded concrete anchors, unless otherwise shown or approved.

00589.41  Natural Gas Lines - Conform to the portions of CFR 49 Part 192 that are applicable to the Work. Provide isolation valves 200 feet from each end of the Bridge as shown.

(Include the following subsection .42 when a casing is requested by the Utility Coordinator. Delete the item from the subsection that does not apply, but do not change the alpha characters next to the title. Fill in the blank with the appropriate information. )

00589.42  Casing –

(a)  Natural Gas Casing - Install steel pipe casing for natural gas lines below bridge deck as shown. Natural gas line will be installed within casing by others.

(b)  Communication Casing - Install (_insert the type of casing as discussed with the Utility Owner_) casings for communications lines below bridge deck as shown. Communication lines will be installed within casings by others.

(Include the following Subsection .43 when a waterline will be attached to a Bridge)

00589.43  Water Line Attachment - Install ductile iron pipe and fittings with restrained joints according to 01140.00 through 01140.52 below the bridge deck as shown and specified.

00589.48  Labeling - Clearly label all piping or conduit systems according to the following APWA color code:

Table 00589-1

| Material | Marker Background Color |
| --- | --- |
| Electrical Power Lines, Cables, Conduits, Lighting Cables | Red |
| Gas, Oil, Steam, Petroleum, Gaseous Materials | Yellow |
| Communications, Alarm, Signal Lines, Cables, or Conduits | Orange |
| Potable Water | Blue |
| Reclaimed Water, Irrigation, Slurry Lines | Purple |
| Sewers, and Drain Lines | Green |

Generate purple by placing purple transparent film over white reflective sheeting. The purple tint of the transparent film shall match Federal Standard Color 595B No. 37100.

Minimum length of label shall be as shown in Table 00589-2.

Table 00589-2

| Pipe O.D. Min. | Pipe O.D. Max. | Length of Label | Width of Label |
| --- | --- | --- | --- |
| 3/4" | 1 1/4" | 8" | 3/4" |
| 1 1/2" | 2" | 8" | 1" |
| 2 1/2" | 6" | 12" | 2" |
| 8" | 10" | 12" | 2" |
| 10" | ─ | 12" | 2" |

Place labels on each pipe or conduit, on each side of every bent, and at each entrance to a box girder.

Where piping is above or below normal line of sight, place pipe labels so that label may be seen from normal eye height.

## Measurement

00589.80  Measurement - No measurement of quantities will be made for Work performed under this Section.

## Payment

00589.90  Payment - The accepted quantities of Work performed under this Section will be paid for at the Contract lump sum amount for the item "Utility Attachment on Structures, ______".

The type of Utility attachment on Structures will be inserted in the blank.

Payment will payment in full for furnishing and placing all Materials, and for furnishing all Equipment, labor, and Incidentals necessary to complete the Work as specified.