SP00755  (Special Provisions for the 2024 Book) 	(Bidding on or after: 05-01-25

Last updated: 01-22-25

Requires SP02050 and SP02510)

# SECTION 00755 - CONTINUOUSLY REINFORCED CONCRETE PAVEMENT

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [purple text] above a subsection, paragraph, sentence, or bullet, then include it in the project. Delete all purple text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

(Use the following lead-in paragraph when none of the following subsections are included in the project.)

Comply with Section 00755 of the Standard Specifications.

(Use the following lead-in paragraph when any of the following subsections are included in the project.)

Comply with Section 00755 of the Standard Specifications modified as follows:

(Use the following subsection when the class of concrete is different than 4000 –1 ½”. Fill in the blank with the class of concrete.)

00755.11  Classes of Concrete - Replace this subsection, except for the subsection number and title, with the following:

Furnish Class __________ paving concrete.

(Use the following subsection .80 when terminal expansion joints are required.)

00755.80  Measurement - Replace the paragraph that begins " The quantities of terminal anchors…" with the following paragraph:

The quantities of terminal anchors, terminal expansion joints, and terminal expansion joint sleeper slab will be measured on the length basis, along the centerline of each anchor and joint as constructed.

(Use the following subsection .90 when pay items (g), or (h) are required. Delete what does not apply.)

00755.90  Payment – Add the following pay item to the pay item list:

(h) Terminal Expansion Joint Sleeper Slab	Foot

Add the following paragraphs to the end of this subsection:

Item (g) includes the dowelled expansion joint.

Item (h) includes excavation for the terminal expansion joint sleeper slab.