SP00759  (Special Provisions for the 2024 Book) 	(Bidding on or after: 05-01-25

Last updated: 01-22-25

Requires SP02510)

# SECTION 00759 - MISCELLANEOUS PORTLAND CEMENT CONCRETE STRUCTURES

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [purple text] above a subsection, paragraph, sentence, or bullet, then include it in the Project. Delete all purple text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Comply with Section 00759 of the Standard Specifications modified as follows:

00759.11  Aggregate Base - Replace this subsection, except for the subsection number

and title, with the following:

Furnish Aggregate Base Materials for Base, foundation courses, Leveling courses, or bedding meeting the requirements of 00640.10 or dense graded Base Aggregate in 00641.10 as appropriate and included in the Special Provisions.

(Use the following lead-in paragraph and subsection .13 when detectable guide strips are required.)

Add the following subsection:

00759.13  Detectable Guide Strips - Furnish surface applied or liquid applied detectable guide strips. Use only adhesives recommended or supplied by the manufacturer. Furnish slip-resistant detectable guide strips from one of the following list of pre-approved products:

Manufacturer	Material

Vanguard ADA Systems	Liquid Applied GuideStrip / TWSI

18122 SR9 Suite F 	Color: Blue

Snohomish, WA 98296

Phone: (360) 668-5700

00759.22(a)  Qualified Smart Levels – Replace this subsection, except for the subsection number and title, with the following:

Slopes will be measured with the use of a 24 inch SmartTool level model 92379 or model 92500, and a 6 inch SmartTool level model 92346 or 92510.

00759.42  Foundations - Replace this subsection with the following subsection:

00759.42  Aggregate Base - Before placing concrete, prepare underlying Aggregate Base surfaces according to Section 00641.

(Use the following subsection .50(a) when detectable guide strips are required.)

00759.50(a)  General - Add the following paragraph to the end of this subsection:

Install detectable guide strips as shown. Place according to the manufacturer's recommendation. Place abutting panels within 1/4-inch of each other and install anchors, as specified by manufacturers, along cut edge. Miter adjacent panels at locations where detectable guide strips change direction. Install detectable guide strips on clean and dry concrete surfaces.

00759.51  Curing - Add the following paragraph to the end of this subsection:

Concrete Structures may be opened to Public Traffic before 7 Calendar Days if the concrete has reached a minimum compressive strength of 2,000 psi as verified by the rebound number determined according to ASTM C805. Test at locations as directed.

(Use the following two paragraphs when detectable guide strips are required.)

Add the following paragraph to the end of this subsection:

Do not apply curing compounds to areas designated for detectable guide strip installation.

(Use the following subsection .90 when any of the paragraphs below are required.)

00759.90  Payment -

(Use the following two paragraphs when earthwork is included as separate Pay Items but the earthwork Pay Items and estimated quantities do not include all or a portion of the Work in this Section. The use of this language requires notation (contact the Senior Standards Engineer) in the Plans to describe the earthwork pay limits relative to SP00330 and requires the State Specification Engineer and RE approval. Consider issuing a LOC.)

Replace the paragraph that begins "When earthwork is included as separate Pay …" with the following paragraph:

When earthwork is included as separate Pay Items under Section 00330, payment will be made according to 00330.90 through 00330.94 as appropriate and only for locations where earthwork quantities are shown. No separate or additional payment will be made for earthwork under Section 00330 where earthwork quantities are not shown.

(Use the following three paragraphs when detectable guide strips are required.)

Add the following Pay Item to the Pay Item list:

(o)	Detectable Guide Strips, Blue	Square Foot

Item (o) includes installation of blue detectable guide strips on a concrete or asphalt surface.