SP00510  (Special Provisions for the 2024 Book) 	(Bidding on or after: 07-01-25

Last updated: 03-11-25)

# SECTION 00510 - STRUCTURE EXCAVATION AND BACKFILL

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [orange text] above a subsection, paragraph, sentence, or bullet, then include it in the Project. Delete all orange text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Comply with Section 00510 of the Standard Specifications modified as follows:

00510.04(a)  Defined Shoring Systems – Replace the sentence that begins “Select Shoring systems…” with the following sentence:

Select Shoring systems for construction from the list of defined Shoring systems provided in Section 16.3.26 of the ODOT GDM.

(Use the following paragraphs and table when shoring is required.)

Add the following to the end of this subsection:

Construct shoring at the location(s) listed below:

(Use the following table to indicate the shoring location and the defined system(s) allowed. Include the beginning and ending stations, use either “Lt.” or “Rt.”, or a separate shoring alignment as required, to locate and provide minimum shoring length, and include the defined type of shoring system type(s) allowed from the list: All, 5A, 5B, 5C, 5D, 5E, 5F, 6A, 6B, 7A. Add or delete rows in the table as necessary to list additional locations and list all applicable defined shoring systems.

Example:

| Beginning Station | Ending Station | Shoring System Type(s) Allowed |
| --- | --- | --- |
| Station 1+55 Rt. | Station 2+45 Rt. | All |
| Station 4+00 Lt | Station 4+40 Lt. | 5A |

)

| Beginning Station | Ending Station | Shoring System Type(s) Allowed |
| --- | --- | --- |
| Station _______(Lt.)(Rt.) | Station_______ (Lt.)(Rt.) |  |

00510.04(b)  Atypical Shoring Systems – Replace the paragraph that begins “Shoring systems that are not on the list…” with the following paragraph:

Shoring systems that are not on the list of Shoring types published in Section 16.2.4.2 of the ODOT GDM are considered atypical Shoring systems. If proposing an atypical Shoring system, submit stamped Working Drawings according to 00150.35. The review and response time allowed for the Agency to return the Working Drawings will be 120 Calendar Days. The submittal of calculations and other data must satisfy the requirements of the Specifications and include sufficient detail and explanation of the design for the Agency to process and comment on the Working Drawings. If the Engineer requests additional information or explanation related to the review of the atypical Shoring system the Engineer may restart the 120-Calendar Day review period.

(Use the following subsection .80(b)(1) when Structure excavation is paid for on the lump sum basis.)

00510.80(b)(1)  Lump Sum - Add the following to the end of this subsection:

The estimated quantity of Structure excavation is:

Location	Structure Excavation

(Cubic Yard)

(Use the following subsections .80(d) when granular wall backfill and/or granular Structure backfill will be measured and paid for on the lump sum basis.)

00510.80(d)(1)  Lump Sum - Add the following to the end of this subsection:

The estimated (quantity)(quantities) of (granular wall backfill) (and) (granular structure backfill) (is)(are):

(Delete headings that do not apply. Obtain quantities from the designer.)

Granular	Granular

Wall Backfill	Structure Backfill

Location	(Cubic Yard)	(Cubic Yard)

(Include the following cofferdam design checklist when required by the Bridge Designer.)

COFFERDAM DESIGN CHECKLIST

Instructions - This cofferdam design checklist was developed to facilitate the design, review, and erection of cofferdams to be used for ODOT bridge construction projects. This checklist is intended to act as a reminder to design or check for specific important aspects of this construction. It is not a substitute for plan and/or design criteria or specification requirements.

The Checklist is to be completed and signed by the cofferdam design engineer. Answer every question. Attach to the Checklist an explanation of any negative responses.

Submit the Checklist according to 00510.03.

| A. Contract Plans, Specifications, Permits, etc. | YES | NO | N/A |
| --- | --- | --- | --- |
| 1. Are the cofferdam Working Drawings prepared, stamped and signed by an engineer registered to practice in Oregon? | ____ | ____ | ____ |
| 2. Have three copies (five copies if railroad approval is required) of the complete design calculations accompanied the cofferdam drawings submittal? | ____ | ____ | ____ |
| 3. Are cofferdam Working Drawings in compliance with the requirements of the construction plans general notes? | ____ | ____ | ____ |
| 4. Are cofferdam Working Drawings in compliance with contract plan structural details? | ____ | ____ | ____ |
| 5. Are cofferdam Working Drawings in compliance with the requirements of the Oregon Standard Specifications for Construction, subsection 00150.35? | ____ | ____ | ____ |
| 6. Are all existing, adjusted or new utilities in proximity with the proposed cofferdam shown on the cofferdam Working Drawings and is projection of these utilities addressed? | ____ | ____ | ____ |
| 7. Are clearance requirements satisfied and shown on the cofferdam Working Drawings? | ____ | ____ | ____ |
| B. Loads | YES | NO | N/A |
| 1. Are the magnitude and location of all loads, equipment and personnel that will be supported by the cofferdam shown noted on the cofferdam Working Drawings? | ____ | ____ | ____ |
| 2. Are design loads and material properties used to determine design stresses shown for each different cofferdam member shown on the cofferdam Working Drawings? | ____ | ____ | ____ |
| 3. Is the assumed water elevation for seal design shown on the Working Drawings? | ____ | ____ | ____ |
| 4. Does the cofferdam design assume water pressure acts on the full height of the cofferdam (from the vent to the bottom of the excavation?) | ____ | ____ | ____ |
| 5. Has percolation into the excavation been addressed? | ____ | ____ | ____ |
| C. Allowable Stresses | YES | NO | N/A |
| 1. Have the design loads used for cofferdam design of all members been noted in the design calculations? | ____ | ____ | ____ |
| 2. Are the allowable stress and the calculated stress listed in the summary for each different cofferdam member? | ____ | ____ | ____ |
| D. Timber Construction | YES | NO | N/A |
| 1. Are timber grades consistent with material to be delivered to the construction site, noted on the cofferdam drawings, and in accompanying calculations for all timber cofferdam material? | ____ | ____ | ____ |
| 2. If "rough" lumber is specified for the cofferdam, are the actual lumber dimensions used in the calculations shown? | ____ | ____ | ____ |
| E. Steel Construction | YES | NO | N/A |
| 1. Are steel structural shapes and plates identified by ASTM number on the cofferdam Working Drawings and in the calculations? | ____ | ____ | ____ |
| 2. Have steel beams been checked for bending, shear, web crippling and buckling of the compression flange? | ____ | ____ | ____ |
| F. Compression Members, Bracing Members and Connections | YES | NO | N/A |
| 1. Has general buckling been evaluated for all compression members? | ____ | ____ | ____ |
| 2. Has bracing been provided at all points of assumed support for compression members? | ____ | ____ | ____ |
| 3. Is bracing strength and stiffness sufficient for the intended purpose? | ____ | ____ | ____ |
| 4. Have all connections been designed and detailed? | ____ | ____ | ____ |

Designer Engineer of Record Signature		 Date

(Include the following shoring design checklist when required by the Bridge Designer.)

SHORING DESIGN CHECKLIST

Instructions - This shoring design checklist was developed to facilitate the design, review, and erection of shoring to be used for ODOT construction projects.  This checklist is intended to act as a reminder to design or check for specific important aspects of this construction. It is not a substitute for plan and/or design criteria or specification requirements.

The Checklist is to be completed by the shoring design engineer. Answer every question. Attach to the Checklist an explanation of any negative responses.

Submit this Shoring Design Checklist for each stage and phase of the project, along with the shoring design summary, Working Drawings and calculations according to 00510.04.

| A. General | YES | NO | N/A |
| --- | --- | --- | --- |
| 1. Are the shoring Working Drawings and supporting calculations prepared, stamped, and signed by an engineer registered to practice in the state of Oregon? | ____ | ____ | ____ |
| 2. Are the temporary shoring installation plans, construction sequence, and removal plan compatible with the project construction staging/phasing? | ____ | ____ | ____ |
| B. Design Standards | YES | NO | N/A |
| 1. Does the shoring design comply with standards identified in ODOT GDM 16.3.26.3 and related sections? | ____ | ____ | ____ |
| 2. Is the design standard and edition identified in the shoring design calculations? | ____ | ____ | ____ |
| C. Loading | YES | NO | N/A |
| 1. Have the design loads, including special loading conditions (e.g. cranes, stockpiles, etc.), used for shoring design of all members been noted in the design calculations? | ____ | ____ | ____ |
| 2. Have the appropriate load and resistance factors or factors of safety on the shoring system been identified, for all applicable load combinations or load cases? | ____ | ____ | ____ |
| 3. If public traffic is near or directly above the shoring system, has a minimum traffic live load surcharge of 250 psf been applied? | ____ | ____ | ____ |
| 4. Have the loads from actual construction equipment and not less than 250 psf been included in the shoring system design? | ____ | ____ | ____ |
| 5. Have the construction loads for different stages of construction been considered and included in the calculations? | ____ | ____ | ____ |
| 6. Have the effects of any construction activities adjacent to the shoring system on the stability/performance of the shoring system been addressed in the shoring design (e.g., excavation or soil disturbance in front of the wall or slope, excavation dewatering, vibrations and soil loosening due to soil modification/construction activities)? | ____ | ____ | ____ |
| 7. Have earth pressure diagrams been included? | ____ | ____ | ____ |
| 8. Does the shoring design consider the effect of water saturated soil pressure acting on the full height of the shoring? | ____ | ____ | ____ |
| D. Geotechnical and Structural Analysis | YES | NO | N/A |
| 1. Has internal stability been evaluated? | ____ | ____ | ____ |
| 2. Has eccentricity/overturning stability been evaluated? | ____ | ____ | ____ |
| 3. Has sliding been evaluated? | ____ | ____ | ____ |
| 4. Has overall/global stability been evaluated? | ____ | ____ | ____ |
| 5. Has bearing capacity been evaluated? | ____ | ____ | ____ |
| 6. Have displacement constraints or other performance objectives of the shoring system been identified and evaluated? | ____ | ____ | ____ |
| 7. Has each stage of the shoring system construction been evaluated to carry traffic and construction loads and ensure internal and external stability through the construction and loading sequence? | ____ | ____ | ____ |
| 8. Are the allowable stress and the calculated stress listed in the summary for each different shoring member? | ____ | ____ | ____ |
| 9. Have steel beams been checked for bending, shear, web crippling and buckling of the compression flange? | ____ | ____ | ____ |
| 10. Have connections for all phases of construction and removal been designed for all interim loading? | ____ | ____ | ____ |
| 11. Has buckling, bracing strength, and stiffness been evaluated for all compression members? | ____ | ____ | ____ |
| E. Materials | YES | NO | N/A |
| 1. Are all soil, rock, and other material properties used for the design of the shoring system provided and consistent with GDM and the subsurface field and lab data? | ____ | ____ | ____ |
| 2. Are timber grades noted on shoring drawings and in accompanying calculations? | ____ | ____ | ____ |
| 3. Are the minimum lumber dimensions shown in the calculations and noted on the Working Drawings? | ____ | ____ | ____ |
| 4. Are steel structural shapes, bolts, connections, and plates identified by ASTM number on the shoring Working Drawings and in the calculations? | ____ | ____ | ____ |
| F. Shoring Working Drrawings | YES | NO | N/A |
| 1. Is the field verified ground topography above and below the shoring wall shown? | ____ | ____ | ____ |
| 2. Are all existing, adjusted or new utilities, structures, and “no work zones” in proximity to the proposed shoring shown on the shoring Working Drawings and is protection of these items addressed? | ____ | ____ | ____ |
| 3. Are horizontal and vertical clearance requirements identified and shown on the shoring Working Drawings? | ____ | ____ | ____ |
| 4. Are plan view, elevation and cross sections drawn to scale, with dimensions defining location and size of the temporary shoring, components, and excavation limits? | ____ | ____ | ____ |
| 5. Are the magnitude and location of all loads, equipment and personnel that will be supported by the shoring shown or noted on the shoring Working Drawings? | ____ | ____ | ____ |
| 6. Has a dewatering plan been shown? | ____ | ____ | ____ |
| 7. Have all connections been detailed? | ____ | ____ | ____ |
| 8. Has bracing been detailed? | ____ | ____ | ____ |
| G. Testing and Monitoring | YES | NO | N/A |
| 1. If a “yes” response to No. D-6, is a monitoring plan provided to verify adequate performance of the shoring system throughout the design life of the system? | ____ | ____ | ____ |
| 2. Has a load testing program been provided for soil nails, tiebacks, or other applicable elements of the shoring system? | ____ | ____ | ____ |

Design Engineer of Record Signature		 Date