SP00715  (Special Provisions for the 2024 Book) 	(Bidding on or after: 06-01-25

Last updated: 03-03-25)

# SECTION 00715 - MULTIPLE APPLICATION EMULSIFIED ASPHALT SURFACE TREATMENT

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [purple text] above a subsection, paragraph, sentence, or bullet, then include it in the project. Delete all purple text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Comply with to Section 00715 of the Standard Specifications modified as follows:

(Use the following subsection .00 when surface treatment is NOT designated on the plans. Delete those that do not apply. Remove "(s)" or parentheses as appropriate.)

00715.00  Scope - Add the following to the end of this subsection:

Provide the following surface treatment(s) on this Project:

Fine Double Chip Seal

Medium Double Chip Seal

Type E-9 Oil Mat

Type E-11 Oil Mat

00715.11  Emulsified Asphalt - Add the following paragraph to this subsection:

(Choose the correct Polymer-Modified or Non-Polymer-Modified emulsified asphalt. It will be specified in the pavement design report. Delete the one that does not apply.  Remove the parentheses.)

Provide (Polymer-Modified)(Non-Polymer-Modified) Emulsified Asphalt for this Project.

00715.40  Season and Weather Limitations - Replace the title of this subsection with “Weather Limitations”.

(Use the following sentence to change hours. Check with the Project Manager for the number of hours that emulsified asphalt and aggregate can be placed before sunset. It may range from 2 - 4 hours, depending on elevation, work in shaded areas, and other conditions.)

In the sentence that begins "Complete the application of…", replace the words "3 hours" with the words "_____ hours".

(Use the following sentence for Region 3 Projects. Use the previous subsection number and title if above hours are not changed.)

In the sentence that begins "The placing of multiple…", replace "July 1" with "June 15".

(Use the following sentence for Region 4 and 5 Projects. Use the previous subsection number and title if above hours are not changed.)

In the sentence that begins "The placing of multiple…", replace "July 1" with "June 1".

(Use the following subsections .42(a) and .43 to change hours. Check with the Project Manager for the number of hours that courses can be squared up before sunset. It may range from 2 - 4 hours, depending on elevation, work in shaded areas, and other conditions.)

00715.42(a)  Type E-9 and E-11 Oil Mats - Throughout this subsection, replace the words "3 hours"  with the words " _____ hours".

00715.43  Applying Emulsified Asphalt - In the third bullet, replace the words "3 hours" with the words " _____ hours".