SP00759  (Special Provisions for the 2024 Book) 	(Bidding on or after: 10-01-25

Last updated: 07-23-25

Requires SP00765 and SP02510)

# SECTION 00759 - MISCELLANEOUS PORTLAND CEMENT CONCRETE STRUCTURES

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [purple text] above a subsection, paragraph, sentence, or bullet, then include it in the Project. Delete all purple text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Comply with Section 00759 of the Standard Specifications modified as follows:

00759.02  Definitions – Delete this subsection.

00759.11  Aggregate Base - Replace this subsection, except for the subsection number

and title, with the following:

Furnish Aggregate Base Materials for Base, foundation courses, Leveling courses, or bedding meeting the requirements of 00640.10 or dense graded Base Aggregate in 00641.10 as appropriate and included in the Special Provisions.

00759.12  Curb Ramp Treatment - Delete this subsection.

00759.22(a)  Qualified Smart Levels – Replace this subsection, except for the subsection number and title, with the following:

Slopes will be measured with the use of a 24 inch SmartTool level model 92379 or model 92500, and a 6 inch SmartTool level model 92346 or 92510.

00759.42  Foundations - Replace this subsection with the following subsection:

00759.42  Aggregate Base - Before placing concrete, prepare underlying Aggregate Base surfaces according to Section 00641.

00759.50(a)  General - Replace the paragraph that begins “Install truncated domes…” with the following paragraph:

Construct tactile surface indicators according to Section 00765 to the slopes, lines, grades, and dimensions as shown or established.

00759.51  Curing - Add the following paragraph to the end of this subsection:

Concrete Structures may be opened to Public Traffic before 7 Calendar Days if the concrete has reached a minimum compressive strength of 2,000 psi as verified by the rebound number determined according to ASTM C805. Test at locations as directed.

00759.80  Measurement - Replace the bullet that begins “Each Basis…” with the following bullet:

Each Basis - Measurement will be by actual count.

(Use the following subsection .90 when any of the paragraphs below are required.)

00759.90  Payment - Delete Pay Items (k), (l) and (m) from the Pay Item list.

Delete the paragraph that begins “Item (k) includes…”.

Delete the paragraph that begins “Item (l) includes…”.

Delete the paragraph that begins “Item (m) includes…”.

Add the following paragraph before the paragraph that begins "No separate or additional payment…":

Tactile surface indicators will be paid for according to Section 00765.

In the paragraph that begins "No separate or additional payment…", add the following bullet to the end of the bullet list:

The additional Work required to construct a curb ramp or replace an existing curb ramp, including any corrective action.

(Use the following two paragraphs when earthwork is included as separate Pay Items but the earthwork Pay Items and estimated quantities do not include all or a portion of the Work in this Section. The use of this language requires notation (contact the Senior Standards Engineer) in the Plans to describe the earthwork pay limits relative to SP00330 and requires the State Specification Engineer and RE approval. Consider issuing a LOC.)

Replace the paragraph that begins "When earthwork is included as separate Pay …" with the following paragraph:

When earthwork is included as separate Pay Items under Section 00330, payment will be made according to 00330.90 through 00330.94 as appropriate and only for locations where earthwork quantities are shown. No separate or additional payment will be made for earthwork under Section 00330 where earthwork quantities are not shown.