SP01069  (Special Provisions for the 2024 Book) 	(Bidding on or after: 10-01-25

Last updated: 06-30-25)

# SECTION 01069 - METAL HANDRAIL AND PEDESTRIAN FENCE

(Follow all instructions and make all edits with “Track Changes” turned on. This Section is not published in the Oregon Standard. If there are no instructions [purple text] above a subsection, paragraph, sentence, or bullet, then include it in the Project, unless the item(s) that are included in the subsection, paragraph, sentence, or bullet are not required on the Project and then they should be deleted. In general do not re-number or re-letter subsections when item(s) are deleted. Delete all purple text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Comply with Section 01069 of the Standard Specifications modified as follows:

01069.90  Payment - Replace this subsection, except for the subsection number and title, with the following:

The accepted quantities of Work performed under this Section will be paid for at the Contract unit price, per unit of measurement, for the following items:

Pay Item	Unit of Measurement

(a)	Metal Handrail, ______ Rail	Foot

(b)	Pedestrian Fence	Each

(c)	Metal Handrail, ______ Rail with Optional Rail	Foot

In item (a), “two” or “three” will be inserted in the blank.

In item (c) “two” or “three” will be inserted in the blank. Item (c) also includes the additional optional bottom rail.

Payment will be payment in full for furnishing and placing all Materials, and for furnishing all Equipment, labor, and Incidentals necessary to complete the Work as specified.