SP00942  (Special Provisions for the 2024 Book) 	(Bidding on or after: 07-01-26

Last updated: 04-03-26

Requires SP02910)

# SECTION 00942 - PERMANENT BARRICADES

(Follow all instructions and make all edits with “Track Changes” turned on. This Section is not published in the Oregon Standard. If there are no instructions [purple text] above a subsection, paragraph, sentence, or bullet, then include it in the Project, unless the item(s) that are included in the subsection, paragraph, sentence, or bullet are not required on the Project and then they should be deleted. In general do not re-number or re-letter subsections when item(s) are deleted. Delete all purple text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Section 00942 is not a Standard Specification and is included in this Project by Special Provision.

## Description

00942.00  Scope - This Work consists of furnishing, fabricating, and installing permanent Type III barricades as shown.

## Materials

00942.10  Materials - Furnish Materials for permanent Type III barricades meeting the following requirements:

Hardware	02910.40

Plywood	02910.11

Posts	02110.40

Reflective Sheeting (Type III or Type IV)	02910.20

## Construction

00942.40  General - Construct permanent barricades as shown.

Place reflective sheeting on the horizontal member before assembling the required splice.

A sheeting manufacturer approved lubricant may be used on the nylon and metal washers to prevent sign sheeting deformation.  Replace damaged horizontal members or horizontal members with sheet deformation at no additional cost to the Agency.

## Measurement

00942.80  Measurement - The quantities of permanent barricades will be measured on the unit basis.

## Payment

00942.90  Payment - The accepted quantities of permanent barricades will be paid for at the Contract unit price, per each, for the item "Permanent Type III Barricades".

Payment will be payment in full for furnishing and placing all Materials, and for providing all Equipment, labor, and Incidentals necessary to complete the Work as specified.

No separate or additional payment will be made for excavation and backfill.