SP00868  (Special Provisions for the 2024 Book) 	(Bidding on or after: 08-01-26

Last updated: 04-24-26

Requires SP00850)

# SECTION 00868 - COLORED LANE MARKINGS

(Follow all instructions and make all edits with “Track Changes” turned on. If there are no instructions [purple text] above a subsection, paragraph, sentence, or bullet, then include it in the Project. Delete all purple text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Comply with Section 00868  of the Standard Specifications modified as follows:

00868.45  Installation - Replace the paragraph that begins "Apply the material …" with the following paragraph:

Apply the material to the pavement according to the manufacturer’s installation instructions to the full width shown in the Plans. Joints are allowed with no overlap or gap allowed at the joint.

Replace the bullet that begins "Methyl Methacrylate …" with the following bullet:

Methyl Methacrylate - Apply methyl methacrylate to the pavement to the full width shown in a single application. Apply colored lane markings 90 mils to 120 mils in thickness, exclusive of projecting surface-applied friction elements, with a continuous and uniform cross sectional configuration.

00868.75  Manufacturer Warranty - Replace the paragraph that begins "Furnish a manufacturer …" with the following paragraph:

Provide a manufacturer warranty that unconditionally warrants to the Agency the product(s) and installation under this Section against failure, according to this Subsection and 00170.85(c)(1). Use Agency-supplied warranty forms, available from the Engineer.

00868.90  Payment - Replace the paragraph that begins "Payment will be payment …" with the following paragraph:

Payment will be payment in full for furnishing and placing all Materials, and for providing all Equipment, labor, and Incidentals necessary to complete the Work as specified.