SP01011  (Special Provisions for the 2024 Book) 	(Bidding on or after: 08-01-26

Last updated: 04-23-26

Requires SP00842, SP01030, SP02320 & SP03020)

# SECTION 01011 - STORMWATER CONTROL, PONDS

(Follow all instructions and make all edits with “Track Changes” turned on. This Section is not published in the Oregon Standard. If there are no instructions [purple text] above a subsection, paragraph, sentence, or bullet, then include it in the Project, unless the item(s) that are included in the subsection, paragraph, sentence, or bullet are not required on the Project and then they should be deleted. In general do not re-number or re-letter subsections when item(s) are deleted. Delete all purple text before preparing the final document. All other modifications to this Section will require ODOT Technical Resource and State Specifications Engineer approval.)

Section 01011 is not a Standard Specification and is included for this Project by Special Provision.

## Description

01011.00  Scope - This Work consists of furnishing and installing stormwater ponds as shown.

## Materials

(Delete material items that do not apply and include other materials as necessary.)

01011.10  Materials - Furnish Material meeting the following requirements:

Concrete	 00440

Drainage Geotextile, Type 1	 02320

Facility Field Markers	 00842.10

Granular Drain Backfill	00430.11

Manholes, Catch Basins, and Inlets	 00470.11

Riprap	 00390.11

Riprap Geotextile, Type 1	 02320

Storm Sewer Pipe	 00445.11

Subsurface Drain Pipe	 00430.10

(Use the following subsection .11 when porous pavers are required. Fill in the blank with one of the following types of pavers to use only in storage ponds: heavy duty, medium duty, or light duty. Obtain the type to use from the Designer.)

01011.11  Porous Pavers - Furnish ___________________ porous pavers from the QPL.

01011.12  Water Quality Mixture - Furnish medium compost meeting the requirements of Section 03020. Furnish soil meeting the following gradation requirements:

Percent Passing

Sieve Size	(by Weight)

No. 4	100

No 10	95 - 100

No. 40	40 - 60

No. 100	10 - 25

No. 200	5 - 10

Sample soil according to AASHTO R 90. Determine sieve analysis according to AASHTO T 27 and AASHTO T 11.

Blend the medium compost and soil so that the mixture:

Is composed of between 20 percent and 25 percent medium compost material and between 75 percent and 80 percent soil material.

Has a pH between 5.5 and 8.0.

Does not have clumps greater than 3 inches in any direction.

## Construction

01011.40  General - Construct storage facility as shown. Perform excavation and fine grading work only when the facility area is dry and only from the top of the pond area. Do not stockpile material in the facility area.

01011.41  Storage Pond - Scarify the subsoil area a minimum 12 inches deep. After scarification, place the water quality mixture in maximum 12-inch Lifts. Compact each Lift with a water filled landscape roller.

01011.42  Bioretention Pond:

(a)  Scarify - Scarify the subsoil area a minimum 12 inches deep.

(b)  Laying Pipe - Lay the pipe according to Section 00445. Place pipe with perforations down unless otherwise directed.

(c)  Joining Pipe - Fasten pipes together with coupling fittings or bands as specified for the type of pipe used. Cap the upstream end of the pipe.

(d)  Inspection and Repair - Place Type 2 water quality mixture only after all the pipe is laid, joined, and inspected. Remove and reinstall or replace all pipe that is out of alignment, has settled, or is damaged at no additional cost to the Agency.

(e)  Placement of Water Quality Mixture - Place water quality mixture in maximum 12-inch Lifts. Compact each Lift by using a water filled roller.

(f)  Seeding - Seed according to 01030.13.

01011.43  Facility Field Markers - Install field markers as shown and according to Section 00842.

## Maintenance

01011.70  Cleaning - If a stormwater control facility is used for erosion and sediment control, remove all accumulated sediment and debris before completing the facility.

## Measurement

01011.80  Measurement - No measurement of quantities will be made for Work performed under this Section. The estimated quantities of Materials are:

(Use the following "Storage Pond" heading and item list when Pay Item (a) is included in the Pay Item list below. Identify ponds by the Drainage Facility Identification Number (DFI). List each DFI separately. Obtain the DFI, the items, and the quantities from the Designer. Fill in all the required blanks. Delete items that do not apply. Copy and paste for multiple facilities.)

Storage Pond __(DFI)__ Quantities:

Item	Quantity

Excavation	 ____ Cu. Yd.

Drainage Geotextile, Type 1	 ____ Sq. Yd.

Riprap Geotextile, Type 1	 ____ Sq. Yd.

Loose Riprap, Class ____	 ____ Cu. Yd.

Water Quality Mixture,	 ____ Cu. Yd.

Catch Basin, ____	 ____ Each

Concrete Storm Sewer Manhole (Flow Control)	 ____ Each

____ Inch Storm Sewer Pipe	 ____ Foot

Porous Pavers	 ____ Sq. Ft.

(Use the following "Bioretention Pond" heading and item list when Pay Item (b) is included in the Pay Item list below. Identify ponds by the Drainage Facility Identification Number (DFI). List each DFI separately. Obtain the DFI, the items, and the quantities from the Designer. Fill in all the required blanks. Delete items that do not apply. Copy and paste for multiple facilities.)

Bioretention Pond __(DFI)__ Quantities:

Item	Quantity

Excavation	 ____ Cu. Yd.

Drainage Geotextile, Type 1 	 ____ Sq. Yd.

Riprap Geotextile, Type 1	 ____ Sq. Yd.

Loose Riprap, Class ____	 ____ Cu. Yd.

Granular Drain Backfill	 ____ Cu. Yd.

Water Quality Mixture	 ____ Cu. Yd.

Catch Basin, ____	 ____ Each

Concrete Storm Sewer Manhole (Flow Splitter)	 ____ Each

____ Inch Storm Sewer Pipe 	 ____ Foot

____ Inch Subsurface Drain Pipe 	 ____ Foot

Porous Pavers 	 ____ Sq. Ft.

Field facility markers will be measured according to 00842.80.

## Payment

(Delete the "(s)" or parentheses from the word "item(s)", as appropriate.)

01011.90  Payment - The accepted quantities of Work performed under this Section will be paid for at the Contract lump sum amount for the item(s):

(Delete Pay Item(s) from the list that are not included in the Schedule of Items, but do not change the alpha characters next to the Pay Items.)

Pay Item	Unit of Measurement

(a)	Storage Pond, _____ 	 Lump Sum

(b)	Bioretention Pond, _____ 	 Lump Sum

The drainage facility identification number is inserted in the blank.

Field facility markers will be paid for according to 00842.90.

Payment will be payment in full for furnishing and placing all Materials, and for providing all Equipment, labor, and Incidentals necessary to complete the Work as specified.