9/09/2019

SUBJECT:	Access Management Strategy

OR214: Johna Ln to Fenne Ln Sidewalks (Silverton)

Silver Creek Falls Highway 163

Key18745

Marion County

# Introduction

An access management strategy is a project delivery process required of all modernization and highway improvement projects included in the Statewide Transportation Improvement Program (STIP) which propose to modify, relocate, or remove existing public or private connections to the state highway.  An access management strategy is developed in collaboration with cities, counties and owners of real property with the intention of documenting the location and type of highway approaches as well as evaluating the need to modify, relocate or remove existing approaches.  The proposed modifications are based on an access management methodology that balances the economic development objectives of properties abutting the state highway with the transportation safety, access management objectives and mobility of state highways, in a manner consistent with local transportation system plans and the land uses permitted in the local comprehensive plans.

# Project Description

This project is an infill project constructing Sidewalks and Bike lanes  where they are not today. The project limits are from Johna Ln to Fenne Ln. On the Silver Creek Falls  Highway 163 (OR214) from mile point 39.23 to 40.20.

# Existing Conditions

### Classification

The Silver Creek Falls  Highway is classified as a District Highway in the 1999 Oregon Highway Plan (OHP). The OHP defines Regional Highways as follows.

### District Highways: (NHS) ODOT has a set of roadway designations to reflect different key characteristics on roadway corridors. Roadway classifications or designations affect long-range planning decisions for ODOT facilities Classifications and designations impact planning for development and redevelopment located along the roadway. Roadways with a functional focus on higher speed travel (mobility) such as a state or district highway- will limit the number of driveways accessing the roadway to reduce conflict points with vehicles moving at high speeds.

### Posted Speed

Posted speed on OR214 within the project is:

| Beginning Mile Point | Ending Mile Point | Speed (MPH) |
| --- | --- | --- |
| 39.23 | 40.07 | 40 |
| 40.07 | 40.20 | 30 |

### Horizontal and Vertical Alignment

### ADD LIKE 2 LANE HIGHWAY WITH VARYING SHOULDER WIDTHS OF 1’ TO 5’ OR WHATEVER

### Minimal urban improvements exist

### Project will be west/east side only or both

### Project will be transition to constructing curb walk drainage on both sides now

### Project will modifiy to a 3 lane highway with standard bike lanes and reconstructed curb and sidewalks on both sides of the highway.

### Average Daily Traffic (ADT)

Traffic volume information within the project area indicated an AADT of 7700 vehicles per day (VPD) at mile point 39.86 to 40.2

### Land Use and Surrounding Development

The properties on both sides of the highway are residential..

### Crash History

A review and summary of the crash history was compiled using the Crash Summary Report (CSR) and the ODOT Safety Investigations Manual (SIM) Crash Pattern Worksheet data for the project.  The purpose of the CSR crash data is to identify potential safety “hot spots”.  The data are for a 3-year study period.  The CSR data cover the years 2013 – 2015.  The SIM Crash Pattern Worksheet data cover a 10 year period from January 20 – December 2018 and can produce a more comprehensive summary.  Below is a summary of the two crash databases.

|  | Silver Creek Falls Highway 163 (OR214) Mile Points 39.23-40.20 |  |  |  |
| --- | --- | --- | --- | --- |
| Type of Crash | CSR |  | SIM |  |
| Rear-end | 13 |  | 0 |  |
| Fixed/Other Object | 1 |  |  |  |
| Angle | 0 |  | 0 |  |
| Turning | 2 |  | 0 |  |
| Side Swipe Meeting or Overtaking | 0 | 2 | 0 | 0 |
| Pedestrian | 0 |  | 0 |  |
| Backing | 1 |  | 0 |  |
| Head-on | 0 |  | 10 |  |
| Non-collision | 0 |  | 0 |  |
| Intersection Crashes | 2 |  | 0 |  |
| Miscellaneous | 0 |  | 0 |  |
| Total Crashes | 19 |  |  |  |
| High SPIS Value | None |  | None |  |
| Top 10% Cutoff |  |  |  |  |
| Top 10% List Sites | 0 |  |  |  |
| Fatalities | 0 |  | 0 |  |

Crash Summary Table

Summary of Crash History Findings

The investigation crash rate (0.61) is lower than the peer rate (1.13), the statewide rate (1.24) and the critical rate (2.80), indicating that crashes are not abnormally high in the segment. Due to the short segment length, crash rates are expected to be a less reliable indicator of problems. No crashes were attributed to alleys (approaches). The single crash was an injury-A event.

This data indicates that there is not an existing safety issue related to private highway approaches.

### Access Control Research

The term access control references locations where the state has acquired the abutter’s right of access from property owners adjacent to the highway or where Oregon Revised Statute (ORS) 374.405 applies.  Research was conducted by the ODOT Right-of-Way Section to determine if the highway frontage on either side of the highway is access controlled.  Approaches to the highway must have a valid right of access to be legal approaches.  Research confirmed that no highway frontage is access controlled within project limits.

# Access Management Methodology

An access management methodology is the criteria used to assist in evaluating road approaches for potential modification, relocation or removal.  An access management methodology is intended to balance the economic development objectives of properties abutting the state highway with the transportation safety, access management objectives, and mobility of state highways.  Establishing an access management methodology by which private connections will be considered for modification, relocation, or closure requires public involvement with cities, counties and owners of real property abutting the highway.  Property owners reasonably expected to be impacted were notified of the following access management methodology and granted a twenty-one day review period.  The access management methodology as mailed for review was approved for the project; criteria are listed below.

Objectives of property owners:

- Ensure that the location and design of the approach adequately serves the volume and type of traffic reasonably anticipated to enter and exit the property, based on the planned uses for the property.

- Ensure that the location and design of the approach reasonably addresses the unique aspects of the business or property use, including, but not limited to, the location of the building(s), parking and site circulation.

- Ensure that the approach to the property is safe to enter and exit.

Public policy considerations:

- Close, modify or relocate approaches which are in conflict with replacement structure or its appurtenances.

# Access Management Approach Analysis

The methodology was applied to each approach within the project limits using the following analysis. Objectives at the methodology and collaborating with potentially affected property owners.

All decisions to modify, relocate or close an existing private approach shall balance the needs of the property owners with the needs of the public and the highway system.

Access shall remain adequate to serve the volume and type of traffic reasonably anticipated to enter and exit the property, based on the planned uses for the property.

### Action Items

One closure of a or how manyprivate connection resulted from application of the methodology.  Approaches not discussed here will not be affected.

Private Approaches

Tax Lot 05S-09W-34 - 00200 (Hancock Forest Management)

Approach 1: This 14’ gravel approach is located at mile point 11.34 on the right/south side of OR22 and serves commercial forest land. The approach is not access controlled and has a valid UPermit (#21666). Strategy: Close approach. This approach is in conflict with the replacement structure and required guardrail (criteria 4). In collaboration with Hancock Forest Management, ODOT learned that the owner’s preference was to retain access at the current location, but if infeasible they were not opposed to alternate access.  A scoping-level estimation was performed by the bridge designer, and modifying the bridge and channel to accommodate the existing approach was estimated to cost $903,000.  The estimated value of the parcel with remaining timber is $225,000. The parcel has adequate frontage on the highway to construct a private bridge and approach to serve the parcel or the owner may construct an approach to US Forest Service (USFS) Road NF-1180.

The AMST contacted USFS District Engineer Joe Acosta to determine process and estimate costs to provide access via NF-1180.  Mr. Acosta reported that Hancock Forest Management would need to obtain a special use permit for commercial logging operations, but nothing is required for maintenance operations. Mr. Acosta also stated that he was familiar with Hancock Forest Management and there were similar arrangements between Hancock and the USFS elsewhere.

Hancock Forest Management will be compensated for this approach closure.  Compensation will be determined through the right of way process. The expected cost to cure will include compensation for eventual construction of a logging road from NF1180 to connect with the existing road network on site and permitting fees to the USFS for the use of NF1180 during future logging operations.

Acquisition of access control was considered and eliminated. No benefit to the highway was perceived. The winding character of OR22 and local topography eliminates the feasibility of an approach along much of Hancock Forest Management’s frontage east of the existing approach, and consequently limits the value of access rights along that section of their frontage.  There appears to be adequate sight distance at the west end of the parcel to construct a highway approach if Hancock Forest Management prefers direct highway access instead of access via NF-1180.  An approach constructed along this frontage will be complicated by the need to construct a private structure across Sourgrass Creek and topography; as such this option was not thoroughly investigated.

### Access Management Sub-team Members

The access management strategy team (AMST) is comprised of individuals internal to the Oregon Department of Transportation (ODOT).  The members of the AMST include:

Christine Hildebrant, ODOT Project Leader

Chris Primm, ODOT Region 2 Access Management Project Delivery Coordinator

Ian Roholt, ODOT Region 2 Roadway Designer

David Thiessen, ODOT Region 2 Senior Right of Way Agent

Ray Drake, ODOT District 3 Permit Specialist

### Notification Process

The Department is required to communicate with the property owners when an approach is modified, relocated or closed as part of construction activities.  Department records indicate that the sole approach is presumed to have written permission.  As such, the owners of the approach will have the right to appeal the proposed closure.

### Permitting of Road Approaches

Upon completion of project construction, the District 3 maintenance office will issue permits for the road approaches indicated on the official project access list (OPAL).  There will be no charge to the property owners for the issuance of a road approach permit.  If a development proposal results in a request for a road approach to the state highway system after the access management strategy has been approved, property owners will be advised to submit an application for a road approach permit.  The application will be subject to review and approval as outlined in Oregon Administrative Rule Section 734, Division 51.

Appendix A: Official Project Access List (OPAL)

Appendix B: Access Management Methodology Criteria Matrix